3GPP TSG SA WG2#165	            S2-2410690

Hyderabad, India, 14-18 October 2024	(revision of S2-2410600)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0267 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | I-SMF insertion for Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To address the outcome of KI#1 in approved eEDGE_5GC_Ph3 WID SP-240996 on reducing the impact on central 5GC NFs by using I-SMF based approach, I-SMF selection/insertion based on Local Offloading Management allowed indication in the subscription data is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce I-SMF selection/insertion based on Local Offloading Management allowed indication |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.X.Y	I-SMF insertion for Local Offloading Management

#### 6.X.Y.1	I-SMF Insertion During PDU Session Establishment

This clause defines procedure for triggering I-SMF insertion for Local Offloading Management.

Editor’s Note: 	The detailed procedure is FFS

.

#### 6.X.Y.2	I-SMF Insertion After PDU Session Establishment

This clause defines procedure for triggering I-SMF insertion or removal for Local Offloading Management after a PDU session is established with the anchor SMF and/or an I-SMF is already inserted when the UE is outside the anchor SMF service area. The procedure described in clause 4.23.5.4 of TS 23.502 [3] is performed with some modifications. The details of these modifications are shown in Figure 6.X.Y.2-1 and further described below.

Figure 6.X.Y.2-1: SMF triggered I-SMF insertion or removal for Local Offloading Management after PDU session establishment

- During the PDU Session Establishment procedure in clause 4.23.5.1 of TS 23.502 [3], SMF receives Local Offloading Management allowed indication from UDM.

- Similar to step 1 of Figure 4.23.5.4-1 of TS 23.502 [3], the PCF sends to the SMF, PCC rule(s) applicable to the ongoing PDU session by invoking Npcf_SMPolicyControl_UpdateNotify service operation. The PCC rule(s) includes a target DNAI or a list of DNAIs that is/are for Local Offloading Management.

- Similar steps 2 and 3 of Figure 6.X.Y.1-1 with the following enhancement in Step 2:

  - if an I-SMF is already in use for the PDU Session and the SMF decides that the SMF itself can serve the target DNAI that is not allowed for local offloading management, hence the existing I-SMF can be removed.

- Similar to step 4 of Figure 6.X.Y.1-1 with the following enhancement:

For the I-SMF removal, AMF removes the existing I-SMF if:

  - the AMF receives from SMF Nsmf_PDUSession_SMContextStatusNotify without including target DNAI;

  - the AMF identifies that the SMF can serve the target DNAI that does not have Local Offloading Management allowed indication; or

  - the existing I-SMF cannot serve the UE location (e.g. due to mobility).

- Similar to step 4 of Figure 4.23.5.4-1 in TS 23.502 [3] with the following enhancement:

Nsmf_PDUSession_CreateSMContext Request to the new I-SMF includes Local Offloading Management allowed indication (step 5a of Figure 6.X.Y.2-1).

- Similar to step 5 of Figure 4.23.5.4-1 in TS 23.502 [3] with the following enhancements:

Based on the Local Offloading Allowed indication, the selected I-SMF:

retrieves EAS Deployment Information from NEF, as described in clause 6.2.3.4.3, and proceeds with UPF selection; and

proceeds with EASDF selection and retrieves EASDF information including EASDF IP address and DNS security information.

The I-SMF sends PDU session create request to SMF including the EASDF information and Local Offloading Management support indication.

Based on the I-SMF request, SMF retrieves Local Offloading Management policy from PCF and provides the policy as part of PDU session create response.

I-SMF generates Local Offloading Management information based on the policy received from SMF.

- I-SMF proceeds with EASDF configuration based on EAS Deployment Information and Local Offloading Management information.

- The step 6 of Figure 4.23.5.4-1 is followed.

End of CHANGES