3GPP TSG-WG SA2 Meeting #165	S2-2410702

# Hyderabad, India, 14th Oct – 18th Oct, 2024		(revision of S2-2409143)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0240 | rev | 4 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Policy provisioning for PDU session with local SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to the conclusion as in the cluase 8.1 of TR 23.700-49: - I-SMF can be configured with or receive the local offloading policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s). - the SMF may forward AF provided traffic influence information to I-SMF. Rev3: - Per the DP S2-2410248, it is further proposed below part: - SMF filters the policy received from PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause 6.X.3 for support of policy provisioning for PDU sessions supporting Local Offloading Management. Rev3: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Forwarding local offloading policy and PCC rule to I-SMF is not clear. Rev3: |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Rev3: In S2#164 meeting, S2-2409143 describe Policy provisioning but be postponed due to time limitation. This contribution proposes to resubmit the CR with some additional change (in yellow highlight color). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.X.3	Policy provisioning for Local Offloading Management

6.X.3.1	General

For Local Offloading Management, traffic routing related policy (e.g., AF traffic influence information in PCC rule, local offloading information) need be provided to the I-SMF.

Based on the receiving traffic routing related policy information and EAS Deployment information, the I-SMF may discover and select ULCL/BP and L-PSA for local traffic handling.

6.X.3.2	Local offloading policy handling for Local Offloading Management

The Local Offloading Policy is generated by PCF and retrieved by SMF for Local Offloading Management as described in clause 6.X.2. The Local offloading Policy may also be provided with an Offload Identifier. The Offload Identifier is assigned by PCF, and is unique within the PLMN.The SMF and I-SMF for Local Offloading Management may also be preconfigured with Local Offloading policy and tagged with Offload Identifier(s).

For Local Offloading Management, the SMF generates Local Offloading Information based on the received Local Offloading Policy and DNAI(s) supported by the I-SMF. The SMF sends the Local Offloading Information to I-SMF as described in clause 6.X.2.

The detailed information of Local Offloading Policy is described in clause 6.4 of TS 23.503 [4].

6.X.3.3	PCC rule handling for Local Offloading Management

6.X.3.3.1	AF request for Local Offloading Management

The AF may send to NEF an AF request to influence traffic routing as described in caluse 4.3.6 of TS 23.502[3]. When the SMF receives AF traffic influence information in the PCC rule, it provides to I-SMF with the AF traffic influence information associated with the DNAI(s) supported by I-SMF.

NOTE X:	The AF does not need to to be aware of whether Local Offloading Management is applied or not.

# * * * * End of changes * * * *