SA WG2 Meeting #165 	S2-24010732

# 14 - 18 Oct 2024, Hyderabad, India

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1238 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on public IP address reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an Editor NOTE about the case when multiple IP addresses are used in the remote end due to client side load balancing. In order not to complex the NAT information exposure function, it is proposed to use only one of these IP addresses to retrieve the public UE IP address. The NAT functionality needs to be changed to NAT information exposure functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Editor NOTE. Change the NAT functionality to NAT information exposure functionality |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.2.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.2.8.2.4.5	AF correlates UE data collection and NWDAF request when there is NAT between UE and AF

This is only valid if there is NAT between the UE and the AF, and the AF has retrieved the UE private IP address assigned by 5GC for the PDU session either from the SMF, or provided from the NWDAF in order to request data collection from the UE Application.

Figure 6.2.8.2.4.5-1: AF correlates UE data collection and NWDAF request when there is NAT between UE and AF

0.	The AF gets or retrieves the UE private IP address as described in clauses 6.2.8.2.4.2, 6.2.8.2.4.3, and 6.2.8.2.4.4.

For untrusted AF, the following steps 1-7 apply. For trusted AF, the following steps 3-6 apply and NEF is replaced by the AF.

1.	The AF requests NEF to provide the UE public IP address towards the AF using Nnef_UEAddress_Subscribe (UE private IP address, IP address of remote end) indicating immediate reporting.

2.	The NEF authorizes the AF request. If the authorisation is not granted, the NEF replies to the AF with a Result value indicating authorisation failure; otherwise the NEF proceeds with the following steps.

3.	The NEF sends the Nnrf_NFDiscovery Request service operation to obtain the address of the UPF implementing NAT information exposure functionality for the UE private IP address. The request includes the UE private IP address. The NEF may also include the DNN and S-NSSAI associated with the AF ID, as well as the IP domain.

4.	The NRF responds with a Nnrf_NFDiscovery service response including the UPF address of the UPF implementing NAT information exposure functionality for the UE private IP address.

5.	The NEF uses the Nupf_EventExposure_Subscribe service operation to request UE public address and port information from the UPF. The request includes the UE private IP address and an IP address of the remote end and indicates immediate reporting.

6.	The UPF sends the UE public IP address and source TCP/UDP port by invoking the Nupf_EventExposure_Notify to the NEF.

7.	The NEF provides the UE public IP address and source TCP/UDP port provided by UPF in step 6 to the AF using Nnef_UEAddress_Notify.

8.	The AF correlates the UE data by using the retrieved UE public IP address and port information.

* * * End of Change * * *