3GPP TSG-WG2 Meeting #165	S2-2410735

Hyderabad, India,  2024-10-14 -- 2024-10-18	(revision of S2-2410040)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4877 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Handling of Headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Vodafone, ETRI, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification of KI#3 outcome Enhancement of UPF handling of headers according to approved WID: SP-240995 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes have been introduced: - Additional information in Nnef_TrafficInfluence service and Npcf_PolicyAuthorizarion service to support requests for Handling of Payload Headers. - Traffic influence on routing API can be used to request Handling of Payload Headers. The procedures in clause 4.3.6 of TS 23.502 have been updated. - The procedure for notification of Handling of Header event has been added to 4.15.12, which has become more general for UPF event exposure. - A new data subset in the Application data set is provisioned in the UDR. - A new Handling of Header event has been added to Nupf_EventExposure service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of Headers Enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6, 4.3.6.1, 4.3.6.2, 4.3.6.4, 4.15.12, 4.15.12.x (new), 4.15.12.y (new), 4.15.12.z (new), 5.2.5.3.2, 5.2.5.3.3, 5.2.12.2.1, 5.2.6.7.2, 5.2.26.1, 5.2.26.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5454, TS 23.503 CR 1329 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR refers from clause 4.3.6.4 to updates in clause 6.3.1 in TS 23.503 (CR 1329), and from clauses 4.3.6.1, 4.3.6.2, 4.3.6.4, 4.3.6.x (new), 5.2.5.3.2, 5.2.5.3.3, 5.2.12.2.1, 5.2.6.7.2, 5.2.26.2.1 to new clause 5.6.x in TS 23.501 (CR 5454) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.3.6	Application Function influence on traffic routing, service function chaining and handling of payload headers

#### 4.3.6.1	General

Clause 4.3.6 describes the procedures between an Application Function and the SMF to maintain an efficient user plane path and/or to provide N6-LAN service function chaining and/or to perform Handling of Payload Headers for Application Functions that require it.

As described in clauses 5.6.7, 5.6.16 and 5.6.x of TS 23.501 [2], an Application Function may send requests to influence SMF routeing decisions for User Plane traffic of PDU Sessions. The AF requests may influence UPF (re)selection and allow routeing of user traffic to a local access (identified by a DNAI) to a Data Network and/or influence the steering of user traffic to service function chain(s) identified by SFC identifier(s) and/or request Handling of Payload Headers according to Header Handling Control Information. The AF may also provide in its request subscriptions to SMF events or UPF events. A V-SMF supporting HR-SBO provides, SUPI, an indication of support for HR-SBO, and HPLMN DNN and S-NSSAI related to the PDU session to the L-PSA UPF at PDU session establishment.

NOTE 1:	NEF uses the SUPI to derive PLMN ID of the UE.

The following cases can be distinguished:

NOTE 2:	Such requests target an on-going PDU Session.

When receiving an AF request on Application Function influence on traffic routing targeting an individual UE IP address the NEF needs to determine whether the target PDU Session is working in HR-SBO mode. If the target PDU Session is NOT working in HR-SBO mode (non roaming or LBO PDU Session) then the NEF contacts the PCF of the PDU Session as further defined in the clause 4.3.6.4. If the target PDU Session is working in HR-SBO mode, the NEF does not contact the PCF of the PDU session but needs to store the AF request in UDR as defined in clause 4.3.6.5.

The NEF determines whether the PDU Session that the AF requests to influence is working in HR-SBO mode or not and in the former case determines the HPLMN of the UE, the DNN and S-NSSAI of the PDU Session as follows:

If the AF has determined based on procedure in clause 4.3.6.5.2 that the PDU session is home routed, the AF provides IP address of the UE (assigned by HPLMN) and HPLMN ID, HPLMN DNN and S-NSSAI, and the NEF determines if the PDU session is subject to HR-SBO using the provided information and proceeds according to clause 4.3.6.5.2.

If the AF has provided an IP address of the UE known to the AF (IP address not assigned by HPLMN), and DNN and S-NSSAI, the NEF determines if the PDU session is subject to HR-SBO based on local configuration. If PDU session is not subject to HR-SBO, the NEF proceeds according to clause 4.3.6.4. If PDU session is subject to HR-SBO the NEF determines the HPLMN and proceeds from according to clause 4.3.6.5.2, excluding step 0a.

NOTE 3:	The DNN can be unique for a PLMN (see TS 23.003 [33]) and can be mapped to a HPLMN, or the IP address can be within a range that can be mapped to a HPLMN.

If the AF has not provided the (H)PLMN ID, DNN and S-NSSAI:

-	If the IP address of the UE in the AF request is a private IP address, the NEF determines if the request relates to HR-SBO or not based on local configuration, or based on interactions with UPF, since the V-SMF shall have provided an indication that the PDU Session is working in HR-SBO mode, SUPI and the HPLMN DNN and S-NSSAI of the PDU Session to the L-PSA UPF. If the request relates to HR-SBO, the NEF proceeds according to clause 4.3.6.5.3, otherwise it proceeds according to clause 4.3.6.4.

NOTE 4:	Whether or not NEF interact with UPF to determine if HR-SBO applies is based on configuration. If NEF does not interact with UPF, the 5GC NF (e.g. SMF/UPF, NEF) in PLMN can be configured to have separate private IP address ranges for HR-SBO PDU Sessions. This would allow NEF to distinguish AF request for influence on traffic routing for HR-SBO PDU Sessions from non-roaming and LBO PDU Sessions.

-	If the IP address of the UE in the AF request is a public IP address:

-	If this Public IP address belongs to an IP range not owned by the PLMN of the NEF, then the target PDU Session is working in HR-SBO mode and the NEF proceeds according to clause 4.3.6.5.4.

-	If the UE IP Address in the AF request is an IP address NATed by the PLMN that the NEF belongs to, the NEF invokes steps 3 to 6 of clause 4.15.10 to get the corresponding private UE IP address. If HR-SBO applies, the V-SMF shall have provided an indication that the PDU Session is working in HR-SBO mode, SUPI and the HPLMN DNN and S-NSSAI of the PDU Session to the L-PSA UPF. Thus, the NEF can also receive these parameters and determines if the AF request relates to HR-SBO PDU Session, the NEF proceeds according to clause 4.3.6.5.3. Otherwise the NEF proceeds according to clause 4.3.6.4.

NOTE 5:	In this Release, the HPLMN allows HR-SBO for a PDU session only if the UE IP address of the PDU Session has not been allocated in a range that may overlap with other PDU sessions to the same DNN and S-NSSAI of that HPLMN.

NOTE 6:	It is assumed that the NEF is configured with the NATed IP range of its own PLMN. It is assumed that the NEF is configured based on HR-SBO roaming agreements for the Public IP address ranges with an HPLMN ID.

NOTE 7:	Whether the AF needs to use the NEF or not is according to local deployment. If the AF request is expected to possibly address PDU Sessions in HR-SBO mode, then the AF sends its requests via the NEF.

When AF requests are routed (by the AF or by the NEF) to an individual PCF this may use the BSF. This case applies to both AF influence on traffic routing as well as AF influence on Service Function Chaining, and Handling of Payload Headers. This is described in clause 4.3.6.4.

-	AF requests described in clause 5.6.7 of TS 23.501 [2], clause 5.6.16 or clause 5.6.x of TS 23.501 [2] targeting a group of UE(s), or any UE accessing a combination of DNN and S-NSSAI and optionally PLMN ID of the DNN and S-NSSAI, or targeting individual UE(s) by one or more GPSI(s) or targeting UEs with External Subscriber Category(s) which can be combined with External Group ID(s) or any UE as described in table 5.6.7-1. These AF requests may also affect UE(s) with an established PDU session. For such requests the AF shall contact the NEF and the NEF stores the AF request information in the UDR. For non-roaming and LBO cases, the PCF(s) receive a corresponding notification if they had subscribed to the create/update/delete operations of the AF request information corresponding to UDR Data Keys / Data Sub-Keys. This is defined in clause 6.3.7.2 of TS 23.501 [2] and further described in clause 4.3.6.2. For HR-SBO case, V-SMF(s) receive a corresponding notification if it has subscribed to the create/update/delete operation of the AF request information corresponding to UDR Data Keys /Data Sub-Keys, this is described in clause 4.3.6.5. For a single UE, NEF determines if HR-SBO applies for the PDU session based on the local configuration using the received GPSI. For "any UE", both PCF(s) and SMF(s) (via NEF) may need to subscribe to creates/updates/deletes of AF influence on traffic routing data in UDR depending on if a specific PLMN is targeted.

NOTE 8:	Such requests can target on-going or future PDU Sessions.

If the AF request targets any UE the procedure described in clause 4.3.6.5 may also be performed. In that case if HPLMN ID, HPLMN DNN and HPLMN S-NSSAI have been provided (see clause 4.3.6.5.2), if the HPLMN ID, HPLMN DNN and HPLMN S-NSSAI have not been provided by the AF, the NEF determines the HPLMN DNN and HPLMN S-NSSAI as described in clause 4.3.6.5.5.

This Release of the specification does not support AF influence on traffic routing for HR-SBO PDU session, if the AF request targets an external group ID, or UE(s) identified by SUPI.

If the AF interacts with 5GC via the NEF, the NEF performs the following mappings or determinations where needed:

-	Map the AF-Service-Identifier into DNN and S-NSSAI combination, determined by local configuration.

-	Map the AF-Service-Identifier into a list of DNAI(s) and Routing Profile ID(s) determined by local configuration.

The NEF can only provide this mapping when the DNAI(s) being used by the applications are statically defined. When the DNAI(s) where applications are instantiated may vary dynamically, the AF should provide the target DNAI(s) in its request together with either Routing Profile ID(s) or with N6 traffic routing information.

-	Map the GPSI in Target UE Identifier into SUPI, according to information received from UDM.

-	Map the External Group Identifier in Target UE Identifier into Internal Group Identifier, according to information received from UDM.

-	Map the External Subscriber Category(s) and any UE, or External Subscriber Category and External Group ID(s) to, Internal Group ID(s) or Internal Group ID(s) and Subscriber Category(s).

-	Map the geographical area in Spatial Validity Condition into areas of validity, determined by local configuration.

-	Determine whether an AF request targeting an UE IP address corresponds to HR-SBO and if yes determine the HPLMN of UE, the DNN/S-NSSAI of the PDU Session based on the description given above.

NOTE 9:	As a user can be associated with multiple Subscriber Category(s), some values of Subscriber Category(s) can correspond to an SLA between an application provider represented by an AF and the 5GC operator. In the NEF API, the combination of application identifier and External Subscriber Category can also be used to refer to this SLA.

#### 4.3.6.2	Processing AF requests to influence traffic routing and/or Service Function Chaining and/or for Handling of Payload Headers for Sessions not identified by an UE address

Figure 4.3.6.2-1: Processing AF requests to influence traffic routing and/or Service Function Chaining for Sessions not identified by an UE address

NOTE 1:	The 5GC functions used in this scenario are assumed to all belong to the same PLMN (HPLMN in non-roaming case or VPLMN in the case of a PDU Session in LBO mode).

0.	The PCF(s) subscribe to modifications of AF requests (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = S-NSSAI and/or DNN and/or Internal Group Identifier or SUPI) from the UDR.

1.	To create a new request, the AF invokes a Nnef_TrafficInfluence_Create service operation. The content of this service operation (AF request) is defined in clause 5.2.6.7. The request contains also an AF Transaction Id. If it subscribes to events related with PDU Sessions the AF indicates also where it desires to receive the corresponding notifications (AF notification reporting information).

To update or remove an existing request, the AF invokes a Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operation providing the corresponding AF Transaction Id.

The Nnef_TrafficInfluence_Create (initiated by target AF) or Nnef_TrafficInfluence_Update (initiated by source AF or target AF) service operation may be used for the case of AF instance change. If Nnef_TrafficInfluence_Update service operation is invoked, the NEF is required to update the subscription resource. The Nnef_TrafficInfluence_Update service operation may include an updated notification target address. The updated subscription resource is used by the target AF.

NOTE 2:	If the source AF transfers the application context to the target AF, then target AF may create new subscription via Nnef_TrafficInfluence_Create operation or update existing subscription via Nnef_TrafficInfluence_Update. However, whether and how the application context transfer is done is out of this specification.

2.	The AF sends its request to the NEF. If the request is sent directly from the AF to the PCF, the AF reaches the PCF selected for the existing PDU Session by configuration or by invoking Nbsf_management_Discovery service.

The NEF ensures the necessary authorization control, including throttling of AF requests and as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC.

3.	(in the case of Nnef_TrafficInfluence_Create or Update): The NEF stores the AF request information in the UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or to AF traffic influence request information for service function chaining, or to AF influence request for Handling of Payload Headers as described in Table 5.2.12.2.1-1.

NOTE 3:	Both the AF Transaction Internal ID and S-NSSAI and DNN and/or Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI are regarded as Data Key when the AF request information are stored into the UDR, see Table 5.2.12.2.1-1. The Subscriber Category(s) is determined by NEF as described in clause 4.3.6.1.

(in the case of Nnef_TrafficInfluence_delete): The NEF deletes the AF requirements in the UDR (Data Set = Application Data; Data Subset, Data Key = AF Transaction Internal ID). The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining, or to AF influence request for Handling of Payload  Headers.

The NEF responds to the AF.

4.	The PCF(s) that have subscribed to modifications of AF requests receive(s) a Nudr_DM_Notify notification of data change from the UDR. The Data Subset identifies whether the information relates to AF traffic influence request information for traffic routing or AF traffic influence request information for service function chaining.

5.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new policy information about the PDU Session by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 5 and 6 in clause 4.16.5.

The PCF validates whether the SFC identifier (if available) corresponds to an authorized SFC for the AF based on local configuration. If the validation has succeeded the PCF maps the SFC identifier to the corresponding Traffic Steering Policy identifier (i.e. TSP ID).

The PCF includes the Traffic Steering Policy ID(s) in the AF influence on traffic routing Enforcement Control information and/or N6-LAN Traffic Steering Enforcement Control information of the relevant PCC rule as defined in clause 6.3.1 of TS 23.503 [20]. The PCF also includes the Metadata in the N6-LAN Traffic Steering Enforcement Control information of the PCC rule when Metadata was provided by the AF.

If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

If the AF request includes an EAS Correlation indication or indication of traffic correlation, PCF includes in the PCC rule(s) an EAS Correlation indication or indication of traffic correlation and a Traffic Correlation ID corresponding to a set of UEs that AF request aims at, also if AF request includes a common EAS IP address or common DNAI for a set of UEs, PCC rule includes the common EAS IP address or common DNAI.

In the case of AF influence on traffic routing, the PCF may, optionally, use service experience analytics per UP path, as defined in clause 6.4.3 of TS 23.288 [50], to provide an updated list of DNAI(s) to the SMF.

The PCF may use the "Subscriber categories" as defined in "PDU Session policy control subscription information" in table 6.2-2 of TS 23.503 [20] to determine whether the PDU Session is impacted by the AF request.

6.	When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions according to the policy information.

In the case of AF influence on traffic routing, SMF may take the appropriate actions to reconfigure the User plane of the PDU Session. Examples of actions are:

-	The SMF may consider service experience analytics and/or DN Performance analytics per UP path (i.e. including UPF and/or DNAI and/or AS instance) as defined in clauses 6.4.3 and 6.14.3, respectively, of TS 23.288 [50] before taking any actions.

-	Determining a target DNAI.

-	Determining if a common DNAI needs to be used as a target DNAI.

-	Adding, replacing or removing a UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies).

-	Updating the UPF in the target DNAI with AF influence on traffic routing control parameters as described in clause 5.6.7.1 of TS 23.501 [2].

-	Subscribe to notifications from the AMF for an Area of Interest via Namf_EventExposure_Subscribe service operation.

-	Determining whether to relocate PSA UPF considering the user plane latency requirements provided by the AF (see clause 6.3.6 of TS 23.548 [74]).

When the updated policy information about the PDU Session is received from the PCF, the SMF may take appropriate actions to assist the EAS discovery and re-discovery for PDU Session with Session Breakout connectivity model such as:

-	Retrieve the EAS deployment information as defined in clause 6.2.3.4.1 of TS 23.548 [74].

-	Providing DNS message handling rule to forward DNS messages of the UE and/or report when detecting DNS messages as defined in clause 6.2.3.2.2 of TS 23.548 [74].

In the case of AF influence on Service Function Chaining, the SMF may take appropriate actions to enforce the N6-LAN traffic steering control:

-	Provide N6-LAN traffic steering control parameters to UPF as described in clause 5.6.16 of TS 23.501 [2].

In the case of AF influence on Handling of Payload Headers, the SMF may take appropriate actions by provisioning UPF with the Header Handling Control information, as described in clause 5.6.x of TS 23.501 [2].

7.	The SMF may decide whether it is required to send the target DNAI to the AMF for triggering SMF/I-SMF (re)selection and then inform the target DNAI information for the current PDU session or for the next PDU session to AMF via Nsmf_PDUSession_SMContextStatusNotify service operation.

# * * * * Next change * * * *

#### 4.3.6.4	Transferring an AF request targeting an individual UE address to the relevant PCF

Figure 4.3.6.4-1: Handling an AF request targeting an individual UE address to the relevant PCF

Depending on the AF deployment (see clause 6.2.10 of TS 23.501 [2]), the AF may send the AF request to PCF directly, in which case step 1 is skipped, or via the NEF.

1.	[Conditional] If the AF sends the AF request via NEF, the AF sends Nnef_TrafficInfluenceCreate/Update/Delete Request targeting an individual UE address to the NEF. This request corresponds to an AF request to influence traffic routing to a local network and/or to a service function chain that targets an individual UE address.

When NEF receives an AF request from AF, the NEF ensures the necessary authorization control and as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC. The NEF responds to the AF.

2.	[Conditional] AF/NEF consumes Nbsf_Management_Discovery service operation (providing at least the UE address) to find out the address of the relevant PCF if the PCF address is not available on the NEF based on local configuration, otherwise step 1 is skipped.

NOTE 1:	The AF/NEF finds the BSF based on local configuration or using the NRF.

3.	BSF provides the PCF address in the Nbsf_Management_Discovery response to AF/NEF.

4.	If step 1 was performed, NEF invokes the Npcf_PolicyAuthorization service to the PCF to transfer the AF request. If an AF sends the AF request directly to the PCF, AF invokes Npcf_PolicyAuthorization service and the PCF responds to the AF. To support the AF instance change, the Npcf_PolicyAuthorization_Create (initiated by target AF) or Npcf_PolicyAuthorization_Update (initiated by source AF or target AF) service operation may be used.

NOTE 2:	If the source AF transfers the application context to the target AF, then target AF may create new subscription via Npcf_PolicyAuthorization_Create or update existing subscription via Npcf_PolicyAuthorization_Update. However, whether and how the application context transfer is done is out of this specification.

5.	The PCF authorizes the AF request. If the PCF determines that the requirements can't be authorized, it rejects the AF request. Once the PCF authorizes the AF request, the PCF updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedure as described in clause 4.16.5.2.

The PCF includes the Traffic Steering Policy ID(s) for AF influence on traffic routing Enforcement Control information and/or N6-LAN Traffic Steering Enforcement Control information and/or Header Handling Control information in the relevant PCC rule as defined in clause 6.3.1 of TS 23.503 [20].

The PCF may, optionally, use service experience analytics per UP path, as defined in clause 6.4.3 of TS 23.288 [50], to provide a an updated list of DNAI(s) to the SMF.

If Npcf_PolicyAuthorization_Update service operation is invoked, the PCF is required to update the subscription resource. The Npcf_PolicyAuthorization_Update service operation may include an updated notification target address. The updated subscription resource is used by the target AF.

When a PCC rule is received from the PCF, the SMF may take appropriate actions, when applicable, to reconfigure the User plane of the PDU Session.

In the case of AF influence on traffic routing, examples of actions are:

-	The SMF may consider service experience analytics and/or DN Performance analytics per UP path (i.e. including UPF and/or DNAI and/or AS instance) as defined in clauses 6.4.3 and 6.14.3, respectively, of TS 23.288 [50] before taking any actions.

-	Determining a target DNAI and adding, replacing or removing UPF(s) in the data path, e.g. to act as UL CL, Branching Point and/or PDU Session Anchor e.g. as described in clause 4.3.5.

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies).

-	Updating the UPF regarding the target DNAI with AF influence on traffic routing control parameters as described in clause 5.6.7.1 of TS 23.501 [2].

-	Subscribe to notifications from the AMF for an Area of Interest via Namf_EventExposure_Subscribe service operation.

-	Determining whether to relocate PSA UPF considering the user plane latency requirements provided by the AF (see clause 6.3.6 of TS 23.548 [74]).

In the case of AF influence on Service Function Chaining, the SMF may take appropriate actions to enforce the N6-LAN traffic steering control:

-	Provide N6-LAN traffic steering control parameters to UPF as described in clause 5.6.16 of TS 23.501 [2].

In the case of AF influence on Handling of Payload Headers, the SMF may take appropriate actions by provisioning UPF with the Header Handling Control information, as described in clause 5.6.x of TS 23.501 [2].

# * * * * Next change * * * *

### 4.15.12	UPF Event Exposure Direct Reporting

#### 4.15.12.1    General

UPF may expose information directly via a service based interface by means of the UPF Event Exposure service described in clause 5.2.26.2 and as described in clause 5.6.x of TS 23.501 [2].

Exposure of QoS Montoring via direct interaction between UPF and Local NEF/AF is described in clause 4.15.12.1 and Notification of Handling of Payload Headers events in clause 4.15.12.2.

#### UPF event exposure direct reporting as part of UPF data collection procedures is described in clause 4.15.4.5.

#### 4.15.12.1   QoS monitoring exposure

This clause contains the description and the procedure for the exposure of QoS monitoring information via direct interaction between UPF (L-PSA UPF) and Local NEF/AF.

Editor's note:	It is FFS whether Local NEF is introduced or NEF is used instead.

Figure 4.15.12-1: Event exposure using Local NEF

1.	The UPF obtains QoS monitoring information as defined in clauses 5.8.2.18 and 5.45 of TS 23.501 [2].

2.	The UPF sends the notification related with QoS monitoring information over Nupf_EventExposure_Notify service operation according to the configuration from SMF as defined in clause 5.8.2.18 of TS 23.501 [2]. The notification is sent to the Notification Target Address that may correspond (4a) to the AF or (4b) to the Local NEF.

3.	If the Local NEF is used, it reports the QoS monitoring information to the AF by invoking Nnef_EventExposure_Notify service operation.

#### 4.15.12.x	Notification of Handling of Payload Header Events

When AF subscribes to UPF event for Handling of Payload Headers, it provides reporting control information (i.e. Header Handling Reporting Endpoints, etc) as described in clause 5.6.x of TS 23.501 [2], and PCF includes it in the PCC Rule.

When SMF receives a PCC Rule with Header Handling Control information, if it is including reporting control information for Handling of Payload Header Events, SMF includes it in the instructions to UPF as described in clause 5.8.5.6 of TS 23.501 [2]. The SMF may also decide based on local configuration to subscribe itself to notifications of Handling of Payload Header event as described in clause 5.6.x of TS 23.501 [2].

When a notification reporting trigger is met, UPF notifies the notification endpoint(s) (i.e. AF/NEF and/or SMF) using Nupf_EventExposure_Notify service operation according to the instructions received from SMF. Figure 4.3.6.x-1 shows the notification procedure.

Figure 4.3.6.x-1: Notification of Handling of Header Event

In this procedure, UPF has received instructions for Handling of Payload Headers including reporting control information as described in TS 23.501 [2] clause 5.8.5.6. At some moment in time:

1.  The conditions for notification of Handling of Payload Headers event is met.

2.  UPF sends notification(s) to the notification endpoint(s) received in the instructions from SMF by means of Nupf_EventExposure_Notify:

2a. to SMF, if SMF has subscribed to Handling of Payload Headers and it has included its own Notification Target address;

2b.	to AF, if AF has subscribed to Handling of Payload Headers event and the Notification Target address that UPF receives is the AF;

2c. to NEF, if AF has subscribed to Handling of Payload Headers event and the Notification Target address that UPF receives is the NEF.

NOTE: Trusted AFs receive notifications directly from UPF. If AF is untrusted, it receives the notifications via NEF.

3.   If NEF receives a Nupf_EventExposure_Notify from UPF (step 2c), it notifies to AF by means of Nnef_TrafficInfluence_Notify.

# * * * * Next change * * * *

##### 5.2.5.3.2	Npcf_PolicyAuthorization_Create service operation

Service operation name: Npcf_PolicyAuthorization_Create

Description: Authorize the request and optionally determines and installs SM Policy Control Data according to the information provided by the NF Consumer or provides Port Management Information Container for ports on DS-TT or NW-TT, or User plane node Management Information Container.

Inputs, Required: UE (IP or MAC) address, identification of the application session context.

Inputs, Optional: GPSI(s) or SUPI(s) if available, Internal Group Identifier, DNN if available, S-NSSAI if available, Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description information as described in clause 6.1.3.6 of TS 23.503 [20], AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, emergency indicator, ASP Identifier, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation, EAS Correlation indication, Common EAS IP address, Traffic Correlation ID, FQDN(s) as described in clause 5.6.7 in 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], TSCTSF parameters provided by the TSCTSF to the PCF as described in clause 6.1.3.23a and clause 6.1.3.23b of TS 23.503 [20], QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], RT Latency Indication as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), TSC Assistance Container, MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20], Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], SFC Identifier(s), Metadata, Header Handling Control information as described in clause 5.6.x of TS 23.501 [2], Periodicity as described clauses 6.1.3.22 and 6.3.1 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description as described in clause 5.37.5 or 5.37.8.3 of TS 23.501 [2], Data Burst Handing Information as described in clause 6.3.1of TS 23.503 [20], Indication of ECN marking for L4S as described in clause 6.1.3.22 of TS 23.503 [20], Notification Target Address for PMIC/UMIC UPF event, Correlation ID for PMIC/UMIC UPF event, Multi-Modal Service ID together with Multi-modal Service Requirements information for each data flow as described in clause 6.1.3.27.3 of TS 23.503 [20], QoS duration, QoS inactivity interval as described in clause 6.1.3.22 of TS 23.503 [20].

NOTE 1:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

NOTE 2:	A dedicated Notification Target Address for PMIC/UMIC UPF event and Correlation ID for PMIC/UMIC UPF event are provided by the event consumer over Npcf_PolicyAuthorization as the corresponding events are reported by the UPF and not by the PCF. Providing such information indicates that the consumer of the Npcf_PolicyAuthorization (TSN AF, TSCTSF) supports PMIC/UMIC via Nupf event reporting.

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clauses 6.1.3.16 and clause 6.1.3.10 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

##### 5.2.5.3.3	Npcf_PolicyAuthorization_Update service operation

Service operation name: Npcf_PolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: Media type, Media format, bandwidth requirements, sponsored data connectivity information if applicable, flow description information as described in clause 6.1.3.6 of TS 23.503 [20], AF Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition, spatial validity condition, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation, AF indication for simultaneous connectivity over source and target PSA at edge relocation as described in clause 5.6.7 of TS 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 of TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 of TS 23.503 [20], Port Management Information Container and related port number, User plane node Management Information Container, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.3.23 of TS 23.503 [20], TSCTSF parameters provided by the TSCTSF to the PCF as described in clause 6.1.3.23a and clause 6.1.3.23b of TS 23.503 [20], QoS Reference or individual QoS parameters as described in clause 6.1.3.22 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS Reference parameters or Requested Alternative QoS Parameter Sets in a prioritized order), TSC Assistance Container, QoS Monitoring parameter(s) as defined in clause 5.45 of TS 23.501 [2], Reporting frequency, Target of reporting and optional an indication of direct event notification as described in clause 6.1.3.21 of TS 23.503 [20], MPS for Data Transport Service indicator as described in clause 6.1.3.11 of TS 23.503 [20], Packet Delay Variation requirements as described in clause 6.1.3.26 of TS 23.503 [20], SFC Identifier(s), Metadata, Header Handling Control information as described in clause 5.6.x of TS 23.501 [2], Periodicity as described clauses 6.1.3.22 and 6.3.1 of TS 23.503 [20], PDU Set QoS Parameters as described in clause 5.7.7 of TS 23.501 [2], Protocol Description as described in clause 5.37.5 or 5.37.8.3 of TS 23.501 [2], Data Burst Handing Information as described in clause 6.3.1of TS 23.503 [20], Notification Target Address for PMIC/UMIC UPF event, Correlation ID for PMIC/UMIC UPF event, updated information for Multi-modal Service Requirements as described in clause 6.1.3.27.3 of TS 23.503 [20].

NOTE:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in clause 6.1.3.16 of TS 23.503 [20]).

Outputs, Optional: The service information that can be accepted by the PCF.

Provides updated application level information and communicates with Npcf_SMPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

# * * * * Next change * * * *

##### 5.2.12.2.1	General

The operations defined for Nudr_DM service use following set of parameters defined in this clause:

-	Data Set Identifier: uniquely identifies the requested set of data within the UDR (see clause 4.2.5).

-	Data Subset Identifier: it uniquely identifies the data subset within each Data Set Identifier. As specified in the procedures in clause 4, e.g. subscription data can consist of subsets particularised for specific procedures like mobility, session, etc.

-	Data Keys defined in Table 5.2.12.2.1-1

For Nudr_DM_Subscribe and Nudr_DM_Notify operations:

-	The Target of Event Reporting is made up of a Data Key and possibly a Data Sub Key both defined in Table 5.2.12.2.1-1. When a Data Sub Key is defined in the table but not present in the Nudr_DM_Subscribe this means that all values of the Data Sub Key are targeted.

-	The Data Set Identifier plus (if present) the (set of) Data Subset Identifier(s) corresponds to a (set of) Event ID(s) as defined in clause 4.15.1

An NF Service Consumer may include an indicator when it invokes Nudr_DM Query/Create/Update service operation to subscribe the changes of the data, to avoid a separate Nudr_DM_Subscribe service operation.

Depending on the use case, it is possible to use a Data Key and/or one or multiple Data sub keys to further identify the corresponding data, as defined in Table 5.2.12.2.1-1 below.

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMF Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMS Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  |  |  | Serving PLMN ID and optionally NID |
|  | Slice Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Group Data (NOTE 5) | Internal Group Identifier or External Group Identifier | - |
|  | Identifier translation | GPSI |  |
|  |  | SUPI | Application Port ID, MTC Provider Information, AF Identifier |
|  | Intersystem continuity Context | SUPI | DNN |
|  | LCS privacy | SUPI | - |
|  | LCS mobile origination | SUPI | - |
|  | UE reachability | SUPI | - |
|  | Group Identifier Translation | Internal Group Identifier or External Group Identifier | - |
|  | UE context in SMSF data | SUPI | - |
|  | V2X Subscription data | SUPI | - |
|  | A2X Subscription data | SUPI | - |
|  | ProSe Subscription data | SUPI | - |
|  | Ranging/SL Positioning subscription data | SUPI | - |
|  | User consent | SUPI | Purpose |
|  | ECS Address Configuration Information (See Table 4.15.6.3d-1) | SUPI, Internal group identifier or external group identifier or any UE | DNN, S-NSSAI, (Serving) PLMN ID (NOTE 7) |
|  | MBS Subscription data (see clause 6.4.3 of TS 23.247 [78]) | SUPI | - |
|  | Ranging/Sidelink Positioning Subscription data | SUPI | - |
|  | Ranging/Sidelink Positioning privacy | SUPI | - |
|  | Operator Determined Barring data (see clause 2.3 of TS 23.015 [90] and TS 29.505 [91]) | SUPI | - |
|  | Shared data | Shared Data ID | - |
| Application data | Packet Flow Descriptions (PFDs) (NOTE 11) | Application Identifier |  |
|  | AF traffic influence request information for traffic routing | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 of TS 23.501 [2]) | For non-roaming and LBO: S-NSSAI and DNN , accompanied with Internal Group Identifier(s) and/or Subscriber Category(s) or SUPI or "any UE" indication For HR-SBO: HPLMN S-NSSAI and DNN and either: HPLMN ID and IP address, or SUPI, or "any UE" indication and HPLMN ID. (NOTE 4) (NOTE 6) (NOTE 12) |  |
|  | AF traffic influence request information for service function chaining | AF transaction internal ID |  |
|  | (See clause 5.6.16 and clause 6.3.7.2 of TS 23.501 [2]) | S-NSSAI and DNN and Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
|  | AF traffic influence request information for Handling of Payload Headers (See clause 5.6.x and clause 6.3.7.2 of TS 23.501 [2]) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) or "PLMN ID(s) of inbound roamer" |  |
|  | UE ID mapping information (See clause 4.3.5 of TS 23.586 [88]) | GPSI or Application Layer ID |  |
|  | EAS Deployment Information (See clause 7.1 of TS 23.548 [74]) | DNN and/or S-NSSAI | Application Identifier and/or Internal Group Identifier |
|  | ECS Address Configuration Information (See Table 4.15.6.3d-1) (NOTE 13) | DNN, S-NSSAI and "any UE" indication |  |
|  | AM influence information (See clause 4.15.6.9.3) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or "any UE" indication or any inbound roaming UEs (NOTE 4, NOTE 8) |  |
|  | AF request for QoS information (See clause 4.15.6.14) | AF transaction internal ID |  |
|  |  | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or "any UE" indication (NOTE 4) |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 of TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 of TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 of TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 of TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
|  | Network Slice Specific Control Data (See clause 6.2.1.3 of TS 23.503 [20]) | S-NSSAI |  |
|  | 5G VN Group Specific Control Data (See clause 6.2.1.3 of TS 23.503 [20]) | S-NSSAI and DNN and/or Internal Group Identifier |  |
|  | Operator Specific Data | SUPI or GPSI |  |
|  | Planned Data Transfer with QoS requirements data (See clause 6.2.1.6 of TS 23.503 [20]) | PDTQ Reference ID. (NOTE 10) |  |
|  |  | None. (NOTE 9) |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
|  | DNAI mapping information | DNN and/or S-NSSAI |  |
| NOTE 1: Retrieval of the stored Background Data Transfer data for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 2: Update of a Background Data Transfer data in the UDR requires a Data key to refer to a Background Data Transfer data as input data. NOTE 3: The Background Data Transfer includes the Background Data Reference ID and the ASP Identifier that requests to apply the Background Data Reference ID to the UE(s). Furthermore, the Background Data Transfer includes the relevant information received from the AF as defined in clause 6.1.2.4 of TS 23.503 [20]. NOTE 4: When the Data Key targets "any UE", then the request to UDR applies on Application data that applies on all subscribers of the PLMN. For encoding, see TS 29.519 [82]. NOTE 5: Group Data includes 5G VN group configuration, DNN and S-NSSAI specific Group Parameters and any other data related to a group stored in the UDR. NOTE 6: If a list of Internal Group IDs is used, the AF traffic influence request information request applies to the UEs that belong to every one of these groups, i.e. a single UE needs to be a member of every group in the list of Internal Group IDs. NOTE 7: When the Data Key targets "PLMN ID", then the request to UDR applies on subscription data about subscribers roaming in this PLMN. NOTE 8: In LBO roaming scenarios, when the AF request targets "any inbound roaming UEs", the AM influence information applies to the roaming subscribers from a PLMN or from any PLMN. NOTE 9: Retrieval of the stored Planned Data Transfer with QoS requirements data for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 10: Update of a Planned Data Transfer with QoS requirements data in the UDR requires a Data key to refer to a Planned Data Transfer with QoS requirements data as input data. NOTE 11: Each PFD (as defined in TS 23.503 [20]) may be complimented with a source NF type which indicates the type of NF that has generated the PFD (i.e. AF or NWDAF). Absence of a source NF type indicates that the AF is the source of the PFD. NOTE 12: Further information about HR-SBO case and how these keys are used, see clause 4.3.6.1. NOTE 13: The ECS Address Configuration Information as part of application data is used for HR roaming case as defined in clause 6.5.2.6 of TS 23.548 [74]. |  |  |  |

The content of the UDR storage for (Data Set Id= Application Data, Data Subset Id = AF TrafficInfluence request information) is specified in clause 5.6.7, Table 5.6.7-1 of TS 23.501 [2], in clause 5.6.16, Table 5.6.16.2-1 of TS 23.501 [2] and in clause 5.6.x, Table 5.6.x.2-1 of TS 23.501 [2]. This information is written by the NEF and read by the PCF(s). PCF(s) may also subscribe to changes onto this information.

# * * * * Next change * * * *

##### 5.2.6.7.2	Nnef_TrafficInfluence_Create operation

Service operation name: Nnef_TrafficInfluence_Create

Description: Authorize the request and forward the request for traffic influence.

Inputs, Required: AF Transaction Id, AF Identifier.

The AF Transaction Id refers to the request.

Inputs, Optional: The address (IP or Ethernet) of one or more UE(s) if available, one or more GPSIs if available, DNN if available, S-NSSAI if available, External Group Identifier(s) if available, External Application Identifier or traffic filtering information, AF-Service-Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, Indication of traffic correlation, Indication of application relocation possibility, Indication of UE IP address preservation, Early and/or late notifications about UP path management events, Notification Target Address, immediate reporting flag, Temporal validity condition, Spatial validity condition, User Plane Latency Requirements, Information for EAS IP Replacement in 5GC, Indication for EAS Relocation and AF indication for simultaneous connectivity over source and target PSA at edge relocation, EAS Correlation indication, External Subscriber Category(s), SFC Identifier(s), Metadata, Common EAS IP address, Traffic Correlation ID, FQDN(s) and Header Handling Control information as described in clause 5.6.7, clause 5.6.16 and clause 5.6.x of TS 23.501 [2], HPLMN of the UE. If an IP address of one or more UE(s) is provided, a corresponding port number (e.g. TCP or UPDP) for each IP address.

The IP address of one or more UE(s) may be provided together with the corresponding port number allowing to support the case where the PSA UPF is carrying out NAT on the traffic exchanged with the EAS.

NOTE 1:	When only one DNAI and corresponding routing profile ID(s) and the Indication for EAS Relocation are available, the presented DNAI is the target DNAI as defined in clause 6.3.7 of TS 23.548 [74].

NOTE 2:	In this Release the following cannot be used by AFs in VPLMN influencing traffic on Home Routed PDU sessions:

-	External Group Identifier;

-	External Subscriber Category.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Next change * * * *

#### 5.2.26.1	General

The following table shows the UPF Services and UPF Service Operations.

Table 5.2.26.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Notify | Subscribe/Notify | NEF, AF, NWDAF, TSNAF/DCCF/MFAF/TSCTSF, SMF |
|  | Subscribe | Subscribe/Notify | NWDAF/DCCF, SMF |
|  | Unsubscribe | Subscribe/Notify | NWDAF/DCCF, SMF |
| Nupf_GetUEPrivateIPaddrAndIdentifiers | Get | Request/Response | NEF |

# * * * * Next change * * * *

##### 5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are several operations for this service:

-	Notifying events on the PDU Session to the NFs.

-	Allow consumer NFs to subscribe and unsubscribe for an Event ID on UPF.

The following events can be notified to a NF consumer:

-	QoS Monitoring. This event provides QoS Flow level performance information and can be used for direct event notification of QoS Monitoring reports to a different NF than the SMF as defined in clause 5.8.2.18 of TS 23.501 [2], or it may be for UPF Data collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

Subscription to this event is always indirect via SMF. The subscription specifies the type of measurement that is being requested. A combination of the information listed below can be requested.

UPF and SMF interact using Session Reporting Rules as defined in clause 5.8.5.11 of TS 23.501 [2].

The event notification may contain following information:

-	QoS monitoring result for the QoS monitoring parameter(s) defined in clause 5.45 of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay.

-	Indication of QoS Flow associated with the default QoS Rule (if requested by SMF, see clause 4.15.4.5.1).

-	User Data Usage Measures or User Data Usage Trends. These events provide information about traffic matching Event Filter Information (see below) for a user's PDU Session or for each PDU Session served by the UPF. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

SMF shall use Service Based Interface subscription service operation to subscribe this UPF event. Other direct consumers can subscribe directly to UPF under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]. The subscription request contains the Type of Measurement that is being requested, the Target of Event Reporting and the Event Filter Information (which describes the target traffic) and the required Granularity of Measurement. The Target of Event Reporting can be either a specific PDU Session or "any UE". The Event Filter Information may contain one or more of the following parameters: DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information. The Granularity of Measurement can indicate per data flow, per application, or per PDU Session. A combination of the information listed below for User Data Usage Measures can be requested.

For User Data Usage Measures, the event notification may contain following event related information:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurement.

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurement.

-	Application related Information: URL(s) and/or Domain information (domain name and protocol) detected for the target traffic. This Type of Measurement requires that Application Id(s) or Traffic Filtering Information is provided (i.e. this measurement is not possible to be applied for all traffic handled by the UPF).

For User Data Usage Trends, the event notification may contain following event related information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement period determined for the requested Granularity of Measurement.

For User Data Usage Measures and User Data Usage Trends, if the subscription is targeting any UE, the UPF shall perform the requested measurements for every PDU Session that matches the Event Filter Information and send the information per PDU Session, including the DNN and S-NSSAI and optionally, the sampling ratio achieved by the UPF. When the subscription requests that information is provided per data flow, the notification includes the Packet Filter Set and the Applications Identifier, if available.

NOTE 1:	If the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact of the per PDU Session measurement.

NOTE 2:	The UPF can combine information for different PDU Sessions into one notification message (see TS 29.564 [93]).

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.5.14 of TS 23.501 [2].

TSC management information event can be the result of an implicit subscription of the PCF on behalf of the TSNAF/TSCTSF via the Npcf_SMPolicyControl service as described in clause 5.28 of TS 23.501 [2].

-	UE NAT mapping. This event supports the NF consumer to subscribe this UPF event to obtain the NATed UE public IP address and port number with the parameters of UE (private) IP address, IP address of the remote end, DNN and S-NSSAI associated with the AF ID, as well as the IP domain, as defined in clause 5.8.2.17 of TS 23.501 [2].

-	Handling of Payload Headers. This event reports that the Handling of Payload Header request has been enforced by UPF as defined in clause 5.6.x of TS 23.501 [2].

Handling of Payload Headers event results out of an implicit subscription received by UPF over N4 triggered by an AF subscription request to PCF via the Npcf_SMPolicyControl service or by the decision in SMF to request notifications of the enforcement of the Handling of Payload Headers instructions provided to UPF.

If the consumer of UPF service is NWDAF and the target of UE is any UE, according to the Analytic ID from consumer, the NWDAF can decide which kind of information should be collect from UPF and event ID to use. Subscription may be performed directly towards UPF or via SMF as described in clause 5.8.2.17 of TS 23.501 [2]. The UPF collects the data according to the event ID and exposes the related information directly regardless of whether the subscription has been relayed by the SMF.

# * * * * End of changes * * * *