3GPP TSG-SA WG2#165	S2-2410746

Hyderabad, India, 14-18 October 2024	(was S2-24010523)

Source:	Qualcomm Incorporated, MediaTek Inc., Samsung, OPPO

Title:	Way forward for Ambient IoT architecture

Document for:	Discussion/Approval

Agenda Item:	19.14.1

Work Item / Release:	Rel-19 / FS_AmbientIoT

Abstract of the contribution: This paper proposes a way forward for the Ambient IoT architecture.

# 1.	Text proposal

It is proposed to agree the following changes to TR 23.700-13.

# >>>>BEGINNING OF CHANGES<<<<

## 8.1	Interim Conclusion on Key Issue #1

### 8.1.3	Additional architecture principles for Topology 2

#### 8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2

-	RRC-based option as defined in clause 8.1.3.3

Editor’s note: Whether a NAS-based option is needed in addition is FFS.

#### 8.1.3.2	User-plane option

The following principles apply:

-	As depicted in Figure 8.1.3.2-X, the UE Reader connects to the AIoTF based on the AIoT Application Protocol (AIoT-AP) using an IP PDU Session between the UE and the UPF as transport. The related protocol stack is shown in Figure 8.1.3.2-Y. The AIoT AP protocol will support procedures and information to be exchanged as specified by RAN2, RAN3 and SA2.

Figure 8.1.3.2-X:	User-plane architecture for Topology 2

Figure 8.1.3.2-Y:	Protocol stack for the user-plane architecture for Topology 2

NOTE 2:	Which transport protocol to use for AIoT-AP can be decided by CT1 in coordination with RAN3.

NOTE 3:	Security for AIoT-AP is assumed to be defined by SA3.

Editor’s note: Further details are FFS.

#### 8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC and a protocol between gNB/AIoT RAN and AIoTF. The related protocol stack options are shown in Figure 8.1.3.3-X and Figure 8.1.3.3-Y.

Editor’s note: Which option to use, i.e., whether gNB/AIoT RAN connect to AIoTF directly or via AMF is FFS.

Editor's note: Further details are FFS.

Figure 8.1.3.3-X:	Protocol Stack for the RRC option (Option A)

Figure 8.1.3.3-Y:	Protocol Stack for the RRC option (Option B)

# >>>>END OF CHANGES<<<<