3GPP TSG-WG SA2 Meeting #165	S2-2410771

Hyderabad, Oct 14 – 18, 2024	(revision of S2-2410239)

Source:	Nokia, Samsung, Futurewei

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#3 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to update the conclusions for KI#3.

RAN has reached the agreements at RAN2#127 as shown in the following table:

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v1.0.0.

FIRST CHANGE

## 8.3	Interim Conclusion on Key Issue #3

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices;

NOTE :	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.r

Editor's note:	Additional conclusions are FFS.

### 8.3.1	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device non-volatile memory.

-	Write: Request to write information to an AIoT Device non-volatile memory.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

### Editor's note:	It is FFS about the solution of enable/(temporary) disable/permanent disable AIoT devices.

### 8.3.2	Principles on the procedures to support AIoT services

Editor's note:	Procedure principles will be captured in this clause based on the outcomes of KI#1 and KI#2.

### 8.3.3	Information provided by the AF

The services, service operations, and related parameters are common to both topology 1 and topology 2, providing the AF with a unified interface. As a result, the AF does not need to differentiate between topology 1 and topology 2.

The AF, common to all service requests, provides one or some of the following information:

- Information about AIoT services (e.g.,

- target area information)

- AIoT Reader Information.

- Information about the Target AIoT device (s) (e.g., Device IDs, Approximate number of device)

Editor's note:	What information can be provided as Information about AIoT services is FFS.

Editor's note:	What information can be provided as AIoT Reader Information is FFS.

Editor's note:	What information can be provided as information about the Target AIoT device (s) is FFS.

Editor's note:	It is for FFS what other information is included in AIoT service request.

### 8.3.4	NEF exposure

NEF supports to expose Ambient IoT service towards the AF.

Editor's note:	It is for FFS what AIoT services can be exposed by NEF.

END OF CHANGES