3GPP TSG-WG2 Meeting #165	S2-2410808

14 - 18 October, 2024, Hyderabad, India	(was S2-2410036)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5452 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to UPF data exposure for KI#2 direct subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Huawei? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-09-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR submitted in SA2#164 was not treated and the implemented TS 23.501 CR 5540r3 still has below issues need to be resolved. - Missing DNN and S-NSSAI not aligned with other related TSes: When the UE IP address is provided to discover the target UPF, DNN and S-NSSAI information is also needed. The IP address information is included in the DnnUpfInfoItem of SnssaiUpfInfoItem data type within the UpfInfo data type as defined in TS 29.510. The step 1 of clause 4.15.4.5.5 in TS 23.502 is also including DNN and S-NSSAI. Hence the DNN and S-NSSAI in this CR is needed to align with stage2 & stage 3 TS definitions. - The NOTE description on the case when ULCL/BP is used for the PDU Session is not straightforward and not fully correct: The Note added in this CR with registration of IP address information in the NRF as condition to discover the local PSA UPF is directly aligned with the procedure to be kept and the UPF supporting ULCL/BP still can support traffic measurement and PSA functionality as below, i.e., the limitation of ULCL/BP used for the PDU Session should be removed in the NOTE: TS 23.501 clause 5.6.4.2: The UL CL applies filtering rules (e.g. to examine the destination IP address/Prefix of UL IP packets sent by the UE) and determines how the packet should be routed. The UPF supporting an UL CL may also be controlled by the SMF to support traffic measurement for charging, traffic replication for LI and bit rate enforcement (Session-AMBR per PDU Session). … NOTE 2: The UPF supporting an UL CL may also support a PDU Session Anchor for connectivity to the local access to the data network (including e.g. support of tunnelling or NAT on N6). This is controlled by the SMF. … NOTE 3: It is possible for a given UPF to support both the UL CL and the PDU Session Anchor functionalities. TS 23.501 clause 5.6.4.3 The UPF supporting a Branching Point functionality may also be controlled by the SMF to support traffic measurement for charging, traffic replication for LI and bit rate enforcement (Session-AMBR per PDU Session). … NOTE 2: It is possible for a given UPF to support both the Branching Point and the PDU Session Anchor functionalities. Hence propose to handle this CR with the changes in clause 5.8.2.17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.8.2.17, update the description and NOTE when UE IP address is provided to discover the PSA UPF and subscribe directly for data collection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not fully aligned with TR 23.700-63 v1.0.0 conclusion on KI#2 use UE IP address to discover the PSA UPF and subscribe directly to PSA UPF for data collection, also not align with other related TS definitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 5.8.2.17	Data exposure via Service Based interface

The UPF may expose information by means of UPF Event Exposure service as described in TS 23.502 [3] clause 5.2.26.2, via a service-based interface directly. The NF consumers, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF and NWDAF/DCCF/MFAF.

When the UPF supports the data exposure via the service based interface, it may register its NF profile to the NRF including the UPF Event Exposure services and the related Event ID(s).

For data collection from UPF (see clause 4.15.4.5 of TS 23.502 [3]), NF consumers can do the subscription to the UPF either directly or indirectly via SMF. An NF consumer may subscribe to the UPF Event Exposure service directly if target of data collection is "any UE" (e.g. to collect user data usage information for NWDAF NF Load analytics (see clause 6.5 of TS 23.288 [86])) or if target of data collection is a UE IP address together with the DNN and S-NSSAI, and if the subscription is for User DataUsage events (as described in clause 4.15.4.5.1 of TS 23.502 [3]) and is not including any of the following parameters: AoI, BSSID/SSID and DNAI. Otherwise, the NF consumer shall subscribe indirectly via SMF.

NOTE:	The UE IP address together with the DNN and S-NSSAI is used to discover the PSA UPF serving the particular UE IP address. The NF consumer cannot use the UE IP address to discover the local PSA UPF(s) if this UPF does not register the corresponding IPv4 address/IPv6 prefix range information in the NRF.

When the UPF changes for the scenario of UPF relocation, if the UPF event subscription for a UE is directly sent to the UPF, the UPF may send a notification with a subscription termination indication and an optional cause code to the UPF event consumer. In the case of subscribing to UPF event via SMF or directly to UPF, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet when the UPF changes (see clause 4.15.4.5 of TS 23.502 [3]).

To alleviate the load of UPF due to frequent event notification for data collection related events, the event subscription may include Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting window is also provided, which defines the last valid reporting time, and UPF shall report the event before that time. If the Reporting suggestion information allows this, the UPF can concatenate several event reports (of the same event) to the same notification endpoint into one notification message.

The UPF may also expose UE information by means of the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 5.2.26.3 of TS 23.502 [3]. An UPF which is deployed with NAPT (Network Address Port Translation) functionality may support to provide the 5GC UE IP address to NEF based on NEF request containing public IP address and port number using the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 4.15.10 of TS 23.502 [3] for AF specific UE ID retrieval.

The UPF event exposure service may allow NF consumer to obtain one NATed UE public IP address and Port number for a particular PDU Session, based on the UE private IP address allocated by 5GC. An UPF which is deployed with NAT functionality may support to provide the UE public IP address and port number to consumer based on Nupf_EventExposure_Subscribe request as defined in clause 5.2.26.2.3 of TS 23.502 [3].  For information flow for getting UE public IP address and port number, see clause 6.2.8.2.4 of TS 23.288 [5].

*** End of Changes ***