3GPP TSG-WG2 Meeting #165	S2-2410809

14 - 18 October, 2024, Hyderabad, India	(was S2-2410731)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4876 | rev | 3 | Current version: | 19.1.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to UPF data exposure for KI#2 direct subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedures in clause 4.15.4.5.5 are unclear on how to handle the case when the Nnrf_NFDiscovery response does not contain the discovered UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify how to handle when the Nnrf_NFDiscovery response does not contains the discovered UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not completely aligned with the TR 23.700-63 v1.0.0 conclusion on KI#2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 4.15.4.5.5	Information flow for direct subscription to UPF event exposure service for certain UE using UE's IP address

Figure 4.15.4.5.5-1: Subscription to UPF event exposure service for certain UE(s) directly using UE's IP address

In the case, UPF event consumer (e.g. AF (when in trusted domain) or NEF) is aware of the UE IP address and the UPF Event ID(s) that are to be subscribed meets the criteria for direct subscription to UPF as defined in clause 5.8.2.17 of TS 23.501 [2]), then UPF Event consumer can subscribe to UPF directly for Event Exposure using the following procedure:

1.	The UPF event consumer triggers the UPF discovery to NRF by Nnrf_NFDiscovery_Request providing the UE IP address information and optionally the DNN, S-NSSAI, and IP domain if IPv4 address is provided. UE IP address provided to NRF maybe the IP address as seen by the DN (e.g. a Public IP address)

2.	The NRF responds with Nnrf_NFDiscovery response message including the information of UPF for the provided UE IP address.

If step 2 does not include any discovered UPF, the information flow described in clause 4.15.4.5.2 can be used instead.

3.	The UPF event consumer sends the Nupf_EventExposure Subscription request for the UPF Event(s) including the following information:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, either Application Id(s) or Traffic Filtering Information.

-	Target of Event Reporting: UE IP address.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

NOTE:	The case when NAT is deployed in UPF is not specified.

4.	As per the information provided in the previous step, UPF reports the collected data to the UPF event consumer by invoking Nupf_EventExposure Notify.

*** End of Changes ***