3GPP TSG-SA2 Meeting #165	S2-2410838

# Hyderabad, India, 14 - 18 October, 2024								                (was S2-2410582)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0609 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of LCS Correlation ID in NRPPa |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 38.305, “Signalling access between the LMF and gNB for non-UE associated NRPPa procedure in Clause 7.2.1 may be via any AMF with signalling access to both the LMF and gNB.” As per TS 29.518, “The LCS Correlation ID shall only be applied when transferring LCS related UE-Specific N1 and/or N2 messages.” But in clause 6.11.2 in TS 23.273, it is not described as UE associated NRPPa procedure. It is not described that only serving AMF applies in this case. And in clause 6.11.3 in TS 23.273, it applies to only NRPPa assistance without measurements’ procedures. It is not described that any AMF applies in this case. LCS Correlation ID is not clarified for its applicability that it is mandatory for UE associated NRPPa transport over serving AMF. And ‘Obtain Measurements’ in clause 6.11.2 has confusing wording to the Measurement Information Transfer (in TS 38.455) which is non-UE associated and not applicable to the clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding ‘any AMF’ case in 6.11.2 and 6.11.2 to sync with TS 38.305 - Clarifies the applicability of LCS Correlation ID to sync with TS 29.518 - The changes also apply to the diagram for consistency |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not consistent between SA2 and RAN and CT about non-UE and UE associated NRPPa transport and related LCS Correlation ID logic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11.2, 6.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

### 6.11.2	Network Assisted Positioning Procedure

Figure 6.11.2-1 shows a UE associated NRPPa procedure that may be used by an LMF to support network assisted and network based positioning. The procedure is based on an NRPPa protocol in TS 38.455 [15] between the LMF and NG-RAN via serving AMF.

Figure 6.11.2-1: UE Associated NRPPa Procedure for Network Assisted Positioning Procedure

Precondition:	A LCS Correlation identifier and the AMF identity have been passed to the LMF by the serving AMF. In the case of PRU, LCS Correlation identifier is generated by LMF and provided to AMF during PRU Registration Accept message.

1.	The LMF invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a Network Positioning message to the serving NG-RAN node (gNB or ng-eNB) for the UE. The service operation includes the Network Positioning message and may indicate if the positioning is initiated towards a PRU and the LCS Correlation identifier. The Network Positioning message may request location information for the UE from the NG-RAN and may include UE unaware indication if it is received by LMF from AMF. LMF may provide an LPHAP Assistance Information to RAN in an NRPPa message, if LPHAP indication is received in step 11 clause 6.1.2.

2.	If the UE is in CM IDLE state and there is no UE unaware indication for the UE in the UE's location context, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19], to establish a signalling connection with the UE. If positioning towards a PRU is indicated in step 1, the AMF verifies the UE is a valid PRU before initiating the procedure.

If the UE is in CM-IDLE state and there is UE unaware indication for the UE in the UE's location context, the AMF should reject the forwarding request of the Network Positioning message with a proper rejection cause value and steps 3-6 are skipped.

3.	The AMF forwards the Network Positioning message to the serving NG-RAN node in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF.

4.	The serving NG-RAN node  handles the request from step 3.

If UE unaware indication is received in the Network Positioning message, and the UE is in RRC_INACTIVE state, the NG-RAN rejects the Network Positioning message with appropriate rejection cause (e.g. UE cannot be paged).

5.	The serving NG-RAN node returns any location information obtained in step 4 to the AMF in a Network Positioning message included in an N2 Transport message. The serving NG-RAN node shall also include the Routing identifier in the N2 Transport message received in step 3.

6.	The AMF invokes the Namf_Communication_N2InfoNotify service towards the LMF indicated by the routing identifier received in step 5. The service operation includes the Network Positioning message received in step 5 and the LCS Correlation identifier. Steps 1 to 6 may be repeated to request further location information and further NG-RAN capabilities.

* * * Second Change * * *

### 6.11.3	Obtaining Non-UE Associated Network Assistance Data

Figure 6.11.3-1 shows a non-UE associated NRPPa procedure which may be used by an LMF to support network assisted and network based positioning. This procedure is not associated with a UE location session, e.g., it can be used to obtain network assistance data from a NG-RAN node (e.g. gNB or ng-eNB). The procedure is based on an NRPPa protocol in TS 38.455 [15] between the LMF and NG-RAN via any AMF as per TS 38.305.

Figure 6.11.3-1: non-UE Associated Procedure

NOTE:	To support location service in the PNI-NPN, local AMF is used, as defined in clause 5.13.

1.	The LMF invokes the Namf_Communication_NonUeN2MessageTransfer service operation towards a AMF (any AMF) to request the transfer of non-UE associated message defined in TS 38.455 [15] to a NG-RAN node (gNB or ng-eNB) in the NG-RAN. The service operation may include the Network Positioning message and the target NG-RAN node identity. The Network Positioning message may request position related information from the NG-RAN.

2.	The AMF forwards the Network Positioning message to the target NG-RAN node indicated in step 1 in an N2 Transport message. The AMF includes a Routing identifier, in the N2 Transport message, identifying the LMF.

3.	The target NG-RAN node obtains any position related information requested in step 2.

4.	The target NG-RAN node returns any position related information obtained in step 3 to the AMF in an N2 Transport message. The target NG-RAN node shall also include the Routing identifier in the N2 Transport message received in step 2.

5.	The AMF invokes the Namf_Communication_NonUeN2InfoNotify service operation towards the LMF indicated by the routing identifier received in step 4. The service operation includes the response message received in step 4. Steps 1 to 5 may be repeated as needed.

NOTE:	LCS Correlation ID is not applicable to this procedure.

* * * End of Change * * *