3GPP TSG SA WG2#165	S2-2410849

# Hyderabad, India, 14-18 October 2024									    	(revision of S2-2410561)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5806 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling access to local services via CAG cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_Femto |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 5G_Femto WID (SP-24xxxx), the items that RAN3 studied and concluded with the required involvement of SA2 in TR are included as part of WT#1. Accessing to local services via NR Femto is one of the items. This CR proposes to adding related enhancements to enable accessing to local services via CAG cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding enhancements to enable accessing to local services via CAG cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature 5G_Femto accessing to local services via CAG cell is not enabled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.50.1, 5.50.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5119 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.50.1	Overview

This clause provides an overview of 5GS functionalities and architecture to support 5G Femto deployments.

Editor's note:	The architecture and impacts will depend on the RAN WG3 output.

The existing CAG concept defined in the clause 5.30.3 for PNI-NPN is used for 5G Femto access control. The Femto cell may broadcast one or multiple CAG Identifiers as specified in the clause 5.30.3 if access control is needed. The enhancements enable (i) the CAG owner or an authorized administrator to control/provision subscribers to allow accessing their 5G Femto cells; and (ii) accessing to local services via 5G Femto cells.

SECOND CHANGE

### 5.50.X	Accessing to local services

In order to support access to local services, a local UPF may be either stand-alone or co-located with the NR Femto node.

The local UPF uses the procedures defined for N4 Association Setup in clause 4.3.3.1 or N4 Association Update in clause 4.3.3.2 of TS 23.502 [3] to indicate the UPF IP address and associated CAG information to be used as part of UPF discovery at SMF.

End of CHANGES