3GPP TSG-SA2 Meeting #165	S2-2410867

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1216 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing MFAF Services and Service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-5-5 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 in the LS C3-243483 asks that Nmfaf_3daDataManagement_TransferInitiation service operation and the Nmfaf_ContextManagement service be clarified. In TS23.288 clause 6.2.6.3.8 a procedure that includes MFAF relocation uses Nmfaf_3daDataManagement_TransferInitiation request/response service operation and an Nmfaf_ContexManagement service. However, these services / service operations are not defined in clause 9 In TS23.288 clause 6.2.6.3.8 a procedure that includes MFAF relocation uses Nmfaf_3daDataManagement_TransferInitiation request/response service operation and an Nmfaf_ContexManagement service. However, these services / service operations are not defined in clause 9 From TS 23.288 Figure 6.2.6.3.8-1: Procedure for DCCF relocation initiated by DCCF …. Conditional on MFAF transfer: 4. Source DCCF uses the Nmfaf_3daDataManagement service to request transfer of UE MFAF data subscription context to the target MFAF. 5. Target MFAF retrieves MFAF subscription context from Source MFAF. 6. Target MFAF accepts the data subscription(s) context transfer. 7. Target MFAF responds to the DCCF indicating the transfer is complete. The response may contain a Transaction Reference ID from the Target MFAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend Nmfaf_3daDataManagement_Configure request/response service operation to support MFAF context transfers, remove Nmfaf3daDataManagement_TransferInititation service operation from the procedure in 6.2.6.3.8 and add an Nmfaf_ContexManagement service in clause 9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete and self-contradicting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.8, 9.1, 9.2.2, new 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 6.2.6.3.8	DCCF and MFAF relocation initiated by DCCF

The procedure depicted in Figure 6.2.6.3.8-1 is used to support DCCF and MFAF reselection when the source DCCF or MFAF can no longer serve the UE.

Figure 6.2.6.3.8-1: Procedure for DCCF relocation initiated by DCCF

0.	The data consumer subscribes to source DCCF. The data consumer may indicate in the subscription request with an indicator that the DCCF may execute the relocation procedure.

NOTE 1:	If the source DCCF or target DCCF does not support relocation, the consumer may execute DCCF relocation based on internal logic.

1.	Source DCCF subscribes UE mobility events from AMF. The UE ID is provided by the data consumer in step 0.

2.	If UE moves out of the service area of the source DCCF, source DCCF determines UE DCCF subscription context to be transferred to target DCCF, e.g., triggered by a UE mobility event notification from AMF. If UE moves out of the service area of the source MFAF, source DCCF determines UE MFAF data subscription to be transferred to target MFAF.

3.	Source DCCF may query the NRF to discover and select the target DCCF and/or MFAF, e.g. based on the UE location information received from AMF.

Conditional on MFAF transfer:

4.	Source DCCF uses the Nmfaf_3daDataManagement service to request transfer of UE MFAF data subscription context to the target MFAF.

5.	Target MFAF retrieves MFAF subscription context from Source MFAF.

6.	Target MFAF accepts the data subscription context transfer. The source MFAF stops data collection and can remove the related context.

7.	Target MFAF responds to the DCCF indicating the transfer is complete. The response may contain a Transaction Reference ID from the Target MFAF. The DCCF regards the data collection context at the Source NWDAF as terminated.

8.	If a DCCF relocation does not occur, Source DCCF updates the data subscription to the data source by changing the Notification Target Address and Notification Correlation ID to the MFAF Notification Address and the MFAF Notification Correlation ID that received in Step 7.

Conditional on DCCF transfer:

9.	Source DCCF using Ndccf_DataManagement_Transfer Request service operation to transfer of UE data subscription context to the target DCCF.

10.	Target DCCF accepts the data subscription(s) context transfer.

11.	If an MFAF is being used, the Target DCCF uses the Nmfaf_3daDataManagement_Configure service to configure the target MFAF.

12.	Target DCCF confirms UE data subscription context transfer to the source DCCF. The confirmation includes the Subscription Correlation ID used by the Target DCCF.

For DCCF or MFAF transfer:

13.	[Optional] Target DCCF subscribes to the relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the data subscription context and repeat step 1 to subscribes for UE mobility events from AMF.

14.	Source DCCF informs the data consumer about the successful UE DCCF or MFAF data subscription context transfer using a Ndccf_DataManagement_Notify message. The notification may contain a Subscription Correlation ID provided by the target DCCF.

15.	[Optional] Source DCCF unsubscribes with the data source(s) that are no longer needed for the remaining UE data subscriptions.

For DCCF or MFAF transfer:

NOTE 3:	At this point, UE data subscription transfer is deemed completed.

16-18.	Target DCCF or MFAF collects data from the data source(s) and notifies the data consumer using a Ndccf_DataManagement_Notify message.

2nd change

## 9.1	General

Table 9.1-1 shows the MFAF services and MFAF service operations.

Table 9.1-1: NF services provided by MFAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmfaf_3daDataManagement | Configure | Request / Response | DCCF, NWDAF |
|  | Deconfigure | Request / Response | DCCF, NWDAF |
| Nmfaf_3caDataManagement | Notify | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
|  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
| Nmfaf_ContextManagement | Transfer | Request/Response | MFAF |

3rd change

### 9.2.2	Nmfaf_3daDataManagement_Configure service operation

Service operation name: Nmfaf_3daDataManagement_Configure

Description: The consumer configures or reconfigures the MFAF to map data or analytics received by the MFAF to out-bound notification endpoints and to format and process the out-bound data or analytics. The consumer may also use this service operation with a target MFAF to request that it transferMFAF UE context(s) from the indicated MFAF.

Inputs, Required: None.

Inputs, Optional: Data Consumer or Analytics Consumer Information, Formatting Instructions, Processing Instructions, MFAF Notification Information, Transaction Reference Id, ADRF ID, MFAF Transfer Information.

"Data Consumer or Analytics Consumer Information" contains for each notification endpoint, the consumer provided Notification Target Address (+ Analytics Consumer Notification Correlation ID) or other endpoint addresses if provisioned on the DCCF to be used by the MFAF when sending notifications. The consumer includes this information except when initiating a UE context transfer between a source MFAF and a target MFAF

"MFAF Notification Information" is used to identify Event Notifications received from a Data Source and comprises the MFAF Notification Target Address (+ MFAF Notification Correlation ID). If a Data Source is already supplying the data to the MFAF, the MFAF Notification Information previously provided by the MFAF and used by the DCCF to obtain data from a Data Source is provided as an Input. If a new subscription to a Data Source is needed, the MFAF Notification Information is not specified as an Input and the MFAF provides Notification Information as an output. The MFAF Notification Information may subsequently be used by the DCCF when subscribing to a Data Source. "ADRF ID" is used to identify the ADRF when DCCF requests that the Messaging Framework to store historical data and analytics in the ADRF via Nadrf_DataManagement_StorageRequest. When a Notification Correlation ID is provided, MFAF shall use the Nmfaf_3caDataManagement_Notify service with the notification correlation ID to send the data or analytics to the ADRF.

“MFAF Transfer Information” consists of a Transfer Initiation flag together with ID of the MFAF currently holding the UE context, UE MFAF Context Identifier(s) (i.e. the Transaction Reference ID(s) that was returned to the consumer when using the Nmfaf_3daDataManagement_Configure service operation to configure or update the MFAF).

Outputs Required: Operation execution result indication.

Outputs, Optional: MFAF Notification Information, Transaction Reference Id.

4th change

## 9.4	Nmfaf_ContextManagement service

### 9.4.1	General

Service Description: This service is used to transfer MFAF UE context information to a consumer (e.g. a target MFAF). It may be triggered by a Nmfaf_3daDataManagement_Configure request.

### 9.4.2	Nmfaf_ContextManagement_Transfer service operation

Service operation name: Nmfaf_ContextManagement_Transfer

Description: Provides MFAF UE context to a consumer.

Inputs, Required: (Set of) UE MFAF Context Identifier(s) (i.e. the Transaction Reference ID that was returned to the consumer when using the Nmfaf_3daDataManagement_Configure service operation to configure or update the MFAF).

NOTE:	These identifiers can have been received from the DCCF via the Nmfaf_3daDataManagement_Configure request (see clause 6.2.6.3.8).

Inputs, Optional: None

Outputs, Required: Operation execution result indication. For success Set of MFAF Context(s) (i.e. for each Transaction Reference ID provided as input Data Consumer Information, optionally Formatting Instructions and Processing Instructions, and possible buffered partial contents of pending notifications).

Outputs, Optional: none

End of changes