SA WG2 Meeting #165	S2-2410881

# Hyderabad, India, 14 – 18 October 2024	(was S2-2410590)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.167 | CR | 0377 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of a Test eCall over IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson, Nokia, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN, TEI19 |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CEN has requested alignment of eCall over IMS with CEN requirements for 5 cases where 3GPP and CEN requirements for eCall over IMS differ as described in their LS to SA#104 in SP-240522. SA has agreed alignment for case 1 in LS SP-240522 according to their LS reply in S2-2409582/SP-241421. Case 1 concerns support of a Test eCall over IMS. CT has approved a CR to TS 24.229 in CP-242256 to provide the stage 3 support for this and SA has asked SA2 to look into alignment of stage 2 support for this. The stage 3 change in CP-242256 supports a Test eCall over IMS establishment with transfer of initial and updated MSD but omits support of a Test eCall callback from a test point with transfer of updated MSD which is part of the CEN requirement in CEN EN 17184:2024 clause 7.3.2. The stage 3 change in CP-242256 also conditions sending of the initial MSD in an INVITE on support of eCall over IMS by the serving PLMN. However, when an HPLMN supports a test eCall over IMS and uses IMS home routing for a roaming UE that is eCall capable, there is no dependence on support of a test eCall over IMS at the IMS level in a VPLMN and hence there is no need to condition inclusion of initial MSD on eCall over IMS support by a VPLMN. Therefore it seems appropriate to provide stage 2 support for both of these differences to CT1. CT1 can later update their stage 3 to support these changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for establishment of a test eCall over IMS from a UE to a test point with transfer of initial MSD and transfer of updated MSD. Add support for a callback from a test point to a UE with transfer of updated MSD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for a test eCall over IMS will be missing at a stage 2 level which may lead to errors and/or differences in different implementations which may prevent or impair a test eCall over IMS in some cases. . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1, 6.2.1,7.7.2, 7.7.3, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 24.229 CR 6669 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[2]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; General Packet Radio Service (GPRS); Service description; Stage 2".

[3]	CCITT Recommendation Q.65: "Methodology – Stage 2 of the method for the characterisation of services supported by an ISDN".

[4]	ITU Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[5]	3GPP TS 23.271: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Functional stage 2 description of LCS".

[6]	3GPP TS 25.301: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Interface Protocol Architecture".

[7]	Void.

[8]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[9]	IETF RFC 3825: "Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information".

[10]	IETF RFC 4676: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information ".

[11]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[12]	3GPP TS 23.002: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Network architecture".

[13]	3GPP TS 24.008: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[14]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[15]	OMA AD SUPL: "Secure User Plane Location Architecture", http://www.openmobilealliance.org.

[16]	OMA TS ULP: "User Plane Location Protocol", http://www.openmobilealliance.org.

[17]	NENA I2 architecture: "Interim VoIP Architecture for Enhanced 9-1-1 Services (i2)".

[18]	ETSI ES 282 004: "Protocols for Advanced Networking (TISPAN); NGN Functional Architecture; Network Attachment Sub-System (NASS)".

[19]	3GPP TS 24.229: "IP multimedia call control protocol based on SIP and SDP; stage 3".

[20]	3GPP TS 23.203: "Policy and Charging Control architecture".

[21]	3GPP TS 23.003: "Numbering, addressing and identification".

[22]	Void.

[23]	ANSI/J-STD-036-B: "Enhanced Wireless 9-1-1, Phase 2".

[24]	3GPP2 X.S0002-0: "MAP Location Services Enhancements".

[25]	3GPP2 X.S0060: "HRPD Support for Emergency Service".

[26]	3GPP TS 22.003: "Circuit Teleservices supported by a Public Land Mobile Network (PLMN)".

[27]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) multimedia core network subsystem; Stage 1".

[28]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[29]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SR VCC); Stage 2".

[32]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[33]	3GPP TS 24.301: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[34]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[35]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[36]	ETSI TS 181 019 V2.0.0: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Business Communication Requirements".

[37]	ETSI TS 182 024 v.2.1.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Hosted Enterprise Services; Architecture, functional description and signalling".

[38]	ETSI TS 182 025 v.2.1.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Business trunking; Architecture and functional description".

[39]	3GPP2 X.S0057-C v1.0: "E-UTRAN - eHRPD Connectivity and Interworking: Core Network Aspects".

[40]	NENA 08-002, Version 1.0 (i3): "NENA Functional and Interface Standards for Next Generation 9-1-1".

[41]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[42]	3GPP TS 26.267: "Digital cellular telecommunication systems (Phase 2+); Universal Mobile Telecommunication System (UMTS); eCall data transfer; In-band modem solution General description".

[43]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

[44]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[45]	ATIS-0700028: "Location Accuracy Improvements for Emergency Calls".

[46]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services".

[47]	IEEE Std 802.11-2012: "IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[48]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[49]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[50]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[51]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[52]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[53]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[54]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state".

[55]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[56]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[57]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

[58]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[xx]	CEN EN 17184:2024: "Intelligent transport systems — eSafety — eCall High level application Protocols (HLAP) using IP Multimedia Subsystem (IMS) over packet switched network".

[yy]	3GPP TS 23.221: "Architectural requirements".

# * * * * Next change * * * *

## 6.1	UE

-	Should be able to detect an emergency session establishment request.

-	In the case of NG-eCall, the IMS emergency session establishment request may be invoked either automatically without user input or manually via user input.

-	Initiate an IMS emergency registration request.

-	The UE may perform an IMS emergency session establishment without prior emergency registration when already IMS registered and it is in home network (e.g. including IP-CANs where roaming outside the home network is not supported).

-	Otherwise, the UE shall perform an IMS emergency registration.

-	Include an emergency service indication in the emergency session request.

-	UE may support the GIBA procedure defined in TS 24.229 [19] as part of the emergency IMS registration procedure.

-	In the case of emergency IMS registration failure the UE shall be able to interpret the indication, if provided by the serving IMS, whether anonymous IMS emergency sessions are supported in the serving IMS. If the serving IMS has indicated support, the UE shall proceed with an anonymous IMS emergency session, otherwise it proceeds according to clause H.5.

NOTE 1:	UEs compliant with pre-Rel-14 versions of this specification are unable to interpret this indication and ignore the indication. Such UEs might attempt an anonymous IMS emergency session or proceed according to clause H.5.

-	Include an equipment identifier in the request to establish an emergency session for "anonymous user".

NOTE 2:	"Anonymous user" in this context is the person who does not have sufficient credential for IMS registration. No Stage 3 work is expected as the anonymous user detection already existed today.

-	Include an equipment identifier in the request to establish an emergency session when the UE supports SRVCC as specified in TS 23.237 [32].

-	Include identity information for the IP-CAN if available (e.g. MCC-MNC or an equivalent)

NOTE 3:	UE provided IP-CAN identity information will not be completely reliable.

-	Attempt the emergency call in CS domain, if capable.

-	Handle a 380 (Alternative Service) response with the type set to "emergency" e.g. as a result of non UE detectable emergency attempt.

-	Handle a response with an indication, IMS emergency registration required as a result of emergency session establishment attempt.

-	Other general requirements of UE shall be referred to the general requirements of emergency calls in TS 22.101 [8].

-	For NG-eCall, where transfer of the MSD is not acknowledged by the PSAP, the UE shall fall back to the in-band transfer of the MSD, as in CS domain defined in TS 26.267 [42].

The UE initiates the emergency session establishment request, and for the purpose of processing the request properly in the network, the following specific information is supplied in the request message.

-	Emergency service indication.

-	A registered Public User Identifier. If the UE performed an emergency registration using a temporary Public User Identifier then the UE should not use the temporary Public User Identifier to initiate the emergency session. The selected Public User Identifier shall be part of an implicit registration set that includes a TEL-URI.

NOTE 4:	The UE can be preconfigured with information to select the appropriate Public User Identifier if more than one Public User Identifier is provisioned in the UE.

-	Optionally, type of emergency service. It could be implied in the above emergency service indication.

-	For an NG-eCall an eCall indication including whether eCall is automatic or manual.

-	A UE that supports test eCall, should be able to detect an IMS call request of the test eCall type, and shall reject the request if the UE is in limited service state. A UE not in limited service state performs domain selection same as for a normal IMS call as defined in TS 23.221 [yy] and TS 23.501 [48]. If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration. The requirements for a UE supporting test eCall are further specified in clause 5.1.6A of TS 24.229 [19]

-	UE's location information, if available.

-	The TEL-URI associated to the Public User Identifier, if available.

-	GRUU, if available.

In the case of a non UE detectable emergency call, upon reception of indication from the network, the UE shall handle the call as described in clause 7.1.2.

NOTE 5:	If the indication was received in a rejection message the UE performs appropriate emergency error handling procedures.

# * * * * Next change * * * *

### 6.2.1	Proxy-CSCF

-	Handle registration requests with an emergency registration indication like any other registration request, except that it may reject an emergency registration request if the IM CN subsystem that the P-CSCF belongs to can not support emergency sessions for the UE (e.g., due to operator policy or UE is not within IM CN subsystem's geographical area or IP-CAN not supported).

-	Detect an emergency session establishment request.

-	Reject/allow unmarked emergency requests.

-	Reject/allow anonymous emergency requests.

-	Prevent non-emergency requests that are associated with an emergency registration.

-	May query IP-CAN for location identifier.

-	May query IP-CAN for additional subscriber related identifier(s).

-	Select an Emergency CSCF in the same network to handle the emergency session request. The selection method is not standardized in the present document.

-	Alternatively, for non-roaming subscribers and when the request is received over a non-emergency registration, the P-CSCF may forward an emergency session to an S-CSCF if so instructed by operator policy or local regulation.

NOTE:	This can be for example the case if the P-CSCF recognizes that an emergency session was not received via a security association for a UE previously authenticated with digest type proxy authentication.

-	Do not apply emergency session detection if requested using private network traffic and forward the session to the S-CSCF, except if operator policy requires the P-CSCF to detect emergency session requests and treat detected emergency session requests as if they are part of public network traffic.

-	For UEs without credentials, forward the equipment identifier to the E-CSCF that was received from the UE.

-	For UEs without credentials and subjected to local regulation, forward the additional subscriber related identifier(s) received from IP-CAN to the E-CSCF.

-	Prioritize the emergency session.

-	Check the validity of the caller TEL-URI if provided by the UE and shall provide the TEL-URI in the session establishment request if it is aware about the TEL-URI associated with the Public User Identifier used for an emergency registration.

-	May respond to a UE with an emergency service indication as a result of detecting a non UE detectable emergency session establishment request

-	May respond to the UE with an indication, IMS emergency registration required as a result of processing the emergency session establishment attempt.

-	Should be able to identify the service data flow associated with emergency service and inform PCRF accordingly.

-	Upon IMS registration failure the P-CSCF may indicate to the UE whether anonymous IMS emergency sessions are supported.

-	P-CSCF may, based on operator policy, insert attestation information related to the asserted calling identity associated with an emergency session, if operating in a network that supports calling number attestation and signing.

-	P-CSCF may assert Resource-Priority information for an emergency session if configured through operator policies.

-	For a test eCall as defined in Annex X, detect a session establishment request for a test eCall. The detection of test eCall can be based on e.g. Request-URI set to a configured value of non-emergency service URN (e.g. urn:service:test.sos.ecall).

# * * * * Next change * * * *

### 7.7.2	MSD Transfer/Acknowledgement during Session Establishment with a PSAP Not supporting NG-eCall (inband means)

Figure 7.7.2-1 illustrates a high level call flow for an interworking scenario between a UE and a PSAP via the CS domain.

This scenario may occur in the case, when PS access is available, but the UE does not detect that the NG-eCall is supported by the IP-CAN and there is no CS access available. In this case, the UE shall establish a regular IMS emergency call.

A similar scenario may occur in the case, when the UE detects that the IP_C AN supports NG-eCall and initiates an NG eCall but the session is handled by a PSAP not supporting NG-eCall. The only exception would be step 2 and where the SIP INVITE shall contain the initial MSD and the eCall type of emergency service indicator (automatic, manual).

Figure 7.7.2-1: eCall Scenario with PSAP not supporting NG-eCall

1.	The UE performs steps 1 to 6 of the procedure shown in Figure 7.1.

2.	The UE sends an initial emergency SIP INVITE to the IMS core. The SIP INVITE shall be setup as follows:

-	if the UE has not received the "eCall supported" indication, the UE shall include the eCall type of emergency service indication (automatic, manual) and shall not include the initial MSD if the PS access is available;

-	if the UE has received the "eCall supported" indication, the UE shall include the initial MSD and the eCall type of emergency service indicator (automatic, manual).

3.	The IMS core routes the emergency SIP INVITE towards the appropriate PSAP. In this call flow, the appropriate PSAP is accessed over the MGCF and the CS domain after translating the eCall type of emergency service indication into the corresponding MSISDN.

3a.	The SIP INVITE is sent to an MGCF/MGW for interfacing to the CS domain.

3b.	The MGCF sends an IAM towards the appropriate PSAP for the emergency voice call.

4.	The emergency voice call establishment is completed with a voice path only.

5.	If the UE has sent the MSD in step 2 and did not receive acknowledgement for the initial MSD included in the SIP INVITE, or the UE did not send the MSD in step 2 the UE shall attempt to transfer the MSD to the PSAP via the eCall Inband Modem, as defined in TS 26.267 [42].

NOTE X:	Receipt of the MSD using audio media stream encoded as described in 3GPP TS 26.267 [42] is optional for a PSAP that supports eCall over IMS according to CEN EN 17184:2024 [xx].

# * * * * Next change * * * *

### 7.7.3	Transfer of an updated MSD

Figure 7.7.3-1 illustrates a high level call flow for transfer of an updated MSD in the case that the emergency centre or PSAP is accessed via the PS domain. The call flow is applicable following establishment of an emergency services session for eCall as described in clause 7.1.1 in which the initial MSD is transferred in the INVITE and prior to release of the emergency services session. The call flow is also applicable during an incoming IMS call for PSAP call back.

Figure 7.7.3-1: Sending an updated MSD for eCall

1.	If the emergency centre or PSAP requires an MSD update, it may send a request for MSD to the IMS Core.

2.	The IMS Core shall forward the request to the UE.

3.	The UE shall send the most recent MSD to the IMS Core.

4.	The IMS Core shall forward the MSD to the emergency centre or PSAP.

NOTE:	Whether an acknowledgement message for the successful receipt of updated MSD by the PSAP is further provided to the UE is resolved in stage 3 level.

# * * * * Next change * * * *

Annex X (normative):
Procedures for a Test eCall over IMS

# X.1	General

The purpose of a test eCall over IMS is to provide a high level of assurance that an eCall capable UE and particularly a UE configured for eCall Only Mode (which may be used very rarely) can reliably support a real eCall over IMS when an actual emergency occurs.

Test eCalls could be used during

a) the initial deployment to see whether devices are working properly, e.g. product acceptance tests;

b) product conformity testing and car type-approval in certification labs;

c) periodic technical inspection over the lifetime of a vehicle (e.g. once every 1-2 years)

A test eCall over IMS includes all features of a real eCall over IMS except for features that are specific to an emergency services call. A test eCall over IMS thus excludes for example the use of an emergency indication at the RRC level, an emergency PDU session or emergency PDN connection, IMS emergency registration and an emergency service indication or eCall type of emergency service indication in a SIP INVITE. However, it includes other aspects of an eCall over IMS as described in clause 7.7, including MSD transfer, MSD update and call back with MSD update.

NOTE:	A test eCall over IMS is a normal IMS session and not an emergency IMS service and thus is established either using the home IMS alone when not roaming or when roaming with IMS home routing or using both the home and visited IMS when roaming with IMS local breakout. Support for a test eCall over IMS is thus always needed from the home IMS. This support is assumed to be enabled through a IMS roaming agreement when the UE is roaming. However, support by a visited IMS is only needed when roaming with IMS local breakout. Because visited IMS support may not always be provided (e.g. in a VPLMN that does not support a real eCall over IMS),  HPLMN operators can choose to enforce use of IMS home routing via a roaming agreement that use the architecture in TS 23.228 [1], clause 4.15b (Roaming Architecture for Voice over IMS with home routed traffic) for any UE that is eCall capable and would require support for a test eCall over IMS.

# X.2	Test eCall over IMS Session Establishment to a Test Point

Figure X.2-1 illustrates a high level call flow for a Test eCall over IMS session establishment to a test point which may either be NG capable and accessible in the PS domain or non NG capable and accessible only in the CS domain. The UE is assumed to be either already attached to or registered on the PS domain if the UE is not in eCall only mode or camped on the PS domain if the UE is in eCall only mode.

NOTE:	A non NG capable test point that is accessible only in the CS domain may be a legacy test point used to test eCall in the CS domain. It is beneficial to make use of such test points for scenarios where an NG capable test point is not available.

Figure X.2-1: Test eCall over IMS Session Establishment to a Test Point

1.	The user requests a test eCall. If the UE is not in limited service state and not already attached to or registered on the PS domain, the UE attaches to or registers on the PS domain. The UE performs a normal IMS registration if not already IMS registered. If the UE is in limited service state, the user request is denied and remaining steps are skipped.

2.	The UE sends a SIP INVITE to the IMS core. The Request-URI in the SIP INVITE includes a non-emergency service identity that is defined by a national regulator, PSAP or eCall test provider for testing purposes. The UE also includes initial MSD and an indication of support for transfer of updated MSD. The initial MSD indicates a test eCall.

3a.	If the public user identity for the test point indicates that the test point is accessible in the PS domain, the IMS core routes the SIP INVITE towards the test point.

3b.	If the public user identity for the test point indicates that the test point is accessible in the  CS domain, the SIP INVITE is sent to an MGCF/MGW for interfacing to the CS domain.

3c.	If step 3b occurs, the MGCF sends an IAM towards the test point.

4a. If step 3a occurs, the test point verifies the correctness of the initial MSD and returns a SIP 200 OK to the IMS core. The SIP 200 OK explicitly includes a positive or negative acknowledgement for the initial MSD and an indication of support for transfer of updated MSD.

4b.	If step 3c occurs, the test point returns an ANM to the IMS core.

4c.	If step 4b occurs, the MGCF returns a SIP 200 OK to the IMS core.

5.	The IMS core returns the 200 OK received in step 4a or step 4c to the UE.

6.	The test call establishment is completed with a voice path only.

NOTE:	If the test call establishment fails at any of steps 1 to 6, the UE may fallback to a different PS RAT or to the CS domain if available, the same as for any other normal call.

Steps 7 and 8 apply only when the test point is accessible in the CS domain:

7.	If the UE did not receive a positive or negative acknowledgement for the initial MSD in the 200 OK at step 5, the UE shall attempt to transfer the MSD to the test point via the eCall Inband Modem, as defined in TS 26.267 [42].

8.	The test point may optionally request an updated MSD from the UE. If step 4a occurred, the test point and UE use the procedure defined in clause 7.7.3 to request and transfer the updated MSD. If step 4c occurred, the test point and UE use the eCall Inband Modem procedure, as defined in TS 26.267 [42].

# X.3	Callback from a Test Point with transfer of updated MSD

Figure X.3-1 illustrates a high level call flow for callback to a UE from test point with transfer of updated MSD.

This scenario may occur immediately after a test eCall over IMS has been established to a test point as defined in Annex X.2 and has been released.  Successful completion of the procedure in Figure X.3-1 would provide some assurance that a UE would support a PSAP callback with transfer of updated MSD for a real eCall over IMS.

Figure X.3-1: Callback from a Test Point with transfer of updated MSD

1.	The UE performs steps 1 to 8 of the procedure shown in Figure X.2-1 followed by a later call release. The UE remains in the PS domain with a normal IMS registration.

2a.	If steps 3a and 4a had occurred in Figure X.2-1 as part of step 1, the test point sends a SIP INVITE to the IMS Core and includes a public user identity of the UE obtained in step 1. The INVITE also includes an indication that the test point supports transfer of an updated MSD.

2b.	If steps 3c and 4b had occurred in Figure X.2-1 as part of step 1, the test point sends or instigates an IAM to an MGCF/MGW in the HPLMN for interfacing to the CS domain. The IAM contains a public telephone identity of the UE as obtained by the test point at step 1.

2c.	If step 2b occurs, the MGCF sends a SIP INVITE to the IMS core in the HPLMN.

4.	The IMS core forwards the SIP INVITE from step 2a or step 2c to the UE, after the UE has been paged if IDLE.

5.	Voice call establishment is completed with a voice path only.

6a.	If step 2a occurred, the test point may instigate the procedure in Figure 7.7.3-1 to request and obtain an updated MSD. The UE shall reject the request for MSD if not received within a configurable time period of release of the test eCall in step 1.

6b.	If step 2b occurred, the test point may request and obtain an updated MSD via the eCall Inband Modem, as defined in TS 26.267 [42]. The UE shall reject the request for MSD if not received within a configurable time period of release of the test eCall in step 1 or if the SIP INVITE in step 4 did not identify the calling party using the same non-emergency service identity that was used by the UE to establish the test eCall over IMS at step 2 in Figure X.2-1.

NOTE:	The configurable time period in steps 6a and 6b is a short period to enable call back to occur as part of the same test eCall from a user perspective and to reduce a risk of transferring updated MSD to an unauthorized calling party.

# * * * * End of changes * * * *