3GPP TSG-WG SA2 Meeting #165	S2-2411058

14 - 18 October, 2024, Hyderabad, India	(revision of S2-2410807)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5004 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on subscription to UPF event exposure service via SMF during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During UPF relocation procedure, whether the UPF event consumers should subscribe the new UPF data to new UPF or whether the old UPF sends the data that has been collected but not sent to the consumer yet is based on the relocation reporting indication from consumer is FFS. Whether new subscription or not: If the UPF event consumer indicates the certain UPF id in subscription request or trigger direct subscription to certain UPF that UPF id indicates, after UPF relocation, the UPF event consumer may not trigger new subscription request to new UPF. If the UPF event consumer indicates the certain DNAI in subscription request, and after UPF relocation the DNAI is changes, for example relocates from one DNAI to another DNAI, the UPF event consumer doesn’t not trigger new subscription request to new UPF in new DNAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the SMF to subscribe the target UPF as optional, and add clarification about why not subscribe any more. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that the SMF always subscribes the UPF event. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

4.15.4.5.6	Information flow for subscription to UPF event exposure service via SMF during UPF relocation

Figure 4.15.4.5.6-1: Subscription to UPF event exposure service via SMF during UPF relocation

This information flow is used in the scenario of UPF relocation, e.g. the source UPF is I-UPF which is relocated in SSC mode 1, or the source UPF is PSA UPF which is relocated in SSC mode 3, or the source UPF is local PSA UPF which is relocated due to UE mobility.

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a relocation reporting indication, which indicates how to handle the data collected by the source UPF when UPF relocation occurs, e.g. discard the collected data or send the collected data to the consumer.

2.	The SMF selects the source UPF and subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation.

3.	The source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the UPF needs to be relocated.

5.	The SMF unsubscribes to the source UPF by invoking Nupf_EventExposure_Unsubscribe service operation. The SMF may include the relocation reporting indication from the UPF event consumer.

6.	The source UPF sends the collected data to the UPF event consumer if the relocation reporting indication from SMF indicates that the source UPF should send the collected data to the UPF event consumer. For example, for periodic reporting type of the UPF event exposure service when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. Or for event triggered reporting with Reporting suggestion information, when the UPF changes, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet based on the relocation reporting indication.

Editor's note:	When the UPF changes, whether the UPF sends the data that has been collected but not sent to the consumer yet is based on the relocation reporting indication from consumer is FFS.

7-8.	To continue the subscription of the UPF event consumer, if the UPF related information is matched with the subscription request in step 1, the SMF may select a target UPF and may re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. The target UPF will send the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

* * * End of Change * * *