3GPP TSG-SA2 Meeting #165	S2-2411152

# Hyderabad, India, October 14 – 18, 2024          		       	       			(revision of S2-2410835)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0572 | rev | 2 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | One Transmission Path used for One Positioning Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | User plane positioning is introduced in Rel-18 5G_eLCS_Ph3 to transfer the LPP messages and supplementary service messages via user plane. For the messages (LPP messages, supplementary services messages) belonging to one positioning procedure, there is no common understanding on how to transfer the messages via user plane and control plane. Some companies believe that for the messages belonging to one LCS session, either control plane or user plane should be used by UE and LMF at the same time. While some companies think it is flexible for LMF to choose control plane or user plane to transfer the messages above, e.g. some messages above are transferred via user plane and the others could be transferred via control plane. If some messages belonging to one LCS session could be transferred via user plane and others could be transferred via control plane, the LMF and UE need to be enhanced to select user plane or control plane before sending a message. So this makes LMF and UE more complicated and increases positioning delay. To reduce complicity and positioning delay, it is proposed that one transmission path (i.e. control plane or user plane) is used for one positioning procedure. The UE and the LMF is enhanced to support the principle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For a positioning procedure, one transmission path is used at a time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent behavior between LMF and UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.10	Support of Positioning over user plane connection between UE and LMF for non-regulatory service

LMF and UE may utilize a user plane connection to transfer supplementary services messages and LPP messages. User Plane protocol (LCS-UPP) to support supplementary services messages and LPP messages transport between the UE and the LMF is defined in TS 24.572 [48].

If LMF decides to use user plane the LMF should indicate the UE to use user plane for positioning with the information to establish a secure connection, and via this secure connection, position messages can be transferred between UE and the LMF. The URSP defined in TS 23.503 [41] is used by UE to determine how to route the position messages. The operator may provide LMF(s) address information and connection capability for LCS use in traffic descriptors and LCS user plane positioning dedicated PDU session parameters (e.g., DNN and S-NSSAI) in Route Selection Descriptors to the UE as part of the URSP rule. The position messages can be routed to an established PDU Session or can trigger the establishment of a new PDU Session. The LMF and UE may maintain the established user plane connection and the established user plane connection may be reused for subsequent user plane position messages transmission trigged by UE or LMF. If the LMF detects the user plane connection is not used for an implementation specific time, the LMF terminates the user plane connection. The supplementary services messages transferred over user plane only support event report messages, periodic triggered invoke messages and MS cancel deferred location messages.

NOTE:	In this Release, the user plane connection between UE and LMF can not be used for regulatory positioning service.

NOTE X:  For one positioning procedure, the LMF selects either control plane or user plane to transfer the associated supplementary services messages and LPP messages. During the positioning procedure, the LMF and UE should avoid switching between control plane and user plane to transfer the messages above.

* * * End of Change * * *