3GPP TSG-WG SA2 Meeting #165	S2-2411180

# Hyderabad, IN, 14th – 18th Oct. 2024                              	   (Revision of S2- 2410470)

# Source:	AT&T

# Title:	MSISDN verification operation support to Nnef_UEId Service

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: MSISDN verification operation support to Nnef_UEId Service

Acronym: TEI19_MVOSNS

Unique identifier :

Potential target Release: Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don’t know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

The Nnef_UEId Service is currently supporting MSISDN retrieval operation where the MSISDN is retrieved from the UDM and exposed to an authorized AF. There are many use cases in the industry where the operator would need to support MSISDN verification use cases in which the authorized AF is providing the MSISDN in the request and asks the operator to verify whether the UE’s MSISDN matches the AF-provided MSISDN in the request, as an alternative to the MSISDN retrieval exposure.

In order to support such a need, the Nnef_UEId Service is being enhanced to allow NEF to perform verification between the MSISDN it retrieves from the UDM with the one it receives from the AF (in the request) and responds with the verification result accordingly (e.g. true/false depending on whether the two MSISDNs match or not).

# 4	Objective

To address the new NEF exposed capability, it is proposed for the Nnef_UEId Service to offer a new operation called Nnef_UEId_Verification.

This work will require 0.5 TU to discuss and agree the corresponding CR(s).

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | Enhancing the Nnef_UEId Service with a new operation (i.e. Nnef_UEId_Verification) to support MSISDN verification exposure by NEF | SA#106 (Dec 2024) |  |
| TS 23.501 | Updating “External Exposure of Network Capability” clause with the newly introduced NEF feature (i.e. Nnef_UEId_Verification) | SA#106 (Dec 2024) |  |

# 6	Work item Rapporteur(s)

Rohit Abhishek (AT&T) ra503n@att.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None identified.

# 9	Supporting Individual Members

Supporting IM name

AT&T

Ericsson

Nokia

Verizon

Deutsche Telekom

InterDigital

Apple

Qualcomm

Samsung