3GPP TSG-SA2 Meeting #165	 S2-2411256

# 14 – 18 October 2024, Hyderabad, India							(revision of S2-2410979)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0584 | rev | 3 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Data Collection by LMF procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Qualcomm, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR provides updates to procedure for Data Collection from the UE described in clause 5.18.1 and also addresses the following ENs. Editor's note: Whether the AMF can further select and return SUPIs in step 2 and whether the AMF needs to consider the UE capabilities supporting AI/ML based positioning is FFS. Editor's note: Whether and how LMF is informed about the UE capability to report input data for ML Model training is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following clarifications are made: - LMF may be triggered by NWDAF to collect data for training an AIML model for Direct AIML positioning Rev 1 includes changes from S2-2410673 - Clarify the interaction between AMF and LMF in step 2. - Clarify the procedure between LMF and UE in step 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for data collection from the UE is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.1, 6.X (new), 6.x.1 (new), 6.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be treated together with CR 0574 CR0595 and CR 0597 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

### 5.18.1	Void

# * * * * Next change * * * *

## 6.X	Procedures of data collection for LMF-based AI/ML Positioning

### 6.x.1	General

This clause describes the procedure for data collection by the LMF for input data to train LMF-based AI/ML models for Direct AI/ML positioning.

### 6.x.2	Data Collection at LMF to train the LMF-based AI/ML positioning model to perform positioning based on UE measurements.

The LMF needs to obtain input data for LMF-based AI/ML Positioning model training and may also request the UE to provide the UE location. The LMF-based AI/ML Positioning model is trained to perform UE positioning for UEs located in an area of interest that may expand over multiple TA or cover multiple NG-RAN nodes.

Figure 6.X.2-1: Data collection by LMF to train the LMF-based AI/ML Positioning model using UE measurements

1.	The LMF starts data collection for the purpose to train an AI/ML Model for UE Positioning (this may be based on an internal trigger in the LMF or a request from the NWDAF). The LMF subscribes to AMF to retrieve the list of SUPIs located in an area of interest and the UE Positioning Capability for each UE using Namf_EventExposure_Subscriber_Request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest" and “UE Positioning Capability”.

2.	The AMF send Namf_EventExposure_Subscriber_Response ("list of SUPIs in the area of interest"). If “UE Positioning Capability” is also requested in step 1, AMF includes UE Positinong Capabiliy, UE User Plane Positioning Capabilities if available for each SUPI in the response message sent to LMF.

3.	The LMF further determines the UEs from the list of SUPIs that received from AMF in step 2 for data collection to train the AI/ML based positioning model based on UE Positioning Capability, UE User Plane Positioning Capabilities, UE consent check result, the PRU information available in the LMF and operator’s policy.

Editor's note:	Further details on user consent for data collection for model training for LMF-based AI/ML Model positioning will be aligned with SA WG3.

4.	For each UE that provides consent to data collection for model training and can support data collection the LMF initiates a request for input data.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the UE is FFS. The procedure will be coordinated with RAN WGs.

Editor's Note:	The input data used for AI/ML based positioning will be decided by RAN WGs.

Editor's Note:	Conditions for the LMF to trigger data collection is FFS, e.g., whether and how the LMF can consider load to determine when to perform step 4 is FFS.

Editor’s Note:	Conditions for the UE to report measurement data are FFS

Editor’s Note:	Data collection for performance monitoring is FFS

At the time the LMF decides that the input data for a UE are not needed, e.g. the user consent for data collection for model training is revoked, the LMF stops to collect the input data from the UE to train the LMF-based AI/ML Positioning model.

************************************ END OF CHANGE *****************************