3GPP TSG-WG SA2 Meeting #165	S2-2411268

14th – 18th October, 2024, Hyderabad, India	(revision of S2-2411068, S2-2411045, S2-2410747 and S2-2410056)

Source:	Lenovo, NEC, HONOR, Futurewei, OPPO, ZTE, CATT, Samsung, vivo, Xiaomi, China Mobile

Title:	Conclusions on the common supported functionality of AIoTF

Document for:	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes conclusion principles for the supported functionality of AIoTF.

1	Discussion

After SA2#164 meeting, initial conclusion principle on architecture support for topology 1 was agreed:

A new core network function is introduced to support Ambient IoT.

Based on the introduction of AIoTF, this paper continues to further discuss the supported functionality of it. An assumption is made that the new network function, i.e., AIoTF, will also be introduced to support Ambient IoT related service in topology 2 when UE is acting as the reader.

Based on this, some common principles on the supported functionality of AIoTF, for both the topology 1 and topology 2 are presented in this paper.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

# * * * * First change * * * *

8.1	Interim Conclusion on Key Issue #1

8.1.1	General

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on "Ambient IoT Device subscription management" and key issue#3 aspect on "Ambient IoT service exposure" is considered in this section.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

Editor’s Note: Final conclusions are assumed to be taken in coordination with RAN WGs.

Editor’s Note: Information needed for radio resource allocation to readers is FFS and requires cooperation with RAN2/RAN3.

Editor’s Note: Which architecture options described in the following clauses will be concluded is FFS.

1.	A new core network function is introduced to support Ambient IoT (e.g., AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

Editor's note:	Whether the new core network function also applies to topology 2 is FFS.

a.  The AIoTF manages the device related information

Editor’s Note: Whether and what AIoT Device related information (e.g. AIoT Device last known Reader ID, optionally the result of AIoT device validation result, etc.) is stored in the AIOTF, are FFS.

Editor’s Note: Whether the UDM should store the device related information is FFS.

b.	The AIoTF registers itself in the NRF with its NF profile.

Editor’s Note: The details of the NF profile are FFS.

c.	The AIoTF receives an AIoT service request from the AF and triggers the BS/UE readers to perform AIoT service operations towards the AIoT Devices (s).

d.	The AIoTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF.

Editor’s Note: For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

Editor’s Note: How the aggregation can be done is FFS.

2.	A Permanent AIoT Device IDis stored in the AIoT Device and the UDM or a Credential Holder’s AAA server.

Editor’s Note: Whether and how the validation of the Ambient IoT Device ID is done will be concluded by SA3.

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.

NOTE: The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

Editor’s Note: Whether an AIoT NAS protocol is supported between the AIoT Device and the AIoTF, and what is the supported functionality, is FFS.

# * * * * End of changes * * * *