3GPP TSG-SA WG2 Meeting #166	S2-241xxxx

# Orlando, USA, 18 - 22 November, 2024									 (revision of S2-2411951)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1520 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call setup of UE-Satellite-UE communication with early media in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Handling of early media for UE-satellite-UE communication has been discussed. S2-2411950 discuss the issue for early media handling, and proposes to sustain using ground-AGW for early media based on operator policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedure for supporting UE-Satellite-UE communication with early media in IMS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early media may be terminated or interrupted when UE-satellite-UE communication is activated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AX.5.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after TS 23.228 CR 1428 and 1492. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change (All following text are new) ---

AX.5.1a	At call setup with early media

During the call setup with both originating and terminating UEs using regenerative NR satellite access, besides selecting IMS-AGW on-board of satellite for regular media, the originating P-CSCF selects the IMS-AGW on ground to transfer early media. See Figure AX.5.1a-1 that describes the procedure.

Figure AX.5.1a-1: IMS session establishment procedure with early media

1-10.	Steps 1-10 of clause AX.5.1 apply.

11.	Step 11 of clause AX.5.1 applies with the difference that the IMS AS allocates early media resources with MRF as well as includes the P-Early-Media header field and a modified SDP answer for early media in the 18X response.

12.	Step 12 of clause AX.5.1 applies with the difference that if the P-Early-Media is received in the SIP 18X response, the P-CSCF A determines to not release the IMS-AGW A' on ground and use it for early media.

13. Step 13 of clause AX.5.1 applies.

14.	Step 14 of clause AX.5.1 apply with the difference that P-CSCF A includes SDP answer related to IMS AGW A' on ground in the SIP 18X response.

15-16.	Steps 15-16 of clause AX.5.1 applies. The early media is sent from the MRF to the UE A via the IMS AGW A' on ground, the UPF A' on ground, and the UPF A on-board of satellite.

17-19.	Steps 17-19 of clause AX.5.1 applies and UE B is ringing.

20.	The UE B answers the IMS session and sends SIP 200 response for the INVITE.

21.	The IMS AS instructs the MRF to stop playing early media.

22.	The IMS AS sends SIP UPDATE request with an SDP offer for regular media towards originating side.

23.	P-CSCF A forwards the SIP UPDATE request with modified SDP offer related to the IMS AGW A on-board of satellite to the UE A. The UE A returns SIP 200 response for the UPDATE with SDP answer.

24.	The IMS AS sends SIP 200 response for the INVITE to the originating side. After receiving the SIP 200 response, the UE A is able to send regular media and is ready to receive regular media. The regular media uses UE-satellite-UE communication path.

25.	UE A sends SIP ACK towards terminating side. After receiving the SIP ACK, the UE B is able to send regular media.

26.	The P-CSCF A releases the IMS AGW A' on ground when the IMS session is answered.

## ---End of the Change---