3GPP TSG-WG SA2 Meeting #166	S2-2413028r03

Orlando, USA, 18th Oct – 22nd Oct, 2024	(revision of S2-2412807)

# Title:	[DRAFT] LS on A-IoT Conclusions in SA WG2

# Response to:	S2-2411311(R2-2409412)

# Release:	Release 19

# Work Item:	FS_AmbientIoT, FS_Ambient_IoT_solutions

Source:	[Huawei to be] SA2

To:	SA3, SA5, RAN2, RAN3

Cc:	RAN, SA

Contact Person:

Name:	Runze Zhou

Tel. Number:

E-mail Address:	zhourunze AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TR 23.700-13, v2.0.0 https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=4255

1. Overall Description:

This LS provides information to the following aspects.

- Reply to LS on RAN2 agreements and assumptions for Ambient IoT

SA2 thanks RAN2 for the LS (R2-2409412) on assistance information from the CN to the Reader for A-IoT. After discussion, SA2 would like to provide the answer to the questions in the LS as following:

Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request?

SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2.

Q2: If such information (see Q1) can be provided by CN to the reader, can it be provided only in some cases or in all cases?

SA2 answer: it can be provided in some cases. See answer to Q1.

- Summary of overall progress in the SA2 study item, which may be useful for the future work of other WGs

SA2 has discussed and drawn the interim conclusion on the Ambient IoT including following aspects:

-	General architecture design which is topology 1 and 2 agnostic.

-	Architecture to support topology 1. Which option(s) to be continued needs further work.

-	Architecture to support topology 2. Which option(s) to be continued needs further work.

-	Ambient IoT Device ID device management

-	Ambient IoT Services and their exposure to an AF.

-	Principles of Ambient IoT Services procedures.

For detailed conclusion content, please check the attached TR.

- Following issues need to finalize in coordination with other WGs

- Issues in coordination with SA WG3:

- Architecture aspects to support security e.g. Authentication, ID Validation.

- Whether the temporary ID in the AIoT NAS layer is required for the privacy protection is FFS and is pending SA WG3 decision.

- Whether and how to support enabling temporarily disabled AIoT devices. (In the approved SP-231806/240969, it is stated that enable/disable device operation will be handled by SA3, but it has potential impacts to SA2 procedures)

- What security related information needs to be stored in the device subscription-like information.

- Whether and how to protect A-IoT Device Identification information in the paging message.

- Issues in coordination with RAN WG:

- Radio resources allocation (what information and how to provide it) to Readers for topology 2.

- For RRC based option of topology2, whether the gNB performs the down selection of UE readers provided by AIOTF.

- The layer to enable AIoT Reader control, whether using NGAP or a layer above NGAP.

- Whether and how AIOTF selects BS readers or AIOT RAN nodes

- UE reader authorization for topology 2

- Whether and how AIOTF selects BS readers or AIOT RAN nodes in topology 1

- How addressing works for UL traffic (i.e. how the BS Reader identifies the appropriate AMF to which to forward UL messages) in the indirect path via AMF

- Following issues will be finalized by SA2

- Information that can be stored in the AIoT Device subscription-like data (static information) and that can be stored in AIoT device context data (dynamic information)

- The entity to store the static and dynamic information

- AIoT reader selection principles.

- AIoT data aggregation principles.

- AMF service enhancement to communicate with AIOTF.

- Whether and which additional methods for dynamically provisioning the UE with DNN/S-NSSAI and AIoT NF FQDN can be specified and in which release.

- NEF exposure service design principle.

- NF profile for AIOTF.

- The length of AIoT device ID is fixed or dynamic and it part2 information.

- How to authorize the UE Reader in topology 2.

- Whether to define areas or Reader sets to select Readers.

2. Actions:

To SA3, SA5, RAN2 and RAN3 group.

ACTION: 	SA2 kindly asks SA3, SA5, RAN2 and RAN3 to take the above information for your future work.

3. Date of Next TSG SA WG2 Meetings:

TSG SA2 Meeting #166-Adhoc-e	20th Jan – 24th Jan, 2025			Online

TSG-SA2 Meeting #167		17th Feb – 21st Feb, 2025			Athens, GR

TSG-SA2 Meeting #168		7th Apr – 11th Apr, 2025			Goteborg, Sweden