3GPP TSG-SA WG2 Meeting #75 	 S2-095067

# Kyoto (Japan), 31 August – 4 September 2009

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.221 | CR | 0084 | rev | - | Current version: | 8.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interaction of CSFB with SMS over SGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Research In Motion |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | SAES-LTE |  |  |  |  |  |  | Date: |  | 24/09/2208 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | Existing CSFB solution in TS 23.272 with the support of SMS over SGs (defined in S2-094958, S2-094958, S2-094860 and S2-094861) inroduces a scenario that is clear in terms of behavior for voice centric UEs, but that is unclear regarding to the behavior expected from data centric UEs with respect to the use of CSFB. In fact, upon receiving a combined EPC/IMS Attach Accept or a combined EPS/IMSI TAU Accept with the indication of “SMS only”, a data centric UE does not know whether CSFB is supported for the data centric UE or not. As a matter of fact, the network may still want to support CSFB only for data centric UEs even if the rtesponse to the UE contained the “SMS only” indication. A CSFB Supported indication has been added in the combined EPS/IMSI Attach/TAU Accept in TS 23.272 from the network to indicate that CSFB is supported. If the network provides such indication, the MME shall attach the UE for EPS and IMSI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The behavior of the UE upon receiving the “SMS only” indication and the CSFB supported indication is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An operator that wants to provide CSFB to data centric UEs even when SMS only can be provided to vocie centric UEs, e.g. to ensure that voice centric UEs reselect to 2G/3G as per the behavior described in TS 23.221, has no way to indicate to a data centric UE that the indication of “SMS only” does not mean that CSFB is not supported for data centric UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2a, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | 23.272 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ***** FIRST CHANGE *****

## 7.2a	Domain selection for UE originating sessions / calls

For UE originating calls, the UE performs access domain selection. The UE shall be able to take following factors into account for access domain selection decision:

-	The state of the UE in the circuit switched domain. This state information shall be included: Detached, Attached, Attached for SMS only.

-	The state of the UE in the IMS. The state information shall include: Registered, Unregistered.

-	The domain used by an existing session / call for the same service.

-	User preferences and any available operator policy.

-	The "IMS voice over PS session supported indication" as defined in TS 23.060 [2] and TS 23.401 [27].

-	Whether the UE is expected to behave in a "Voice centric" or "Data Centric" way

-	The “CSFB Supported indication” as defined in TS 23.272 [XX]

-	The “SMS only” indication as defined in TS 23.272 [XX]

The service domain selection functionality will decide whether the call is serviced in the CS domain or the IMS. Service domain selection functionality may take into account for originating calls whether the user is roaming or not, user preferences, service subscription and operator policy. If the UE is configured for Voice over IMS, the service domain selection functionality should take the "IMS voice over PS session supported indication" into account and should only initiate IMS voice calls (with the voice bearer in the PS domain) using the RAT where the "IMS voice over PS session supported indication" applies and indicates support. The "IMS voice over PS session supported indication" applies to E-UTRAN when received in E-UTRAN, and applies to UTRAN when either received in GERAN or UTRAN.

To allow for appropriate domain selection for originating voice calls, the following applies for a CSFB and IMS capable UE which support E-UTRAN:

-	It should be possible to provision UEs with the following HPLMN operator preferences on how a CSFB/IMS enabled UE is supposed to handle voice services (See Annex A for examples on UE behaviour for each configuration):

-	CS Voice only: the UE will not attempt to initiate voice sessions over IMS using a PS bearer. This does not preclude the use of IMS signalling to control a CS bearer (see TS 23.292 [31]). The UE will attempt combined EPS/IMSI attach for CSFB and SMS (as defined in TS 23.272 [30]). Upon successful combined EPS/IMSI attach with “SMS only” indication” and CSFB Supported indication, a data centric UE will use CSFB and SMS services over the current RAT (as defined in TS 23.272 [30])

-	CS Voice preferred, IMS PS Voice as secondary: the UE will try preferably to use the CS domain (possibly using IMS signalling, see TS 23.292 [31]) to originate and terminate voice calls. The UE will attempt combined EPS/IMSI attach for CSFB and SMS (as defined in TS 23.272 [30]) and if combined EPS/IMSI attach for CSFB fails or succeeds with an “SMS only” indication, the UE attempts voice over IMS. If combined EPS/IMSI attach succeeds and the UE is returned the “SMS only” and the CSFB Supported indication, a data centric UE will use CSFB and SMS services over the current RAT (as defined in TS 23.272 [30])

-	IMS PS Voice preferred, CS Voice as secondary: the UE will try preferably to use IMS to originate and terminate voice sessions. If the UE fails to use IMS for voice e.g. due to "IMS voice over PS session supported indication" indicates voice is not supported (in Attach accept or subsequent TAU accept), then the services are provided using CS domain (as defined in TS 23.272 [30]), possibly using IMS services (see TS 23.292 [31]). The UE can either perform combined EPS/IMSI attach or EPS attach when attaching to E-UTRAN.

If the UE initially performed an EPS attach, after the UE fails to use IMS the UE attempts a combined EPS/IMSI TAU  for CSFB and SMS (as defined in TS 23.272 [30]). If combined EPS/IMSI attach for CSFB fails or succeeds with an “SMS only” indication, a data centric UE a data centric UE will use SMS services over the current RAT (as defined in TS 23.272 [30]). A data centric UE will also use CSFB if it receives the CSFB Supported indication.

If the UE initially performed a combined EPS/IMSI attach procedure, and the attach for CSFB fails or succeeds with an “SMS only” indication, and the UE fails to use IMS, then a data centric UE will use SMS services over the current RAT (as defined in TS 23.272 [30]). A data centric UE will also use CSFB if it receives the CSFB Supported indication in the combined EPS/IMSI attach accept.

NOTE:	Whether to perform combined EPS/IMSI attach or EPS attach for the case of "IMS PS Voice preferred" is to be evaluated in each release of the specifications.

-	IMS PS Voice only: the UE will not attempt combined EPS/IMSI attach (to support voice services) and perform IMS registration indicating support for voice.

-	A CSFB/IMS enabled UE may behave in either a "Voice centric" or "Data centric" way:

-	UE acting in a "Voice centric" way would always try to ensure that Voice service is possible. A CSFB/IMS enabled UE acting in a "Voice centric" way that cannot obtain IMS voice over PS session service, should select a cell of any RAT that provides access to the CS domain. In this case, when CSFB (as defined in TS 23.272 [30]) is not supported in the network, the UE should camp only on RATs that provides access to the CS domain (e.g. GERAN and UTRAN) and disable E-UTRAN capability.

-	UE acting in a "Data centric" way would always try to ensure it gets PS data connectivity, e.g. the UE would stay in the current RAT for PS data connectivity even when voice service is not obtained. A CSFB/IMS enabled UE acting in a "Data centric" way that cannot obtain IMS voice over PS session service in EPS, should continue to stay in EPS even when the EPS does not support CSFB (as defined in TS 23.272 [30]).

In limited service state, a UE should always camp on a RAT which is likely to support emergency calls.

The TS 23.167 [33] describes when an emergency call is to be initiated in the CS domain or using IMS. When the CS domain is to be used the following applies:

-	A CSFB/IMS enabled UE which is registered in the CS domain, e.g. using CSFB procedures or using above-mentioned selection principles, shall initiate emergency call attempts in the CS domain by using available CS domain procedures.

-	A CS Voice capable UE which is not registered in the CS domain shall initiate emergency calls in the CS domain by autonomously switching to a RAT which supports the CS domain (e.g. GERAN or UTRAN).

### ***** NEXT CHANGE *****

Annex A (informative):
Guidance for CSFB and IMS enabled UE implementations

# A.1	General

This annex provides some examples on how the UE can make use of the following indications/settings prior to accessing E-UTRAN:

-	"CS Voice only", "IMS PS Voice only", "prefer CS Voice with IMS PS Voice as secondary", or "prefer IMS PS Voice with CS Voice as secondary", and

-	"Voice centric" or "Data centric".

# A.2	IMS PS Voice preferred

## A.2.1	EPS Attach

The following figure A.2.1-1illustrates the UE behaviour when performing non-combined EPS/IMSI attach, with the setting of: "prefer IMS PS Voice with CS Voice as secondary".

Figure A.2.1-1: UE behaviour for IMS PS Voice preferred with CS Voice as secondary, non combined EPS/IMSI attach

## A.2.2	Combined EPS/IMSI attach

The following figure A.2.2-1illustrates the UE behaviour when performing combined EPS/IMSI attach, with the setting of: "prefer IMS PS Voice with CS Voice as secondary".

Figure A.2.2-1: UE behaviour for IMS PS Voice preferred with CS Voice as secondary, combined EPS/IMSI attach

# A.3	CS Voice preferred

The following figure A.2.1-1illustrates the UE behaviour with the setting of: "prefer CS Voice with IMS PS Voice as secondary".

Figure A.3-1: UE behaviour for CS Voice preferred with IMS PS Voice as secondary

# A.4	IMS PS Voice only

The following figure A.4.1-1illustrates the UE behaviour with the setting of: "IMS PS Voice only".

Figure A.4-1: UE behaviour for IMS PS Voice only

# A.5	CS Voice only

The following figure A.5.1-1illustrates the UE behaviour with the setting of: "CS Voice only".

Figure A.5-1: UE behaviour for CS Voice only

### ***** END OF CHANGES *****