TSG-SA WG3 (Security) meeting #9		TSGS-WG3#9(99)493

Helsinki, 7-9 December, 1999

From:	S3

To:	R2, R3, T3, N1(also to plenaries?)

Title:	USIM triggered authentication and key setting during PS connections

3G TS 33.102 v3.2.0 section 6.4.3 specifies a mechanism which allows the USIM to trigger authentication at the start of an RRC connection if a counter on the USIM exceeds an operator controlled threshold. An S3-approved CR in <CR number> (attached) extends this so that authentication can be triggered by the USIM during a PS connection. Note that section 6.4. in 33.102 v3.2.0 already specifies that the network should be able to initiate authentication and key setting during a PS connection. See also the S3-approved CR to this section in <CR number> (attached).

USIM triggered authentication and key setting during a connection may be useful in the PS mode where long connections lasting several days might be expected. One of the objectives for 3G security is to minimise the amount of trust that needs to be placed in the serving network. USIM triggered authentication during a PS connection can help to minimise the trust that the home environment needs to place in the serving network to implement an appropriate re-authentication policy for long PS connections.

In order to provide this capability three things are necessary:

- It shall be possible that the security mode negotiation can be run during a PS connection and that the new keys are taken into use immediately. R2 and R3 are asked to confirm that this can be done.

- It shall be possible for the USIM  to request the current HFN value to the USIM on a regular basis during a connection and for the USIM to trigger an-authentication and key setting procedure if the operator controlled threshold is exceeded. T3 are asked to confirm that appropriate functionality can be provided by the USIM.

- It shall be possible for the UE to indicate to the MSC/SGSN during a connection that the-authentication and key setting procedure should be run. This could be done, for example, by including KSI as part of an existing or new Layer 3 signalling message (KSI would be set to all 1s to indicate that no valid key set is available for use). N1 are asked to confirm that such and indication can be made to the network during a connection.

USIM and network initiated authentication and key setting during a CS connection is for further study.