| 3GPP TSG SA WG3 Meeting #9, Helsinki, Finland, 7th-9th December |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | S3-99516 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | TS 33.102 |  |  |  |  |  |  |  | CR |  |  | xx |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | V 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | SA # 9 |  |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | S3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 03-12-1999 |  |  |  |  |  |
| Subject: |  |  | Handling of AVs in VLR at Location Update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | Security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Current text explaining VLR/SGSN behaviour at reception of “location update” request is modified to clarify that proposed procedure shall not apply to subsequent/periodic “location update” requests in the same VLR/SGSN area after first registration. Otherwise, VLR/SGSN behaviour would be totally inefficient since valid AVs at VLR/SGSN would be discarded and new AVs would have to be requested to HE/AuC each time a MS performs periodic “location update” requests within the same VLR/SGSN area. Therefore, it is clarified that proposed behaviour applies to first registration at VLR/SGSN (the rest of the clause has also been updated to show that Authentication and Key Agreement is applicable to SGSN). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 6.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

### 6.3.3	Authentication and key agreement

The purpose of this procedure is to authenticate the user and establish a new pair of cipher and integrity keys between the VLR/SGSN and the MS. During the authentication, the user verifies the freshness of the authentication vector that is used.

Figure 7: Authentication and key establishment

The VLR/SGSN invokes the procedure by selecting the next unused authentication vector from the ordered array of authentication vectors in the VLR/SGSN database.  The VLR/SGSN sends to the user the random challenge RAND and an authentication token for network authentication AUTN from the selected authentication vector.

Upon receipt the user proceeds as shown in Figure 8.

Figure 8: User authentication function in the USIM

Upon receipt of RAND and AUTN the user first computes the anonymity key AK = f5K (RAND) and retrieves the sequence number SQN = (SQN Å AK) Å AK.

Next the user computes XMAC = f1K (SQN || RAND || AMF) and compares this with MAC which is included in AUTN. If they are different, the user sends user authentication reject back to the SN/VLR with an indication of the cause and the user abandons the procedure.

Next the user verifies that the received sequence number SQN is in the correct range.

The USIM keeps track of a counter: SQNMS.

To verify that the sequence number SQN is in the correct range, the USIM compares SQN with SQNMS. If SQN > SQNMS the MS considers the sequence number to be in the correct range and subsequently sets SQNMS to SQN.

Note:	The MS and the HE have some flexibility in the management of sequence numbers. Annex C and Annex F.3 contain alternative methods for the generation and verification of sequence numbers.

If the user considers the sequence number to be not in the correct range, he sends synchronisation failure back to the VLR/SGSN including an appropriate parameter, and abandons the procedure.

The synchronisation failure message contains the parameter RANDMS || AUTS. 
Here RANDMS is the random value stored on the MS which was received in user authentication request causing the last update of SQNMS . 
It is AUTS  = Conc(SQNMS ) || MACS.
Conc(SQNMS) = SQNMS  f5K(RANDMS) is the concealed value of the counter SQNMS in the MS, and. 
MACS = f1*K(SQNMS || RAND || AMF) where RAND is the random value received in the current user authentication request. 
f1* is a message authentication code (MAC) function with the property that no valuable information can be inferred from the function values of f1* about those of f1, ... , f5 and vice versa.

The AMF used to calculate MACS assumes a dummy value of all zeros so that it does not need to be transmitted in the clear in the re-synch message.

The construction of the parameter AUTS in shown in the following Figure 9:

Figure 9: Construction of the parameter AUTS

If the sequence number is considered to be in the correct range however, the user computes RES = f2K (RAND) and includes this parameter in a user authentication response back to the VLR/SGSN. Finally the user computes the cipher key CK = f3K (RAND) and the integrity key IK = f4K (RAND). Note that if this is more efficient, RES, CK and IK could also be computed earlier at any time after receiving RAND. The MS stores RAND for re-synchronisation purposes.

Upon receipt of user authentication response the VLR/SGSN compares RES with the expected response XRES from the selected authentication vector. If XRES equals RES then the authentication of the user has passed. The VLR/SGSN also selects the appropriate cipher key CK and integrity key IK from the selected authentication vector.

Conditions on the use of authentication information by the VLR/SGSN: Using the procedures described in subsections 6.3.1, 6.3.2 and 6.3.4, authentication vectors will have to be used in the specific order in which they were generated, otherwise the user will reject the authentication attempt. The VLR/SGSN shall use an authentication vector only once and, hence, shall send out each user authentication request RAND || AUTN only once no matter whether the authentication attempt was successful or not. A consequence is that authentication vectors cannot be reused. When a user changes from one VLR/SGSN to another one and the new VLR/SGSN requests remaining authentication vectors from the old VLR/SGSN (cf. subsection 6.3.4) then the old VLR/SGSN shall not retain any copies of these authentication vectors. When a VLR/SGSN receives a “cancel location” request for a certain user it shall delete all authentication vectors relating to that user. When a VLR/SGSN receives a “location update” request as the first registration of a user and the VLR/SGSN notices that authentication vectors relating to that user are still stored in the VLR/SGSN it will delete this information and request fresh authentication vectors from the HE/AuC or unused authentication vectors from the previous VLR/SGSN.

Different rules may apply when one of the alternative schemes for sequence number handling described in Annex C or Annex F.3 are applied. This is true in particular when the schemes based on windows or lists described in Annexes C.3 and C.4 are applied.