3GPP TSG-SA3 Meeting #100e 	S3-201588-r1

- e-meeting, 17 – 28 August 2020

- Source:	Nokia, Nokia Shanghai Bell, Huawei, Hisilicon

- Title:	External 5GS TSN user plane interfaces

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Description of KI resuting in security requirements for the external user plane interfaces at DS-TT and NW-TT for inclusion into TR 33.851.

# Rational

RFC 7384 defines security requirements for time protocols. A non-compliant 5GS may not meet security requirements in a TSN domain and thus be unusable as TSN bridge or if used pose a security risk to other nodes in the network domain. For integrating a 5GS as a TSN bridge in an IEEE TSN network, the 5GS must be compliant with potential security requirements in a TSN domain.

In this KI, the security requirements for the external user plane interfaces at DS-TT and NW-TT are to be determined. Exemplary security requirements are confidentiality, authenticity, integrity and replay protection. RFC 7384 introduces IPsec, MACsec or IEEE 1588-2008 Annex K to fulfil these security requirements.

# References

[1]	3GPP TR 21.905

[2]	3GPP TS 33.501

[3]	3GPP TS 23.501

[4]	3GPP TR 23.700-20

[5]	IEEE 802.1Qcc

[6]	IEEE 1588-2008

[7]	RFC 7384

## Abbreviations

5G				Fifth Generation

5GS				Fifth Generation System

ARP				Address Resolution Protocol

BMCA				Best Master Clock Algorithm

CNC				Centralized Network Configuration

CP				Control Plane

CUC				Centralized User Configuration

DS-TT				Device Side Translator

DoS				Denial of Service

gPTP				generalized Precision Time Protocol

IIoT				Industrial Internet of Things

IP				Internet Protocol

KI				Key Issue

Ln				Layer n

MAC				Media Access Control

NW-TT				Network Side Translator

PTP				Precision Time Protocol

TSC				Time Sensitive Communication

Rel				Release

UE				User Equipment

TSN				Time Sensitive Networking

TSN AF				TSN Application Function

UPF				User Plane Function

UP				User Plane

****** START OF CHANGES

## 5.X	Key issue #X: External 5GS TSN user plane interfaces

## 5.X.1	Key issue details

RFC 7384 [7] defines security requirements for time protocols. A non-compliant 5GS may not meet security requirements in a TSN domain and thus be unusable as TSN bridge or if used pose a security risk to other nodes in the network domain. For integrating a 5GS as a TSN bridge in an IEEE TSN network, the 5GS must be compliant with potential security requirements in a TSN domain.

In this KI, the security requirements for the external user plane interfaces at DS-TT and NW-TT are to be determined. Exemplary security requirements are confidentiality, authenticity, integrity and replay protection. RFC 7384 [7] introduces IPsec, MACsec or IEEE 1588-2008 [6] Annex K to fulfil these security requirements.

NOTE: We are not aware of any IEEE TSN specifications on security requirements on user plane, these requirements are derived depending on the use case.

Editor's note: It is ffs if TSN communication security requirements other than for time synchronization ahould be addressed as well.

### 5.X.2	Threats

TSN domains not meeting the security requirements from RFC 7384 [7] for time synchronization are vulnerable to:

- Rogue grand master attacks: A malicious node may send tampered messages to win the Best Master Clock Algorithm (BMCA)-based selection of the Grand Master Clock node for the TSN domain. Being the Grand Master Clock node, the malicious node may distribute manipulated times (accuracy degradation and false time).

- Spoofing: A malicious node may pose as Grand Master Clock node or intermediate node with high clock priority to distribute manipulated times (accuracy degradation and false time).

- Packet interception: In a man-in-the-middle scenario, an attacker may intercept (g)PTP messages and remove them to prevent synchronization (DoS), manipulate the packet delay (accuracy degradation and false time) or manipulate the messages content (accuracy degradation and false time).

- L2/L3 DoS attacks: Malicious nodes may use IP spoofing, ARP spoofing, MAC flooding and other attack techniques on L2/L3 to prevent (g)PTP communication (DoS). These attack categories may also affect other services in the network domain.

The impact of these attacks are DoS, accuracy degradation and false times being synchronized.

### 5.X.3	Potential security requirements

5GS shall support the security requirements for time synchronisation within a TSN working domain as described RFC 7384 [7]:

5GS acting as a TSN bridge shall mutual authenticate with other non-5GS TSN bridges.

gPTP and PTP communication shall be integrity and replay protected.

gPTP and PTP communication may be confidentiality protected.

NOTE: Confidentiality is stated as optional for time synchronization in RFC 7384 [7].

Editor’s note: Whether 5GS mandates security properties is ffs as this prevents usage of the 5GS in TSN networks not supporting the security requirements, e.g. in brownfield deployments.

****** END OF CHANGES