3GPP TSG-SA3 Meeting #100e 	S3-201589

- e-meeting, 17 – 28 August 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	N60 interface security

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# Rational

Existing solutions in 3GPP 33.501 [2] require secure communication with UP protection mechanisms between DS-TT and NW-TT ([2], Annex L). NW-TT/UPF to RAN communication via N3 interface is secured as defined in [2], clause 9.3. However, for the communication via N60 interface, currently no associated security requirements are specified in case DS-TT and UE are implemented separately. Thus, this KI studies whether additional specification of N60 interface security requirements is needed.

# References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501

[3]	3GPP TS 23.501

[4]	3GPP TR 23.700-20

[5]	IEEE 802.1Qcc

[6]	IEEE 1588-2008

[7]	RFC 7384

## Abbreviations

5G				Fifth Generation

5GS				Fifth Generation System

ARP				Address Resolution Protocol

BMCA				Best Master Clock Algorithm

CNC				Centralized Network Configuration

CP				Control Plane

CUC				Centralized User Configuration

DS-TT				Device Side Translator

DoS				Denial of Service

gPTP				generalized Precision Time Protocol

IIoT				Industrial Internet of Things

IP				Internet Protocol

KI				Key Issue

Ln				Layer n

MAC				Media Access Control

NW-TT				Network Side Translator

PTP				Precision Time Protocol

TSC				Time Sensitive Communication

Rel				Release

UE				User Equipment

TSN				Time Sensitive Networking

TSN AF				TSN Application Function

UPF				User Plane Function

UP				User Plane

****** START OF CHANGES

## 5.X	Key issue #X: N60 interface security

### 5.X.1	Key issue details

While 3GPP Rel-16 co-locates NW-TT/UPF, it does not specify if DS-TT and UE are co-located or separate (see Rel-16, 3GPP 23.501 [3], clause 4.8.2, Note 3). If both are separate, the N60 interface is used for the communication between DS-TT and UE. Rel-16 leaves the N60 interface up to implementation. However, security requirements are needed to assure threat mitigation.

Existing solutions in 3GPP 33.501 [2] require secure communication with UP protection mechanisms between DS-TT and NW-TT ([2], Annex L). NW-TT/UPF to RAN communication via N3 interface is secured as defined in [2], clause 9.3. However, for the communication via N60 interface, currently no associated security requirements are specified in case DS-TT and UE are implemented separately. Thus, this KI studies whether additional specification of N60 interface security requirements is needed.

Note 1: 3GPP Annex L has to be paraphrased to better reflect the current 3GPP security architecture

Editor’s Note: It is ffs, if this key issue impacts TSN communication in general, in which case a SID update is to be considered.

### 5.X.2	Threats

TBD

### 5.X.3	Potential security requirements

TBD

****** END OF CHANGES