3GPP TSG-SA3 Meeting #100-e 	S3-201627

- e-meeting, 17-28 August 2020													Revision of S3-20xxxx

- Source:	China Telecom

- Title:	TUAK AKA authenticaiton

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution proposes content for the TR33.845 to describe the TUAK AKA authenticaiton.

# 2	References

[1] TS 35.231, " Specification of the Tuak algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm specification ".

# 3	Rationale

In TR 33.845, the content about the TUAK AKA authenticaiton is empty.

# 4	Detailed proposal

*********************************************** Start of Changes ************************

## 5.3	TUAK AKA authentication

To enable TUKA authentication algorithm, the following parameters are needed:

-	TOP (the operator variant algorithm configuration field);

-	TOPc (value derived from TOP and K);

-	ALGONAME (value specified as the ASCII representation of the string "TUAK1.0");

-

-	the length of K (K is a 128-bit or 256-bit subscriber key that is an input to the functions f1, f1*, f2, f3, f4, f5 and f5*);

-	the length of MAC-A (MAC-A is a 64-bit, 128-bit or 256-bit network authentication code that is the output of the function f1);

-	the length of MAC-S (MAC-S is a 64-bit, 128-bit or 256-bit resynchronization authentication code that is the output of the function f1*);

-	the length of RES (RES is a 32-bit, 64-bit, 128-bit or 256-bit signed response that is the output of the function f2);

-	the length of CK (CK is a 128-bit or 256-bit confidentiality key that is the output of the function f3);

-	the length of IK (IK is a 128-bit or 256-bit integrity key that is the output of the function f4);

*********************************************** End of changes ******************************