3GPP TSG-SA3 Meeting #100e 	S3-201759

- e-meeting, 17 – 28 August 2020														merge of S3-201617, S3-201949, S3-201808, S3-202024

- Source:	Huawei, Hisilicon, Interdigital, Qualcomm Incorporated,CATT, Ericsson

- Title:	New key issue on security of UE-to-Network Relay

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

NA

# 3	Rationale

Comparing to LTE ProSe, the UE-to-Network Relay is enhanced in 5G to support commercial case. This may bring new security requirements comparing to LTE Prose where the UE-to-Network Relay is only used in public safety scenario as defined in clause 4.4.3 of TS 23.303[YY]. Public safety enabled UEs can be considered under control by polices or government. When a UE-to-Network Relay is used in commercial case, the UE-to-network Relay may be a commercial UE that could belong to any person. In this case, the trust relationship between remote UE and relay UE is not as strong as the trust relationship between public safety enabled UEs.

# 4	Detailed proposal

*************** Change 1***************

## X.Y	Key Issue #Y: Security of UE-to-Network Relay

### X.Y.1	Key issue details

In KI#3 of TR 23.752[xx], the UE maybe be able to access the network via the direct network communication or the indirect network communication as showing in figure x.y.1. The path#1 is direct network communication path and the path#2 and path#3 are indirect network communication paths via different UE-to-network Relays.

Figure x.y.1

The UE-to-Network relay is registered to the 5GS as a UE. In order to provide service to the remote UE, the UE-to-Network relay needs to establish an NR PC5 link with the Remote UE. Security for PC5 link establishment is documented for LTE Prose in TS 33.303 and for eV2X in TS 33.536. However, it should be studied how to accommodate such procedures to 5G Prose.

For UE-to-Network relay, two options (Layer-2 UE-to-Network relay and Layer-3 UE-to-Network relay) are under consideration in TR 23.752 [xx]. Both options commonly provide network access service to remote UE with the following differences.

- Layer 2 relay: remote UE is registered to the 5GC and has an AS security context established with the gNB in the connected mode.

- Layer 3 relay: remote UE may be registered to the 5GC, but does not have an AS security context.

Meanwhile, both options require PC5 unicast link between the remote UE and UE-to-Network relay. Therefore, it should be studied how to establish PC5 link securely (e.g., authentication and security context establishment) for both options.

TR 23.752 [1] in Clause 5.3, Key Issue #3: Support of UE-to-Network Relay has the following key issue:

-	How to transfer data between the Remote UE and the network over the UE-to-Network Relay.

NOTE 1: Security and privacy aspects will be handled by SA WG3.

Comparing to LTE ProSe, the UE-to-Network Relay is enhanced in 5G to support commercial case. This may bring new security requirements comparing to LTE Prose where the UE-to-Network Relay is only used in public safety scenario as defined in clause 4.4.3 of TS 23.303[YY]. Public safety enabled UEs can be considered under control by polices or government. When a UE-to-Network Relay is used in commercial case, the UE-to-network Relay may be a commercial UE that could belong to any person. In this case, the trust relationship between remote UE and relay UE is not as strong as the trust relationship between public safety enabled UEs.

### X.Y.2	Security threats

Lack of security at PC5 link establishment for UE-to-Network relay may cause to DoS attacks against the remote UE.

Lack of security at PC5 link establishment for UE-to-Network relay may allow MitM attack where the attacker can eavesdrop, modify, or inject messages to the remote UE.

If the UE-to-network is used for commercial services, and a remote UE uses this relay to connect to the 5G network. Failure to protect integrity and confidentiality of information exchanged between the Remote UE and the network over the UE-to-Network Relay will open vulnerability in 5GS and allow various attacks such as unauthorised access. If the UE-to-Network Relay is compromised, the security (i.e., the integrity and confidentiality) of information between the Remote UE and the network may be compromised.

The data sent between the Remote UE and the UE-to-Network Relay may be modified or eavesdropped by an attacker. The attacker may even replay or insert malicious data to the receiver.

X.Y.3	Potential Security requirements

The system shall support a secure means to establish a PC5 link between the remote UE and the UE-to-Network relay.

Confidentiality protection, Integrity protection and replay-protection shall be supported between the remote UE and the 3GPP network.

The system shall provide means to terminate security (i.e, the integrity and confidentiality protection of information between the Remote UE and the network over the UE-to-Network Relay) on the Remote UE and the network.

Data sent between the Remote UE and the UE-to-Network Relay, and between the UE-to-Network Relay and the 3GPP network should be encrypted, integrity protected and replay protected.

********** End of Change **********