3GPP TSG-SA3 Meeting #100e 	S3-201835

- e-meeting, 17 – 28 August 2020						  								Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:		EC: New Key issue on the transport security for the EDGE-1-9

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new Key issue on transport security for the interfaces between 5GC and ECS/Edge Data network

# 2	References

[1]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

# 3	Rationale

TR 23.558 [1], clause 6.2 describes a new architecture for enabling edge applications, i.e.

New interfaces (i.e. EDGE-1-9) were introduced. Security protection on these interfaces shall be studies, i.e. confidentiality, integrity and replay-protection, except EDGE-5, since EDGE-5 is out of the scope of this release of this specification, according to TS 23.558[xx].

# 4	Detailed proposal

It is proposed to approve the following key issue.

*************** Start of the changes ****************

## 5.X	Key issue #X: Transport security for the EDGE-1-9 interfaces

### 5.X.1	Key issue details

TS 23.558 [aa], clause 6.2 describes a new architecture for enabling edge applications, i.e.

New interfaces (i.e. EDGE-1-9) were introduced in the architecture for enabling Edge Applications. This key issues studies the related transport security, i.e. confidentiality, integrity and replay-protection.

- Type A (Between UE and Edge servers):

  - EDGE-1: between EEC and EES

  - EDGE-4: between EEC and EAS

  - EDGE-5: between EEC and Application Client(s)

NOTE:	Details of the EDGE-5 is out of scope of this release of this specification, according to TS 23.558[xx]

- Type B (Between 3GPP core and Edge servers):

  - EDGE-2: between 3GPP Core network and EES

  - EDGE-7: between 3GPP Core network and ECS

  - EDGE-8: between 3GPP Core network and EAS

- Type C (Between Edge servers):

  - EDGE-3: between EAS and EES

  - EDGE-6: between EES and ECS

  - EDGE-9: between EES(s)

### 5.X.2	Threats

Without confidentiality, integrity and replay protection, an attacker may eavesdrop or manipulate or replay the communication or initiate the MITM attacks on the interface.

### 5.X.3	Potential security requirements

Confidentiality protection, integrity protection and replay-protection shall be supported on the EDGE-1-4, and EDGE 6-9 interfaces.

*************** End of the changes ****************