3GPP TSG-SA3 Meeting #100e 	S3-201836

- e-meeting, 17 – 28 Augest 2020						  							merge of S3-20201623 and S3-202025

- Source:	Huawei, Hisilicon, Interdigital, Ericsson

- Title:		5G ProSe: New Key issue on authorization in the UE-to-UE relay scenario

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

This contribution proposes a new Key issue on authorization in the UE-to-UE relay scenario

# 2	References

[1]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS) ".

# 3	Rationale

TR 23.752 [1], key issue #4 describes the issue on the support of UE-to-UE Relay, i.e.

“-	Whether and how for the network can control the UE-to-UE Relay operation, at least including how to:

-	Authorize the UE-to-UE Relay, e.g. authorize a UE as UE-to-UE Relay?

-	Authorize the Remote UE to access a UE-to-UE Relay?

…

NOTE 2:	For security aspects, coordination with SA3 is needed.”

The above authorizaiton issue shall be studied.

# 4	Detailed proposal

It is proposed to approve the following key issue.

*************** Start of the changes ****************

## X.Y	Key issue #X.Y: Authorization in the UE-to-UE relay scenario

### X.Y.1	Key issue details

TR 23.752 [aa], key issue #4 describes the issue on the support of UE-to-UE Relay, i.e.

“-	Whether and how for the network can control the UE-to-UE Relay operation, at least including how to:

-	Authorize the UE-to-UE Relay, e.g. authorize a UE as UE-to-UE Relay?

-	Authorize the Remote UE to access a UE-to-UE Relay?

…

NOTE 2:	For security aspects, coordination with SA3 is needed.”

From the security point of view, whether the UE can play the UE-to-UE Relay rule shall be assured by the Remote UE. On the contrary, whether the UE can play the remote UE rule shall be assured by the UE-to-UE relay.

3GPP system has to be able to autorise a UE to perform as UE-to-UE Relay and a UE to communicate with another UE via a UE-to-UE Relay. This key issue is to study the authorization issue in the UE-to-UE relay scenario.

### X.Y.2	Threats

An attacker may impersonate the UE-to-UE Relay, If the authorization of the UE-to-UE relay role is not supported, the attacker UE could play the UE-to-UE relay role, and force a UE to camp on to it by passing all the message on between two UEs. It may then deny the UE services between the two UEs, such as drop the message.

An attacker may impersonate the source UE or the target UE.

X.Y.3	Potential security requirements

The 5GS shall support to authorize the UE as a UE-to-UE relay in the UE-to-UE relay scenario.

Authorisation of a UE that requests to be a source UE or a target UE, discovering a UE-to-UE Relay, should be provided.

3GPP system shall provide means to authorise a UE to communicate with another UE via a UE-to-UE Relay.

*************** End of the changes ****************