3GPP TSG-SA WG3 Meeting #100-e 	S3-201989

- e-meeting, 17-28 August 2020														revision of S3-20xabc

- Source:	Ericsson, Huawei

- Title:	New KI: Provisioning

- Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

It is proposed to approve this KI to TR 33.857

# 2	References

[1]	3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

# 3	Rationale

The architectural study documented in 3GPP TR 23.700-07 [1] contains a key issue (KI#4) related to onboarding and remote provisioning of credentials. It is split up into two parts, the actual onboarding for the UE to obtain connectivity and for the UE to obtain NPN credentials from the Provisioning Server. It is left to SA3 to decide on what Default Credentials needs to be available on the UE to securely provide onboarding and provisioning features. The Key Issue proposed in the present document assumes onboarding is completed and treats the security aspects during provisioning of credentials.

# 4	Detailed proposal

#### ***** Start of Change *****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[XX]	3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

#### ***** Next Change *****

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

## 5.X	Key Issue #X Provisioning of Credentials

### 5.X.1	Key Issue Details

This Key Issue aims at addressing security implications introduced in solutions related to Key Issue #4 in TR 23.700-07 [XX].

The objective of Key Issue #4 in TR 23.700-07 [XX] is twofold, UE onboarding and then remote provisioning of non USIM credentials for SNPN. This Key Issue aims at treating the security implications related to the provisioning part.

Designing completely new protocols is not in scope of this key issue.

Editor’s note: other details are FFS.

### 5.X.2	Security Threats

Unprotected provisioning of NPN credentials may cause the NPN credentials to be obtained or manipulated by on-boarding network.

### 5.X.3	Potential Security Requirements

NPN credentials shall be protected from unauthorized access and manipulation during provisioning.

#### ***** End of Change *****