SA WG3 Meeting S3#101-e	S3-202818

e-meeting, November 9 – 20, 2020

Source:	Interdigital

Title:	New solution for TR 33.847 – Privacy handling for Layer-3 UE-to-UE Relay based on IP routing

Document for:	Approval

Agenda Item:	5.9

Work Item / Release:	FS_5G_ProSe_Sec / Rel-17

Abstract of the contribution: This contribution proposes a new solution addressing Key Issue #8: Privacy of information over the UE-to-UE Relay

# 1	Decision/action requested

This contribution proposes to new solution addressing Key Issue #8: Privacy of information over the UE-to-UE Relay and is describing operations of the 5G ProSe UE-to-UE Relay in support of privacy for TR 33.847.

# 2	References

[1]	3GPP TR 23.752 V0.5.1 (2020-10) Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)

[2]	3GPP TR 33.847 V0.2.0 (2020-10) Study on security aspects of enhancement for proximity based services in the 5G System (5GS) (Release 17)

# 3	Introduction

Solution #10 and Solution #32 from TR 23.752 [1] provide solutions for 5G ProSe UE-to-UE Relay operations related to Key Issue #4 from TR 23.752 [1]. These solutions, however, do not describe privacy protection., i.e., the handling of privacy-related information (such as source/destination IDs, UE-to-UE Relay IP address, and other identifiers) is not defined.

To minimize the impacts on the UEs, this solution is proposing to re-use the PC5 Link Identifier Update procedure as defined in TS 33.536 to support IP address change and privacy requirements.

# 4	Proposal

It is proposed to add the following solution #X in TR 33.847 [2]. Note that all the text below is new.

***	BEGIN OF CHANGES	***

6.X Solution #X: Privacy handling for Layer-3 UE-to-UE Relay based on IP routing

6.X.1 Description

This solution addresses Key Issue #8: Privacy of information over the UE-to-UE Relay and describes operations of the 5G ProSe UE-to-UE Relay in support of privacy.

A Source UE (UE1) communicating with a Target UE (UE2) over PC5 unicast links, via a UE-to-UE Relay, may need to change its IP address/prefix and/or other identifiers, e.g., for privacy reasons.

When a Source UE changes its IP address/prefix, its Target UE must be informed of UE1’s new IP address/prefix since communication between UE1 and UE2 is IP-based. Furthermore, the UE-to-UE Relay must as well be informed of UE1’s new IP address/prefix and/or other new identifiers since communication between UE1 and the UE-to-UE Relay is over a PC5 unicast link and the UE-to-UE Relay also handles the IP routing of messages exchanged between the peer UEs. Moreover, since the PC5 unicast link used by UE1 is established with the UE-to-UE Relay, this UE-to-UE Relay also needs to update its identifiers at the same time as the Source UE.

Likewise, UE1 may be communicating with more than one Target UE over the PC5 unicast link via the UE-to-UE Relay. In that case, all Target UEs must be informed of UE1’s new IP address/prefix.

6.X.2 Procedures

Figure 6.X.2-1 illustrates the IP address/prefix change procedure between the Source UE (UE1), the Target UE (UE2), and the UE-to-UE Relay. The Link Identifier Update procedure defined in TS 23.287 [9] is reused between UE1 and the UE-to-UE Relay and a new procedure is defined between the UE-to-UE Relay and UE2 (i.e., Target UE(s)).

This solution applies to Layer-3 based Solution #10 and Solution #32 described in TR 23.752[2]. Since Solution #10 and Solution #32 described in TR 23.752[2] are similar, this proposed solution applies to both of them. The main difference between Solution #10 and Solution #32 described in TR 23.752[2] is the proposed method of IP address/prefix allocation. In Solution #10, the UE-to-UE Relay allocates the IP address/prefix to the UE. In Solution #32, a link-local IP address that is assigned by the UE itself, is used and sent to the UE-to-UE Relay. Differences in the procedure detailed below are explained when needed.

For simplicity, Figure 6.X.2-1 and the detailed text below focus on the change of IP addresses/prefixes. However, it is understood that other identifiers that are updated and exchanged during the Link Identifier Update procedure as specified in TS 23.287 [9] are also included in this proposed procedure. For example, Layer-2 IDs (source/destination) and security information IEs are changed on the PC5 unicast link between UE1 and the UE-to-UE Relay although not shown in Figure 6.X.2-1.

Figure 6.X.2-1: Privacy handling for Layer-3 based UE-to-UE Relay

0) A PC5 unicast link is established between UE1 and the UE-to-UE Relay. Another PC5 unicast link is established between the UE-to-UE Relay and UE2. The Relay maintains a mapping table that includes its PC5 peer UEs’ user info, their IP addresses/prefixes, and associated PC5 unicast links. Both Source/Target UEs learn their peer UE’s IP addresses/prefixes via the Relay UE using DNS. IP data is exchanged between UE1 and UE2 via the UE-to-UE Relay over the PC5 unicast links. The UE-to-UE Relay handles the routing of IP packets to another PC5 unicast link based on the destination IP address/prefix.

1) UE1 is informed (e.g., via the application layer, timer expiration) that its IP address/prefix, Layer-2 ID, and possibly other identifiers (e.g., Application ID) must be changed. UE1 triggers the Link Identifier Update procedure, as defined in TS 23.287 [9], with some changes added for the support of UE-to-UE Relay.

2) UE1 sends a Link Identifier Update Request message to the UE-to-UE Relay over the PC5 unicast link, which includes a “new IP address needed” indication and its peer UE info (i.e., UE2 IP address, UE2 User Info), in addition to the usual parameters sent on the Link Identifier Update request message, e.g., Layer-2 ID and security info.

- The “new IP address needed” indication is used when the UE-to-UE Relay assigns the IP address/prefix to its peer UEs, as defined in sol#10. If the UE uses a link-local IP address, as defined in sol#32, a new link-local IP address is self-allocated on UE1 and sent to the UE-to-UE Relay using the Link Identifier Update Request message.

3) UE-to-UE Relay assigns a new IP address/prefix to UE1 and saves it locally, while still preserving UE1’s current IP address/prefix. UE-to-UE Relay fetches UE2’s entry from its local table based on the peer UE info specified on the Link Identifier Update Request message. UE-to-UE Relay sends a new PC5 Relay Update Request message to UE2 (including UE1’s old IP address/prefix, UE1’s User Info, and UE1’s new IP address/prefix) to inform UE2 about UE1’s new IP address/prefix.

4) UE2 saves UE1’s new IP address/prefix. UE2 sends a new PC5 Relay Update Response message to UE-to-UE Relay including all parameters received on the PC5 Relay Update Request message to ACK them.

- At this point, UE2 still sends/receives IP data using UE1’s old IP address/prefix until an IP packet using UE1’s new IP address/prefix is received. At that point, UE2 starts using UE1’s new IP address/prefix and forgets UE1’s old IP address/prefix.

5) UE-to-UE Relay sends a Link Identifier Update Response message to UE1 including UE1’s new IP address/prefix (and UE-to-UE relay new info e.g., new IP address/prefix, new Layer-2 ID, new security info).

6) UE1 saves its new IP address/prefix and UE-to-UE Relay new IP address/prefix and other new info. UE1 sends back a Link Identifier Update ACK message to the UE-to-UE Relay including all parameters received on the Link Identifier Update Response message. UE1 may start using its new IP address/prefix. The new Layer-2 IDs associated with the PC5 unicast link (i.e., for UE1 and UE-to-UE Relay, as well as new security info) are also used at that point. Once UE1 has received data using UE1’s new IP address/prefix, the old IP address/prefix may be released or deleted.

Editor’s Note: The IP addresses for both UE1 and UE2 should be updated at the same time. Otherwise the changed IP could be indirectly traced using other parameters that are associated with the Link ID Update request/response. It is FFS how to provide such IP addresses update in this solution.

Editor’s Note: Whether IP address change for privacy is always needed is FFS.

### 6.X.3	Solution evaluation

### TBD

***	END OF CHANGES	***