3GPP TSG-SA3 Meeting #101-e 	S3-202918-r1

- e-meeting, 9 - 20 November 2020													Revision of S3-20xxxx

- Source:	ZTE, Huawei

- Title:	Update solution 14 in TR 33.839

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes to update solution 14 in TR 33.839.

# 2	References

[1]			3GPP TR 33.839 v0.2.0

# 3	Rationale

In the step 2a-2b, AF and NEF need to generates a Key-ID for the UE and derives a KLE. But it’s not clear how to generates a Key-ID for the UE and derives a KLE in AF and NEF, Some editor note need to be added.

In the step 3-4. in this solution, the Key-ID and  KLE is generate in NEF, not receive from AF.

# 4	Detailed proposal

**** START OF CHANGES ****

## 6.14	Solution #14: Protection of Network Information Provisioning to Local AF directly

### 6.14.1	Solution overview

This solution addresses the security requirement for the case that the UPF exposes information to local AF directly in the key issue 7.

Editor’s Note: which interface for UPF and AF communicate over is FFS.

### 6.14.2	Solution details

Figure 6.14.2-1 Protection of Network Information Provisioning to Local AF

1. AF establish a TLS session with the NEF, to secure the communication between the AF and NEF.

2a-2b. The AF randomly generates a Key-ID for the UE. And both the AF and NEF derive a KLE based on the shared key negotiated during the TLS establishment. The Key-ID is used to identify the KLE, and the KLE is used to protect the message transmission between UPF and AF. The AF provides the Key-ID to NEF in the request service.

3-4. The NEF initiates the policy authorization with PCF, including the Key-ID and KLE .  PCF initiates the PDU session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3] and provides the Key-ID and KLE to the SMF.

5. SMF sends the notification information with Key-ID and KLE to the UPF.

6. When the QoS monitoring information is received from RAN, the UPF generates MAC-I over the message using the KLE to prove its authenticity. The UPF sends the message including Key-ID and MAC-I to AF.

7. The AF retrieves the KLE based on the received Key-ID and verify the MAC-I.

**** END OF CHANGES ****