3GPP TSG SA WG3 Meeting #101e 	S3-202952-r2

- e-meeting, 9-20 November 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.536 | CR | 0019 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on cross-layer indication triggered by updating the security context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  |  | Date: |  | 2020-11-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the LS from RAN 2 group (R2-2005978) and 3GPP TS 38.331 v16.1.0, the sidelink PDCP re-establishment can be triggered by the following conditions: - During the connection set-up: “Once AS security is activated fo a PC5 unicast link in the upper layers as specified in TS 33.536” - During the rekeying procedures: “If the change of the key is indicated by the upper layers as speficied in TS 33.536” However the current rekeying procedure lacks detailed description about when and how V2X layer triggers the AS layer regarding the key change. |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions of V2X layer indicating the AS layer about the activate/update of the security context. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete standard. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.4.3, 5.3.3.1.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

5.3.3.1.4.3	Security establishment during connection set-up

The clause describes how security is established during connection set-up. The signalling flow is shown in figure 5.3.3.1.4.3-1.

Figure 5.3.3.1.4.3-1: Security establishment at connection set-up

1.	UE_1 has sent a Direct Communication Request to UE_2. This message shall include UE_1's security capabilities (the list of algorithms that UE_1 will accept for this connection) and UE_1's signalling security policy. The UE_1 shall also include Nonce_1 (for session key KNRP-sess generation), and the most significant 8-bits of the KNRP-sess ID in this message if UE_1's signalling integrity protection policy is either "REQUIRED" or "PREFERRED". The most significant 8-bits of the KNRP-sess ID shall be chosen such that UE_1 will be able to locally identify a security context that is created by this procedure. The message may also include a KNRP ID if the UE_1 has an existing KNRP for the UE that it is trying to communicate with. The absence of the KNRP ID parameter indicates that UE_1 does not have a KNRP for UE_2. The message also contains Key_Est_Info (see clause 5.3.3.1.3.2).

2.	UE_2 shall reject the Direct Communication Request if UE_1's signalling security policy is "NOT NEEDED" while UE_2's security policy is "REQUIRED". UE_2 shall also reject the Direct Communication Request if UE_1's signalling security policy is "REQUIRED" while UE_2's security policy is "NOT NEEDED". UE_2 may initiate a Direct Auth and Key Establish procedure with UE_1. This is mandatory if the UE_2 does not have the KNRP and KNRP ID pair indicated in step 1, and signalling is needed to establish the keys for the particular use case.

3.	UE_2 shall send the Direct Security Mode Command message to UE_1. This message shall only contain the MSB and of KNRP ID and optionally Key_Est_Info if a fresh KNRP is to be generated (see clause 5.3.3.1.3). UE_2 shall include the Chosen_algs parameter to indicate which security algorithms the UEs will use to protect the data in the message. The Chosen-algs may only indicate the use of the NULL integrity algorithm if UE_2's signalling integrity security policy is either NOT NEEDED or PREFERRED. UE_2 shall also return the UE_1's security capabilities and UE_1's signalling security policy to provide protection against bidding down attacks. In the case that the NULL integrity algorithm is chosen, the NULL confidentiality algorithm shall also be chosen and UE_2 shall set the KNPR-sess ID of this security context to the all zero value.

The following procedures in step 3 shall only be executed if the UE_2 decides to at least activate the integrity security protection for this connection: UE_2 shall also include Nonce_2 to allow a session key to be calculated, as well as the least significant 8-bits of KNRP-sess ID in the messages. These bits are chosen so that UE_2 will be able to locally identify a security context that is created by this procedure. UE_2 shall calculate KNRP-Sess from KNRP and both Nonce_1 and Nonce_2 (see clause A.3) and then derive the confidentiality (if applicable) and integrity keys based on the chosen algorithms (clause A.2). UE_2 shall integrity protect the Direct Security Mode Command before sending it to UE_1. UE_2 is then ready to receive signalling protected with the new security context UE_2 shall form the KNRP-sess ID from the most significant bits it received in step1 and least significant bits it sent in step3. The AS layer shall be provided with the details of the new security context and indication that this new security context shall be used to integrity protect the Direct Security Mode Command message and can be used to receive protected (both confidentiality and integrity as appropriate) signalling traffic.

4.	On receiving the Direct Security Mode Command, the UE_1 shall first check the Chosen_algs and shall accept the NULL integrity algorithm only if its security policy for signalling integrity protection is either NOT NEEDED or PREFERRED. Then UE_1 shall check the returned UE_1's security capabilities and UE_1's signalling security to avoid bidding down attacks if NULL integrity algorithm is selected for signalling integrity protection. If the above check passes, UE_1 shall send unprotected Direct Security Mode Complete message to UE_2. UE_1 shall set the KNPR-sess ID of this security context to the all zero value.

Under the condition of non-NULL integrity algorithm indicated in the Chosen_algs, UE_1 shall first check that the received LSB of KNPR-sess ID is unique, i.e. has not been sent by another UE responding to this Direct Communication Request. If the LSB of KNPR-sess ID is not unique, then UE_1 shall respond with a Direct Security Mode Reject message including a cause value to specify that the LSB of KNPR-sess ID is not unique. The peer UE-2 receiving a Direct Security Mode Reject message shall inspect the cause value and, if the cause is related to the session identifier uniqueness then, the UE-2 shall generate a new LSB of KNPR-sess ID and reply to UE-1 again (i.e., UE-2 shall send a Direct Security Mode Command message with the new LSB of KNPR-sess ID). UE_2 shall associate the new LSB of KNRP-sess ID with the security context that is created in step 3. UE-2 shall erase the former LSB of KNPR-sess ID from its memory. On receiving this new Direct Security Mode Command, UE_1 shall process the message from the start of step 4.

If the LSB of KNPR-sess ID is unique, UE_1 shall calculate KNRP-sess and the confidentiality and integrity keys in the same way as UE_2. UE_1 shall check that the returned UE_1 security capabilities and UE_1's signalling security policy are the same as those it sent in step 1. UE_1 shall also check the integrity protection on the message. If both these checks pass, then UE_1 is ready to send and receive signalling with this new security context. UE_1 shall send integrity protected and confidentiality protected Direct Security Mode Complete message to UE_2. UE_1 shall form the KNRP-sess ID from the most significant bits it sent in step1 and least significant bits it received in step3. The AS layer shall be provided with the details of the new security context and indication shall be used to send signalling traffic starting with the Direct Security Mode Complete message and can be used to receive protected signalling traffic.

5.	If the Chosen_algs in step 3 includes non-NULL integrity algorithm, UE_2 checks the integrity protection on the received Direct Security Mode Complete. If this passes, UE_2 is now ready to send control signalling and send and receive user plane data protected with the new security context. UE_2 shall send integrity protected and confidentiality protected Direct Communication Accept message to UE_1 with the new security context. The AS layer shall be provided with indication that the new security context shall be used to send signalling traffic starting with the Direct Communication Accept and an indication that after sending that message of the activation of the PC5 unicast user plane integrity and confidentiality protection for the PC5 unicast link (if applicable). UE_2 deletes any old security context it has for UE_1.

6.	After receiving the Direct Communication Accept message, the AS layer shall be provided with an indication of activation of the PC5 unicast user plane integrity and confidentiality protection for the PC5 unicast link (if applicable).

********** END OF 1st CHANGE **********

********** START OF 2nd CHANGE **********

5.3.3.1.4.4	Security establishment during re-keying

By rekeying, the UEs ensure fresh session keys KNRP-sess are used. Optionally the rekeying can also enforce refresh of KNRP. Either UE may rekey the connection at any time. This shall be done before the counter for a PDCP bearer repeats with the current keys. A rekeying operation shall refresh the KNRP-sess and NRPEK and NRPIK, and may refresh KNRP. There is no benefit in running the rekeying procedure if the NULL integrity algorithm is in use, hence it is recommended not to trigger it when using the NULL integrity algorithm. A rekeying operation follows the flows given in figure 5.3.3.1.4.4-1.

NOTE:	The rekeying procedure is not required from security point of view if the connection is unprotected.

Figure 5.3.3.1.4.4-1: Security establishment during rekeying

1.	UE_1 sends a Direct Rekey Request to UE_2. This message shall include UE_1 security capabilities (the list of algorithms that UE_1 will accept for this connection). In addition, if a non-Null integrity algorithm is in use, the message shall include Nonce_1 (for session key generation) and the most significant 8-bits of the KNRP-sess ID. These bits are chosen such that UE_1 will be able to locally identify a security context that is created by this procedure. The message may also include a Re-auth Flag if UE_1 wants to rekey KNRP. The message also contains Key_Est_Info (see clause 5.3.3.1.3.2).

2.	UE_2 may initiate a Direct Auth Key Establish procedure with UE_1. This is mandatory if UE_1 included the Re-auth Flag and signalling is needed to establish KNRP.

3.	This step is the same as step 3 in clause 5.3.3.1.4.3 except that the following differences. The chosen integrity algorithm shall only be NULL if and only if the NULL integrity algorithm is currently in use, the chosen confidentiality algorithm shall only be NULL if and only if the NULL confidentiality algorithm is currently in use and UE_1's signalling security policy is not included in this message.  UE_2 is then ready to receive both signalling and user plane traffic protected with the new security context. The AS layer shall be provided with an indication that the new security context can be used to receive protected user plane traffic in addition to the indications from step 3 in clasue 5.3.3.1.4.3.

4.	This step is the same as step 4 in clause 5.3.3.1.4.3 except the following differences.  UE_1 shall only accept the NULL integrity algorithm if and only if the NULL integrity algorithm is currently in use, UE_1 shall only accept the NULL confidentiality algorithm if and only if the NULL confidentiality algorithm is currently in use, and UE_1 does not check the returned signalling security policy (as it is not sent in this case).  UE_2 is then ready to both send and receive both signalling and user plane traffic protected with the new security context. The AS layer shall receive an indication that the new security context shall be used to send and can be used to receive protected user plane traffic after the Direct Security Mode Complete message has been sent in addition to the indications given in step 4

5.	This step is the same as step 5 in clause 5.3.3.1.4.3 except the following. The Direct Communication Accept message is replaced by Direct Link Rekeying Response message. There is no need for the indication to be ready to receive user plane with the new context as this indication was received at at step 3 but the rest of the indication from step 5 of clause 5.3.3.1.4.3 are needed.

6.	When UE_1 receives message integrity protected with the new security context, it shall delete any old security context it has still stored for UE_2. No indication to the AS layer is needed at this step.

********** END OF 2nd CHANGE **********