3GPP TSG SA WG3 Meeting #101e 	draft_S3-202953-r1

- e-meeting, 9-20 November 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.536 | CR | 0020 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposal for improvement of the user plane security policy handling logic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  |  | Date: |  | 2020-11-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 5.3.3.1.4.2.3 specifies the sequence to handle the user plane security policy, however the current processing logic may lead to the receiving UE not sending the Direct Communication Accept message (i.e. Accept the Direct Communication Request) to the initiating UE which makes incomplete link establishment procedure. The receiving UE only accepts the DCR in step (d) in the current version, but this step starts with the condition “otherwise” which means (b) and (c) are orthogonal to step (d). For this reason, a description to accept the DCR should be added to step (b) to complete the policy handling logic. The description in step (b) about accepting DCR aligns with the description in step (d). |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | A description to accept the DCR should be added to step (b) to complete the policy handling logic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current version leads incorrect UP security policy handling logic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

5.3.3.1.4.2.3	Security policy handling

For a NR PC5 unicast link, the UE shall be provisioned with the following:

-	The list of V2X services, e.g. PSIDs or ITS-AIDs of the V2X applications, with Geographical Area(s) and their security policy which indicates the following:

-	Signalling integrity protection: REQUIRED/PREFERRED/NOT NEEDED

-	Signalling confidentiality protection: REQUIRED/PREFERRED/NOT NEEDED

-	User plane integrity protection: REQUIRED/PREFERRED/NOT NEEDED

-	User plane confidentiality protection: REQUIRED/PREFERRED/NOT NEEDED

NOTE 1:	No integrity protection on signalling traffic enables services that do not require security.

NOTE 2:	Ensuring that only a connection with security is used for a V2X service is guaranteed if the signalling integrity security policy of at least one of the UEs for that V2X service is set to REQUIRED. It is recommended to set this security policy to REQUIRED in order to guarantee security protection.

NOTE 3:	While some V2X applications are similar to Emergency Services and may require similar security policies handling, such V2X applications are outside of the scope of 3GPP.

REQUIRED means the UE shall only accept the connection if a non-NULL confidentiality or integrity algorithm is used for protection of the traffic.

NOT NEEDED means that the UE shall only establish a connection with no security.

PREFFERED means that the UE may try to establish security but may will accept the connection with no security. One use of PREFERRED is to enable a security policy to be changed without updating all UEs at once.

The handling of signalling security policy proceeds as follows:

-	At initial connection, the initiating UE includes its signalling security policy in the Direct Communication Request message. The receiving UE(s) takes this into account when deciding whether to accept or reject the request and when deciding the agreed security policy to be sent back in the Direct Security Mode Command message. The initiating UE can reject the Direct Security Mode Command if the algorithm choice does not match its policy (see clause 5.3.3.1.4.3 for full details of the handling).

All the UP data of PC5 unicast link shall have the same security.

The handling of the user plane security policy proceeds with the following sequence:

a)	At initial connection, the UE that sent the Direct Communications Request shall include the user plane security policy for the service in the Direct Security Mode Complete message.

b)	If the signalling confidentiality protection is not activated, then UEs shall treat their user plane confidentiality policy for the V2X service for this connection as NOT NEEDED and the receiving UE shall set confidentiality for the user plane to off. The receiving UE may accept the Direct Communication Request and the response message shall include the configuration of not activating (i.e. set to off) the user plane confidentiality protection.

c)	The receiving UE shall reject the Direct Communication Request when the following cases occur: 1) if the received user plane security policy had either confidentiality/integrity set to NOT NEEDED and its own corresponding policy is set to REQUIRED or, 2) if the received user plane security policy had either confidentiality/integrity set to REQUIRED and its own corresponding policy is set to NOT NEEDED.

d)	Otherwise, the receiving UE may accept the Direct Communication Request and the response message shall include the configuration of user plane confidentiality protection based on the agreed user plane security policy, set as follows:

1)	User plane confidentiality protection set to off if the received user plane security policy had either confidentiality set to NOT NEEDED and/or its own user plane security policy for the service is set to NOT NEEDED; or

2)	User plane confidentiality protection set to on if the received user plane security policy had either confidentiality set to REQUIRED and/or its own user plane security policy for the service its own corresponding policy is set to REQUIRED; or

3)	User plane confidentiality protection set to off or on otherwise (i.e. when both the received user plane security policy and its own user plane security policy for the service had the confidentiality set to PREFERRED).

User plane integrity protection set following the same rules as confidentiality protection but based on the received and its own user plane integrity protection policy for the service.

Due to the purpose of adding a new V2X service to an existing PC5 unicast link, if the signalling and user plane security policies of the new V2X service are satisfied by the security in use for the PC5 unicast link, the initiating UE shall send the Link Modification Request to the receiving UE. The receiving UE shall reject the Link Modification Request if the security in use for the PC5 unicast link does not match the signalling and user plane security policies of the new V2X service.

The V2X layer of the UE shall pass the security configurations to its AS layer. The security configurations are mutually agreed by both sides' UEs, including the configuration of confidentiality and integrity protection.

********** END OF 1st CHANGE **********