3GPP TSG-SA3 Meeting #101e 	S3-203067-r2

- e-meeting, 9 – 20 November 2020													Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	EC: Framework on the EEC authentication and authorization.

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This contribution proposes to link the solution #1 to key issue #9.

# 2	References

[1]	3GPP TS 33.839	Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC

[2]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in the 5G Core network (5GC)".

# 3	Rationale

The contribution propose an EEC authentication and authorization framework with ECS and EES.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the 1st change ****************

## 6.X	Solution #X: EEC authentication and authorization framework with ECS and EES

### 6.X.1	Introduction

This solution addresses the security requirement for authentication/authorization between EEC and ECS/EES in the key issue #1, and key issue #2. In this solution, the AKMA, and TLS is reused as the building blocks for the EEC authentication framework if the EDGE-4 is deployed based on the UP connection. NAS and SBI interface protection is reused if the EDGE-4 is deployed based on the CP connection. For the authorization, the Oauth is selected for the authorization between EEC and EES.

### 6.X.2	Solution details

Figure 6.X.2-1 EEC authentication and authorization framework with ECS and EES

It is assumed that AKMA, TLS, and OAuth is supported by the UE, ECS, and EES.

Step 1-3. UE with EEC functionality registers in the 5G network, and retrieves ECS information (i.e. ECS Id and AKMA capability) from 5GC. The AKMA capability indicates the ECS support to use AKMA.

There are two options for the authentication and data protection between EEC and ECS.

Option A (The EDGE-4 is deployed based on the UP connection):

Step 4-6. The UE determines to use AKMA based on the received AKMA capability, AKMA defined in the TS 33.535 [6] is reused here to negotiate the preshared key KECS between UE and ECS. Here the A-KID sent from the UE is authenticated by the ECS based on the AKMA mechanism.

Step 7a. EEC and ECS establish the TLS security tunnel based on the preshared key KECS. The authentication is fulfilled based on the TLS.

Option B (The EDGE-4 is deployed based on the CP connection):

Step 7b. The protection between EEC and ECS relies on the NAS and SBI interface protection. The authentication is implicitly performed as the primary authentication.

Editor’s Note: “It is ffs if a CP connection between EEC and ECS is possible."

NOTE: the following EDGE-4 data could be protected based on Option A or Option B according to its connection option.

Step 8. EEC sends the Provisioning request message to the ECS, including its EEC ID, application info.

Step 9. ECS determines the EES info (including EES Id, AKMA capability), and generate the Oauth token with the following claims, i.e. EEC ID, ECS ID, EEC info, the authorized services.

Editor’s Note: It is ffs which identifier for the UE/EEC is included in the token.

NOTE: whether the EEC ID could use the edge service can be authorized based on the local policy. The token for consuming the ECS service may not be needed.

Step 10. ECS sends the token and EES info back to the EEC via the Provisioning response message.

Step 11. Similar with step 4-6, preshared key KEES is negotiated between EEC and EES.

Step 12. EEC and EES establish the TLS security tunnel based on the preshared key KEES. The authentication is fulfilled based on the TLS.

Step 13. EES sends the EEC registration/ discovery request message to the EES, including the token.

Step 14. EES authorizes the EEC based on the verification of the token.

Step 15. If the verification successes, the EES sends the EEC registration/ discovery response message back to the EEC.

### 6.X.3	Solution Evaluation

TBD.

*************** End of the 1st change ****************