- 3GPP TSG SA WG3 Meeting #101e                            	S3-203087

- e-meeting, 9 – 20 November 2020	                         revision S3-20abcd

- Source:	Huawei, Hisilicon

- Title:	New Solution on groupcast security and privacy

- Document for:	Approval

Agenda Item:	5.9

# Decision/action requested

This contribution proposes a new solution for groupcast security and privacy

# References

[1]		3GPP TR 33.847: "Study on security aspects of enhancement for proximity based services in the 5G System (5GS)".

# Rational

The contribution proposes a new solution for groupcast security and privacy.

# Detailed proposal

*************** Start of 1st Change ****************

- 6.Y	Solution #Y: Solution on securely creating destination Layer-2 ID in groupcast communication

- 6.Y.1	Introduction

This solution addresses the KI #13 "Security and privacy of groupcast communication".

- 6.Y.2	Solution details

The detailed solution is illustrated in Figure.

Figure: Procedure for secure conversion of application layer group ID to destination Layer-2 ID

0. Group Setup: Once the group is created, the group management server will send the application layer group ID to the associated UEs and a timer T. It will also send a set of random numbers and a specific sequence in which these random number are to be used. It is assumed that the application layer signalling is protected.

1.	ID Conversion: All the UEs use the application layer group ID and the first random number according to the sequence as an input to a hash function to generate the destination Layer-2 ID.

2.	ID Update: When the timer T expires, a new destination Layer-2 ID is calculated using the next random number according to the sequence. The UEs can listen to both old a new destination Layer-2 IDs, to avoid any time synchronization issues, for a certain period of time or until receives a message with the new ID.

The destination Layer-2 ID is updated until the ProSe application layer changes the group ID.

- The group management server can also send the corresponding materials to generate random numbers rather than sending the random numbers itself.

Editor’s Note: security for group management (including message 0 in the figure) is FFS

- Editor’s Note: time synchronization among group members is FFS

- 6.Y.3	Evaluation

TBA.

*************** End of 1st Change ****************