3GPP TSG-SA3 Meeting #101-e 	S3-203210

- e-meeting, 9th - 20th November 2020												Revision of S3-20xxxx

- Source:	Ericsson

- Title:	ProSe: Key management server(s) in UE-to-network relay scenarios

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new solution for how the Remote UE and the UE-to-network relay finds out the AF for key management for PC5 communication in the commercial case.

# 2	References

[1]	3GPP TR 33.847 "Study on Security Aspects of Enhancement for Proximity Based Services in 5GS"

# 3	Rationale

This solution describes for commercial services how the Remote UE and the UE-to-network relay finds out the AF for key management for PC5 communication between Remote UE and the UE-to-network relay.

# 4	Detailed proposal

It is proposed to add a new solution to the study in [1].

**** START OF CHANGES ****

## 6.Y	Solution #Y: AF for key management in PC5 communication

### 6.Y.1	Introduction

This solution describes how the Remote UE and the UE-to-network relay finds out the address of the common key management server (AF) to be able to communicate over PC5 interface. This solution addresses key issue#4.

This solution is for commercial services. The Remote UE and the UE-to-network Relay have no knowledge of each other beforehand.

### 6.Y.2	Solution details

In this solution the Remote UE has an AF (AF-1) in it’s home PLMN for ProSe key management. The UE-to-network relay has an AF (AF-2) in its home PLMN for ProSe key management. These two AF’s (AF-1 and AF-2) can be located in the same or different PLMN’s and can communicate with each other.

The Remote UE receives the address of the AF in its home PLMN (AF-1(Remote UE)) from the home 5GDNNMF of the Remote UE. The UE-to-network Relay receives the address of the AF in its home PLMN (AF-2(UE-to-network relay)) from the home 5GDNNMF of the UE-to-network relay.

When the Remote UE has discovered a UE-to-network relay in its vicinity, it either sends the address of the AF-1(Remote UE) explicitly on the PC5 interface or includes the address of the AF-1(Remote UE) into the Remote UE info (described in solution #6 in TR 23.752 [xx]) sent on PC5 interface to the UE-to-network relay.

The UE-to-network relay contacts the AF-1(Remote UE) via the AF-2(UE-to-network relay) (as described in Option 1 in Figure 6.Y.2.-1) or the UE-to-network relay contacts the AF-1(Remote UE) directly (as described in Option 2 in Figure 6.Y.2-2).

A third option could be that the UE-to-network relay uses the Remote UE info to query the AF-1(Remote UE) address from its 5GDDNMF in it’s home PLMN. The third option is not described below in the signaling flow.

Editor’s note: How this solution would work when Remote UE is out of 3GPP coverage is FFS.

Figure 6.Y.2-1: AF for key management in PC5 communication

Step 0) The Remote UE contacts the 5GDNNMF in it’s home PLMN to retrieve the address of the AF-1(Remote UE) used for ProSe key management located in its home PLMN.

The UE-to-network Relay contacts the 5GDNNMF in it’s home PLMN to retrieve the address of the AF-2 (UE-to-network Relay) used for ProSe key management located in its home PLMN.

Step 1) The Remote UE retrievs the discovery keys for discovery of the UE-to-network relay.

Editor’s note: its FFS how the Remote UE retrieves the discovery keys in advance for commercial services.

Step 2) The Remote UE establishes a secure connection with the AF-1 (Remote UE) server.

Editor’s note: How to protect the UE - AF-1 (Remote UE) interface is FFS. This is similar to protection of PC3 interface if its via user plane.

Step 3a) The Remote UE contacts the AF-1(Remote UE) by initiating a Key Request message for PC5 communication including the Relay Service Code.

Step 3b) The AF-1(Remote UE) generates the Key for PC5 communication and provides it in the Key Response message together with a Key ID to the Remote UE.

Step 4) The UE-to-network relay retrievs the discovery keys for discovery of the Remote UE.

Editor’s note: its FFS how the UE-to-network relay retrieves the discovery keys in advance for commercial services.

Step 5) The UE-to-network relay discovery is taken place on PC5 interface using either model A or model B discovery.

Step 6) The Remote UE sends a Direct Communication Request on PC5 interface. The Remote UE includes the address of the AF-1(Remote UE) and the Key ID received from the AF-1 (Remote UE) together with the Relay Service Code and Key ID.

When the Remote UE discovers a UE-to-network relay in its vicinity, it can either send the address of the AF-1(Remote UE) explicitly on the PC5 interface or it can include the address of the AF-1(Remote UE) into the Remote UE info (described in solution #6 in TR 23.752 [xx]) sent on PC5 interface, to the UE-to-network relay.

Editor’s note: The privacy of the address of the AF-1(Remote UE) sent over PC5 interface needs to be considered.

A third option could be that the UE-to-network relay uses the Remote UE info to query the AF-1(Remote UE) address from its 5GDDNMF in it’s home PLMN. The third option is not described below.

Option 1:

Step 7) The UE-to-network Relay contacts the AF-2(UE-to-network Relay) and includes the address of the AF-1(Remote UE) in the Key Request message for PC5 communication including the Key ID.

Step 8) The AF-2(UE-to-network Relay) contacts the AF-1(Remote UE) and forwards the Key Request message.

Editor’s note: In Option 1, the communication between AF-1(Remote UE) and AF-2(UE-to-network Relay) is out of 3GPP scope.

Step 9) The AF-1(Remote UE) includes the Key for PC5 communication identified by Key ID in the Key Response message to the AF-2(Remote UE).

Step 10) The AF-2(UE-to-network Relay) forwards the Key Response message to the UE-to-network Relay.

Option 2:

Step 7a) The UE-to-network Relay uses the address of the AF-1(Remote UE) and contacts directly the AF-1(Remote UE) by initiating a Key Request message for PC5 communication including the Key ID.

Editor’s note: In Option2, further details needs to be added to the solution. The Remote UE needs to provide its Remote UE ID to the UE-to-network Relay in order for the UE-to-network Relay to provide its own ID and the Remote UE ID to the AF-1(Remote UE). The AF-1(Remote UE)  needs to authenticate the UE-to-network Relay and check whether the Remote UE and the UE-to-network Relay are allowed to communicate with each other.

Step 7b) The AF-1(Remote UE) includes the Key for PC5 communication identified by Key ID in the Key Response message to the UE-to-network Relay.

Option 1 and Option 2:

Editor’s note: The details of the security establishment (e.g. DSMC) between steps 6-10 are FFS.

Step 11) the UE-to-network Relay responds with a Direct Communication Accept on PC5.

### 6.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

**** END OF CHANGES ****