3GPP TSG-SA3 Meeting #101-e  	draft_S3-203233-r1

- e-meeting, 9 – 20 November 2020													Revision of S3-20xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Solution on multiple working domains

- Document for:	Approval, Information, Discussion

Agenda Item:	5.10

# 1	Decision/action requested

Solution proposal related to Multiple TSN working domains.

# 2	References

[1]	3GPP TR 33.851

# 3	Rationale

3GPP Rel-16, TS 23.501, clause 5.27.1.3, defines support for multiple TSN working domains. Each TSN working domain send its own gPTP messages. The messages are carried in a Multicast Ethernet frame. The gPTP message includes a specific parameter, the domainNumber parameter, to indicate to which domain it belongs. Based on this parameter, the gPTP message is distributed in the network.

If this parameter is manipulated a gPTP message may reach any TSN working domain in the network.

Rel-17 addresses gPTP and PTP, therefore below solution is considering both types of time synchronisation protocols.

# 4	Detailed proposal

******* START OF CHANGES

### 6.X	Solution X: Filtering incoming messages based on authorization policies

This solution addresses authorization aspects to overcome the security threats of KI#2 on security requirements for multiple TSN working domains.

It is assumed that the identities of TSN nodes are verified by DS-TT and NW-TT at the 5GS network boundaries.

Time synchronization messages, received at DS-TT or NW-TT ethernet ports, are addressed to certain TSN working domain, the parameter indicating this is the domainNumber parameter.

The domainNumber parameter in a time synchronization message and the DS-TT/NW-TT ethernet port where it is received, could be used for determining the authorization policy. Such a policy could be used to determine whether the message is dropped or further processed. Authorization can be enforced by filtering incoming messages based on an authorization policy stored co-located with the filter itself.

Three different time synchronization scenarios have to be considered:

- Downlink time synchronization: The time synchronization message is received at the NW-TT and exists the 5GS at the DS-TT. Therefore, a filter may be located at the NW-TT/UPF or at the DS-TT. NW-TT is preferred as it is more trusted than the DS-TT.

- Uplink time synchronization: The time synchronization message is received at the DS-TT and exists the 5GS at the NW-TT. Therefore, a filter may be located at the NW-TT/UPF or at the DS-TT. NW-TT is preferred as it is more trusted than the DS-TT. Filtering at DS-TT has the advantage that the message is filtering before it traverses the 5GS.

- UE-UE time synchronization: The time synchronization message is received at the DS-TT and exists the 5GS at another DS-TT. Therefore, a filter may be located at the DS-TT or at the entitiy where the message is rerouted (currently UPF, later maybe also gNB).

Editor's Note: How the parameter is configured in the UE and UPF is FFS.

******* END OF CHANGES