3GPP TR 33.846 V0.9.0 (2020-11)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on authentication enhancements in 5G System;

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Current status of the primary authentication procedure in the 5G System	8

5	Key issues	8

5.1	Key issues on anchor keys security	9

5.1.X	Key Issue #1.X: <Key Issue Name>	9

5.1.X.1	Key issue details	9

5.1.X.2	Security threats	9

5.1.X.3	Potential security requirements	9

5.2	Key issues on resilience against identifier linkability	9

5.2.1	Key Issue #2.1: Mitigation against the linkability attack	9

5.2.1.1	Issue details	9

5.2.1.2	Security Threats	9

5.2.1.3	Potential security requirements	9

5.2.2	Key Issue #2.2: SUCI replay	10

5.2.2.1	Key issue details	10

5.2.2.2	Potential threats	10

5.2.2.3	 Potential security requirements	11

5.2.X	Key Issue #2.X: <Key Issue Name>	11

5.2.X.1	Key issue details	11

5.2.X.2	Security threats	11

5.2.X.3	Potential security requirements	11

5.3	Key issues on availability aspects of SUCI usage	11

5.3.1	Key Issue #3.1: Key issue on fraudulent attack due to expired authentication result in the UDM	11

5.3.1.1	Key issue details	11

5.3.1.2	Security threats	11

5.3.1.3	Potential security requirements	11

5.3.2	Key Issue #3.2: Key issue to mitigate the SUPI guessing attacks	12

5.3.2.1	Key issue details	12

5.3.2.2	Security threats	12

5.3.2.3	Potential security requirements	12

5.4	Key issues on re-synchronisation in AKA	12

5.4.1	Key Issue #4.1: Protection of SQN during AKA re-synchronisations	12

5.4.1.1	Key issue details	12

5.4.1.2	Security threats	12

5.4.1.3	Potential security requirements	12

6	Solutions	13

6.1	Solutions for anchor keys security	14

6.1.Y	Solution #1.Y: <Solution Name>	14

6.1.Y.1	Introduction	14

6.1.Y.2	Solution details	14

6.1.Y.3	Evaluation	14

6.2	Solutions for resilience against identifier linkability	14

6.2.1	Solution #2.1: Handling of Sync failure by AUTS encryption	14

6.2.1.1	Introduction	14

6.2.1.2	Solution details	15

6.2.1.2.1	Synchronization failure or MAC failure	16

6.2.1.2.1.1	Synchronization failure or MAC failure in USIM	16

6.2.1.2.1.2	Synchronization failure recovery in Home Network	16

6.2.1.3	Evaluation	16

6.2.2	Solution #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF	17

6.2.2.1	Introduction	17

6.2.2.2	Solution details	17

6.2.2.3	Evaluation	18

6.2.3	Solution #2.3: Unified authentication response message by UE	18

6.2.3.1	Introduction	18

6.2.3.2	Solution details	18

6.2.3.3	Evaluation	20

6.2.4	Solution #2.4:  MAC-S based solution	20

6.2.4.1	Introduction	20

6.2.4.2	Solution details	20

6.2.4.3	Evaluation	23

6.2.4.3.1	Active attack	23

6.2.4.3.2	Modification of failure message	23

6.2.4.3.3	Unified failure message	23

6.2.4.3.4	Authentication for resynchronization	23

6.2.5	Solution #2.5: Encryption of authentication failure message with SUCI method	24

6.2.5.1	Introduction	24

6.2.5.2	Solution details	24

6.2.5.3	Evaluation	25

6.2.6	Solution #2.6: Certificate based encryption of unicast NAS messages	26

6.2.6.1	Introduction	26

6.2.6.2	Solution details	26

6.2.6.2.1	Provisioning and certificate distribution	26

6.2.6.2.2	Provisioning Process	26

6.2.6.2.3	Call Flows	26

6.2.6.2.3.1	 Scenario 1 for valid SUPI:	27

6.2.6.2.3.2	 Scenario 2 for invalid SUPI:	27

6.2.6.3	Evaluation	27

6.3	Solutions for availability aspects of SUCI usage	28

6.3.Y	Solution #3.Y: <Solution Name>	28

6.3.Y.1	Introduction	28

6.3.Y.2	Solution details	28

6.3.Y.3	Evaluation	28

6.4	Solutions on re-synchronisation in AKA	28

6.4.1	Solution #4.1: Using MACS as freshness in the calculation of AK	28

6.4.1.1	Introduction	28

6.4.1.2	Solution details	28

6.4.1.3	Evaluation	29

6.4.2	Solution #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA	30

6.4.2.1	Introduction	30

6.4.2.2	Solution details	30

6.4.2.3	Evaluation	30

6.4.3	 Solution #4.3: SQN protection by concealment with SUPI in USIM	31

6.4.3.1	Introduction	31

6.4.3.2	Solution details	31

6.4.3.3	Adaptation of Authentication procedures	33

6.4.3.3.0	General	33

6.4.3.3.1	Initiation of authentication and selection of authentication method	33

6.4.3.3.2	Successful Authentication case:	34

6.4.3.3.3	Authentication failure case:	35

6.4.3.4	Solution summary	36

6.4.3.5	Evaluation	36

6.4.4	 Solution #4.4: SQN protection during re-synchronisation procedure in AKA	37

6.4.4.1	Introduction	37

6.4.4.2	Solution details	37

6.4.4.3	Solution summary	39

6.4.4.4	Evaluation	39

7	Conclusions	39

7.3.1	Conclusion on key issue #3.1	39

Annex <A>: Change history	40

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Editor’s Note: This clause contains some background information for the study.

# 1	Scope

The scope of this document is the following

- key issues, potential security requirements and solutions of how to enhance the authentication process to ensure the security of session anchor keys in case the long-term key is leaked.

- key issues, potential security requirements and solutions of how to mitigate the linkability attacks

- key issues, potential security requirements and solutions of how to mitigate the impacts of potential DDoS threats due to concealing the SUPI.

- key issues, potential security requirements and solution of how to mitigate the leaking of SQN values during AKA re-synchronisation.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.102: "Security architecture".

[4]	Ravishankar Borgaonkar (published online: July 2019), "New Privacy Threat on 3G, 4G, and Upcoming 5G AKA Protocols", https://eprint.iacr.org/2018/1175.pdf

[5]	3GPP TS 29.503: "5G System; Unified Data Management Services".

[6]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[7]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[8]	3GPP TS 23.003: "Numbering, addressing and identification".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	Current status of the primary authentication procedure in the 5G System

Editor’s Note: This clause contains a high-level overview of the primary authentication procedure, the potential envisioned enhancements and their potential impacts on the current Rel-15 security mechanisms.

# 5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

## 5.1	Key issues on anchor keys security

### 5.1.X	Key Issue #1.X: <Key Issue Name>

#### 5.1.X.1	Key issue details

#### 5.1.X.2	Security threats

#### 5.1.X.3	Potential security requirements

## 5.2	Key issues on resilience against identifier linkability

### 5.2.1	Key Issue #2.1: Mitigation against the linkability attack

#### 5.2.1.1	Issue details

In 5G, 5G AKA and EAP AKA’ are subjected to the linkability attack like UMTS AKA because they inherit the error messages (MAC failure, Synch failure) from UMTS AKA. In the linkability attack, the attacker can detect the presence of a victim subscriber, in one of his monitored areas, an active attacker just needs to have previously intercepted one legitimate authentication request message containing the pair (RAND,AUTN) sent by the network to UE. The captured authentication request can now be replayed by the adversary each time he wants to check the presence of UE in a particular area. In fact, thanks to the error messages, the adversary can distinguish any UE from the one the authentication request was originally sent to. On reception of the replayed authentication challenge and authentication vectors (RAND, AUTN), the victim UE successfully verifies the MAC and sends a synchronization failure message. However, the MAC verification fails when executed by any other UE, and as a result a MAC failure message is sent. The implementation of few false base station would then allow an attacker to trace the movements of a victim UE, resulting in a breach of the subscriber’s untraceability.

The attacker could distinguish the MAC failure and synchronization failure using the several methods, such as according to the failure cause embedded in the authentication failure message sent by the victim UE, or according the difference between the length of the response for MAC failure and the length of the response for SYNC failure. Hence, the mechanism used for mitigating the linkability attack shall take all the above methods into consideration.

NOTE 1:	The attacker could also distinguish other interaction differences between the UE and the network to trace the victim UE, such as whether NAS SMC is triggered, Registration Accept/Reject, or the other message length differences. However, these types of linkability attacks are not in focus of this key issue.

NOTE 2: For the linkability attack where the attacker replays legitimate authentication requests, there will no be subsequent NAS messages from AMF.

#### 5.2.1.2	Security Threats

In case the linkability attack occurs, it represents a breach of the user’s untraceability, the attacker can establish the traceability of a subscriber based on the study of the failure messages and can detect subscriber’s presence in a specific area by replaying one old legitimate authentication vector including (RAND and AUTN).

#### 5.2.1.3	Potential security requirements

The 5G system shall support mechanisms to mitigate the Linkability attacks.

### 5.2.2	Key Issue #2.2: SUCI replay

#### 5.2.2.1	Key issue details

This key issue addresses SUCI replay attacks.

A specific SUCI linkability attack and a DoS attack related to SUCI replay are described in the following.

NOTE: KI#2.1 holds the related generic requirement for mitigation of linkability attacks, but this key issue holds specific requirements related to SUCI usage.

Linkability attack

The SUCI can be used for a linkability attack, i.e. an attack by which it is possible to find out whether a UE observed at some location/time X is identical to a UE observed at some location/time Y. For this, an attacker records a SUCI that is used over the radio interface by a UE_A.

As the UE uses a 5G-GUTI rather than a SUCI in most cases, the attacker may also execute an active attack, such as corrupting the 5G-GUTI when it is sent by the UE, which leads with a significant probability to an identity request, so the UE will send a SUCI subsequently in an identity response. An attacker may catch many SUCIs by this method from location/time X.

The attacker can set up a base station at a different location and actively changes the registration request of users registering to this base station. The linkability attack works then like this: If some UE_B makes a registration request to this false base station operated as a relay by the same attacker, the attacker can modify UE_B's registration request message by exchanging the SUCI used in this request by the previously captured SUCI of UE_A or in case of 5G GUTI being used forcing the UE first to respond to an identity request and then modifying the registration request..The modified request is forwarded to the network. Note, this is possible, because UE B's first message is unsecured pre-authentication traffic.

Subsequently, the attacker observes whether a successful AKA run is performed, and the registration request is accepted by the network. If so, then UE_ B needs to be the same as  UE_A. Thus the location and time of a UE moving from one tracking area to another can be linked and may compromise the privacy of the owner of the UE.

Note that this attack cannot by mitigated by hiding only the content of the AKA response, because the attacker can detect from the subsequent messages (e.g. RRC SMC procedure, repetition of the AKA, or RRC release) whether the AKA run was successful or not. If the content of the AKA response is not hidden that could be used directly to determine the UE under attacke is present.

DoS attack

The current ECIES scheme is vulnerable to replay attacks because it does not have the mechanisms in the network side to justify whether the received SUCI was the previous one sent by the UE to the network or not.

If an attacker launches the replay attack multiple times, the UDM and the UE have to spend a lot of resources to process the replayed SUCI and the authentication request message respectively because these messages are legitimate. This raises a DoS attack on the UDM and the UE respectively.

#### 5.2.2.2	Potential threats

Linkability attack: An attacker observes whether a successful AKA run is performed with a replayed SUCI, i.e. if the registration request is accepted by the network. If so, the attacker can link a UE observed in one location with a UE observed in another location. If done at several location, even though UE may still be anonymous, the tracking profile of the same UE being in different location may compromise privacy.

Based on the response to the valid AUTN and RAND from the UE, the attacker could trace the UE with some confidence even though its SUPI is unknown to the attacker.

DoS attack: Replaying the same SUCI many times could lead to a DoS attack on the UDM and the UE respectively. A DoS attack on the UE may result in a decrease in the processing capability of the UE and a rapid consumption of the battery. A DoS attack on UDM will cause the processing power of the UDM to decrease and the response to the request of the legitimate UE to be slow.

#### 5.2.2.3		Potential security requirements

TBD

### 5.2.X	Key Issue #2.X: <Key Issue Name>

#### 5.2.X.1	Key issue details

#### 5.2.X.2	Security threats

#### 5.2.X.3	Potential security requirements

## 5.3	Key issues on availability aspects of SUCI usage

### 5.3.1	Key Issue #3.1: Key issue on fraudulent attack due to expired authentication result in the UDM

#### 5.3.1.1	Key issue details

TS 33.501 [2] clause 6.1.4 specifies that the authentication result will be stored by the UDM after the successful authentication, and can be used to link authentication confirmation to the subsequent procedures, in order to prevent certain types of fraud, e.g. fraudulent Nudm_UECM_Registration Request for registering the subscriber's serving AMF in UDM while the subscriber is not actually present in the visited network.

However, in the case of UE/network-initiated deregistration procedure or SMC failure after AKA in the registration procedure, UDM still has the authentication result, according to TS 29.503 [5]. This may cause security issues. Fraudulent UDM service request like fraudulent Nudm_UECM_ Registration service request sent by the AMF may be accepted by the UDM, , and the UDM may store the AMF ID as the current AMF serving for the UE, even though the UE is already deregistered from the serving network. Therefore, in the deregistration or NAS SMC failure case described above, the authentication result is removed, considering the authentication result usage specified in TS 33.501 [2] clause 6.1.4.

Editor’s Note: Align with the objective of the study needs to be clarified.

Editor’s Note: Key issue details are FFS.

#### 5.3.1.2	Security threats

Editor’s Note: security threats are FFS.

In case of UE-initiated deregistration procedure, or network-initiated deregistration, or SMC failure after AKA during the registration procedure, the fraudulent service request can be initated by an attacker to tamper, or steal the UE data, or subscribe the service from the UDM if the authentication result stored in the UDM is not removed. For example, fraudulent Nudm_UECM_ Registration service request can be sent by the attacker, and accepted by the UDM to store the AMF ID as the current AMF which is serving for the UE. Fraudulent Nudm_SDM_Get service may be used by the attacker to steal the data from the UDM.

#### 5.3.1.3	Potential security requirements

Editor’s Note: Potential security requirements are FFS.

The fradulent attack due to expired authentication result in the UDM shall be mitigated.

### 5.3.2	Key Issue #3.2: Key issue to mitigate the SUPI guessing attacks

#### 5.3.2.1	Key issue details

It is plausible that the attacker knows the network’s public key provisioned in the attacker-controlled SIM card. The attacker could launch a SUPI guessing attack as follows. It generates a guessed SUPI and converts it into SUCI by using network’s public key. Then it sends a fabricated Registration Request message contining the SUCI. Up on receipt of a Registration Request message, the network decrypts SUCI and obtains the SUPI. If SUPI is valid, the network sends an Authenticiation Requst message; otherwise, the network issues a Registration Reject message.  Thus the attacker can ascertain that the guessed SUPI is valid if it receiving Authenticiation Requst message.  After a great number of SUPI guessing attacks, the adversary could figure out the whole database of SUPIs for a dedicated network.

The attacker further could determine whether the valid SUPI belongs to a dedicated victim or not. It forwards the received Authenticiation Requst message to the victim. If the victim replies with the Authentication Failure message, then the valid SUPI does not match the victim. If the victim responds with Authentication Response message, then the victim with the valid SUPI is found.

#### 5.3.2.2	Security threats

The attacker is able to determine whether a SUPI belongs to a given network. The attacker can make an association between the SUPI and a pair of AUTN and RAND. Based on the AUTN and RAND, the attacker could determine whether the victim UE is in the vicinity or not. With this association, an attacker could continuously trace a dedicated victim.

#### 5.3.2.3	Potential security requirements

TBD.

## 5.4	Key issues on re-synchronisation in AKA

### 5.4.1	Key Issue #4.1: Protection of SQN during AKA re-synchronisations

#### 5.4.1.1	Key issue details

The freshness parameter for the calculation of AK (the keystream that protects the SQN returned to the network) during a re-synchronisation in AKA is the random number RAND from the challenge. An attacker can force the re-use of RAND and hence the keystream AK may be used multiple times to protect different SQNs [4]. The attacker could leverage the feedback from the network side and conduct an activity monitoring attack to break subscribers’ privacy. It is preferable to avoid such keystream re-use.

#### 5.4.1.2	Security threats

The re-use of key stream allows an attacker to obtain the XORes value between two different SQNs. It has been shown in [4] how this property in conjunction with issuing fresh challenges can enable an attacker to estimate the least significant bits of SQN. Hence some information about SQN can leak despite the encryption of SQN, which lead to the privacy leakage of the subscribers.

#### 5.4.1.3	Potential security requirements

The protection of SQN during AKA re-synchronisations should prevent the information leakage of SQN values.

# 6	Solutions

Editor’s Note: This clause contains the proposed solutions addressing the identified key issues.

- 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1.X | #2.1 | #2.2 | #3.1 | #3.2 | #4.1 |  |  |
| Solutions for anchor keys security |  |  |  |  |  |  |  |  |
| No solution so far |  |  |  |  |  |  |  |  |
| Solutions for resilience against identifier linkability |  |  |  |  |  |  |  |  |
| #2.1: Handling of Sync failure by AUTS encryption |  | x |  |  |  | x |  |  |
| #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF |  | x |  |  |  | x |  |  |
| #2.3: Unified authentication response message by UE |  | x |  |  |  |  |  |  |
| #2.4: MAC-S based solution |  | x |  |  |  | x |  |  |
| #2.5: Encryption of authentication failure message with SUCI method |  |  | x |  |  |  |  |  |
| #2.6: Certificate based encryption of unicast NAS message |  | x |  |  |  | x |  |  |
| Solutions for availability aspects of SUCI usage |  |  |  |  |  |  |  |  |
| No solution so far |  |  |  |  |  |  |  |  |
| Solutions on re-synchronisation in AKA |  |  |  |  |  |  |  |  |
| #4.1: Using MACS as freshness in the calculation of AK |  |  |  |  |  | x |  |  |
| #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA |  |  |  |  |  | x |  |  |
| #4.3: SQN protection by concealment with SUPI in USIM |  | x | x |  |  | x |  |  |
| #4.4: SQN protection during re-synchronisation procedure in AKA |  |  |  |  |  | x |  |  |

6.1	Solutions for anchor keys security

### 6.1.Y	Solution #1.Y: <Solution Name>

#### 6.1.Y.1	Introduction

Editor’s Note: Each solution should list the key issues being addressed.

#### 6.1.Y.2	Solution details

#### 6.1.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

## 6.2	Solutions for resilience against identifier linkability

### 6.2.1	Solution #2.1: Handling of Sync failure by AUTS encryption

#### 6.2.1.1	Introduction

This solution addresses the key issue #2.1 Mitigation against the linkability attack and key issue #4.1 Protection of SQN during AKA re-synchronisations. The AUSF and UE stores the Kausf during successful authentication. When sync failure occurred, the UE uses the KAUSF stored during previously successful authentication to encrypt the AUTS, and the AUSF uses the KAUSF stored during previously successful authentication to decrypt the AUTS.

Editor’s Note: The security risk of using a fixed key is FFS.

If no stored KAUSF,  the KEY is a 256-bit binary string of all 0s.

#### 6.2.1.2	Solution details

Figure 6.2.1.2: Authentication procedure for 5G AKA

The authentication procedure of 5G AKA in 6.1.3.2.0 of TS 33.501 [2] is followed with the modifications in step 3. step 8 and step 11. Changes are marked in blue.

Step 3.	The AUSF stores the XRES* temporarily together with the received SUCI or SUPI. The AUSF should store the KAUSF.

- Step 8.	The UE returns RES* to the SEAF in a NAS message Authentication Response. The UE should store the KAUSF.

- Step 11.When the AUSF receives as authentication confirmation the Nausf_UEAuthentication_Authenticate Request message including a RES* it may verify whether the AV has expired. If the AV has expired, the AUSF may consider the authentication as unsuccessful from the home network point of view, and in this case the AUSF should remove the KAUSF stored in step 3. AUSF compares the received RES* with the stored XRES*. If the RES* and XRES* are equal, the AUSF considers the authentication as successful from the home network point of view, and in this case the AUSF should remove the old KAUSF stored during previously successful authentication if exists. AUSF informs UDM about the authentication result (see sub-clause 6.1.4 of the present document for linking with the authentication confirmation).

The handling of Sync failure in 6.1.3.3 of TS 33.501  [2] is followed with the modifications. Changes are marked in blue.

##### 6.2.1.2.1	Synchronization failure or MAC failure

###### 6.2.1.2.1.1	Synchronization failure or MAC failure in USIM

This clause describes synchronisation failure or MAC failure in USIM.

In step 7 in Figure 6.1.3.2-1 when 5G AKA is used; or in step 5 in Figure 6.1.3.1-1 when EAP-AKA’ is used, at the receipt of the RAND and AUTN, if the verification of the AUTN fails, then the USIM indicates to the UE the reason for failure and in the case of a synchronisation failure passes the AUTS parameter (see TS 33.102 [9]) to the UE.

If 5G AKA is used: The ME shall respond with NAS message Authentication Failure with a CAUSE value indicating the reason for failure. In case of a synchronisation failure of AUTN (as described in TS 33.102 [9]), the UE shall also include encrypted AUTS that the plaintext AUTS was provided by the USIM. Upon receipt of an authentication failure message, the AMF/SEAF may initiate new authentication towards the UE. (see TS 24.501 [35]).

The AUTS encryption is done by following: The UE generates a keystream by computing HMAC-SHA-256 with a KEY and RAND as input. If the UE has stored a KAUSF, the KEY is the KAUSF, otherwise the KEY is 256-bit string of all 0s. The UE uses a bit per bit binary addition of the AUTS and the keystream.

If EAP-AKA’ is used: The UE shall proceed as described in RFC 4187 [21] and RFC 5448 [12] for EAP-AKA’.

###### 6.2.1.2.1.2	Synchronization failure recovery in Home Network

Upon receiving an authentication failure message with synchronisation failure (encrtyped AUTS) from the UE, the SEAF sends an Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" to the AUSF. The AUSF generates a keystream by computing HMAC-SHA-256 with a KEY and RAND sent to the UE in the preceding Authentication Request as input. If the AUSF has stored a KAUSF that generated during previously successful authentication, the KEY is the KAUSF, otherwise the KEY is 256-bit string of all 0s. The AUSF uses a bit per bit binary addition of the encypted AUTS and the keystream to get the plaintext AUTS. The AUSF then sends an Nudm_UEAuthentication_Get Request message to the UDM/ARPF, together with the following parameters:

-	RAND sent to the UE in the preceding Authentication Request, and

-	Plaintext AUTS.

An SEAF will not react to unsolicited "synchronisation failure indication" messages from the UE.

The SEAF does not send new authentication requests to the UE before having received the response to its Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" from the AUSF (or before it is timed out).

When the UDM/ARPF receives an Nudm_UEAuthentication_Get Request message with a "synchronisation failure indication" it acts as described in TS 33.102 [9], clause 6.3.5 where ARPF is mapped to HE/AuC. The UDM/ARPF sends an Nudm_UEAuthentication_Get Response message with a new authentication vector for either EAP-AKA’ or 5G-AKA depending on the authentication method applicable for the user to the AUSF. The AUSF runs a new authentication procedure with the UE according to clauses 6.1.3.1 or 6.1.3.2 depending on the authentication method applicable for the user.

#### 6.2.1.3	Evaluation

Editor’s Note: Further evaluation is needed.

The protection of SQN will only work over 5G network. A mis-aligned KAUSF will mean that re-synchronisation will not run successfully, e.g. a USIM not storing KAUSF is moved to a new phone.

The solution impacts the SEAF/AMF.

### 6.2.2	Solution #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF

#### 6.2.2.1	Introduction

This solution addresses the key issue#2.1 and key issue #4.1.

#### 6.2.2.2	Solution details

The basic idea of the solution is that the UE sends only one encrypted message to the network to indicate the error message type (MAC_FAIL, SYNC_FAIL) if the UE authentication network fails. This message is protected by using the encryption key KE and the integrity key KM, which are derived from the session anchor key KAUSF that is known to the UE and the network.  Since this message is encrypted, the attacker cannot get the content of this message, and can not initiate a linkability attack. Moreover, the SQN is not disclosed even if the key stream to encrypt the SQN is resued, since the AUTS which is the XORes value between SQN and the key stream is encrypted with the key KE. The procedure of the proposal is illustrated in the following figure.

The steps of the proposal are as follows.

1.	The network authenticates the UE using the selected authentication protocol.

2.	The UE verifies the network. If the verification fails, an Authentication Response message is generated, which includes FAIL_CAUSE, Nonce, SUPI, and RES_DATA. The value of FAIL_CAUSE can be MAC_FAIL, SYNC_FAIL. Nonce is a one-time random number that makes the ciphertext of the Authentication Response message different each time to prevent the attacker from guessing the actual value of FAIL_CAUSE. The SUPI of the UE may be optionally sent to the network in this message to prevent the SEAF from initiating an Identity Request message to the UE. In this way, SEAF could only send an Authentication Request message to the UE regardless of whether it is MAC_FAIL or SYNC_FAIL, thereby avoiding the linkability attack raised by that the SEAF acknowledeges the UE with different actions after receiving the different Authentication Response message specified in reference [3].  If the value of FAIL_CAUSE is SYNC_FAIL and RES_DATA is AUTS, it is generated according to reference [2] for restoring SQN synchronization between UE and network. If the value of FAIL_CAUSE is MAC_FAIL, RES_DATA is a random number whose length is the same as that of AUTS. This Authentication Response message is encrypted by using the ncryption key KE, and its MAC  is generated by using the integrity key KM. The generation of the keys KE and KM is carried out in accordance with the key derivation function KDF of TS 33.220 [7] Appendix B, which is calculated as

KE=KDF (KAUSF, RAND || length of RAND‖"Encryption Key"‖ Length of " Encryption Key"),

KM=KDF (KAUSF, RAND || length of RAND‖"MAC Key"‖Length of "MAC Key" ).

Here  " ‖" represents the string concatenation.

The authentication failure message for the first UE registration is left unencrypted, i.e.  the initial value of KAUSF is set to zero.

3.	The UE sends an Authentication Response message to the SEAF.

4.	The SEAF forwards the Authentication Response message to the AUSF in the message Nausf_UE Authentication_Authenticate request.

5.	Like UE, the AUSF derives the encryption key KE and the integrity key KM from the session achnor key KAUSF. Further the AUSF verifies the MAC using the integrity key KM, and if the verification is successful, decrypts the message using the  encryption key KE.

6.	If FAIL_CAUSE is SYNC_FAIL, the AUSF sends a Nudm_UEAuthentication_Get request message to the UDM with the following parameters: RAND and AUTS. The UDM synchronizes the SQN of the network with the SQN of the UE according to AUTS and RAND.

7.	The AUSF sends FAIL_CAUSE and the SUPI of the UE to the SEAF.

#### 6.2.2.3	Evaluation

Editor’s Note: Further evaluation is needed.

The protection of SQN will only work over 5G network.

- The solution has impact on the serving network when 5G AKA is used.

The solution requires changes to the serving network for 5G AKA.

### 6.2.3	Solution #2.3: Unified authentication response message by UE

#### 6.2.3.1	Introduction

This solution addresses the aspect of “breach of the user’s untraceability” in Key Issue #3.1 “Mitigation against the linkability attack”.

#### 6.2.3.2	Solution details

The authentication response message shall have the uniform format to protect the user privacy.

Figure 6.2.3: Authentication procedure for 5G AKA

When the UE receives the authentication request message from the AMF/SEAF, it shall send unified authentication response message with same format in the following scenarios. The message includes three part: RES*, AUTS, and a cause value. The cause value in these three cases can be the same value.

Cause A: If the verification of the AUTN passes, the UE shall generate the authentication response message containing a RES* calculated according to Annex A.4 in TS 33.501 [2], in which a pseudo AUTS and a cause value are also included. The pseudo AUTS is generated with random numbers.

Case B: If the UE determines the SQN to not be in the correct range, it shall generate the authentication response message containing an AUTS calculated according to Clause 6.3.3 in TS 33.102 [3], in which a pseudo parameter RES* and a cause value are also included. . The pseudo RES* is generated with random numbers.

Case C: If the MAC failure occurs, the UE shall generate the authentication response message containing a cause value, in which a pseudo RES* and AUTS are slao included. The pseudo RES* and AUTS are generated with random numbers.

The above authentication response message with the same format is then sent to the AMF/SEAF.

Once the AMF/SEAF receives the authentication response message, it shall interact with the AUSF according to the verification of authentication response message, which is as specified in the following:

-	If the AMF/SEAF determines the HRES* is correct, it shall send a Nausf_UEAuthentication_Authenticate Request message containing the parameter RES* to the AUSF. The following procedure shall be performed according to Clause 6.1.3.2 in TS 33.501 [2].

-	 Otherwise, the AMF/SEAF shall send a Nausf_UEAuthentication_Authenticate Request message containing the AUTS to the AUSF. The AUSF shall then send a Nudm_UEAuthentication_Get Request to the UDM containing the AUTS. If the verification of AUTS passes, the UDM shall update the SQN and generate a new AV, and then send it back to the AMF/SEAF. If the AMF/SEAF receives this new AV, it shall neglect the cause value in the authentication response message received from the UE and launch the reauthentication procedure; Or, if the AMF/SEAF receives a reponse indicating the failure of AUTS verification from the UDM, it shall send a reject message to UE based on the cause value in the authentication response message.

#### 6.2.3.3	Evaluation

This solution enables the network to mitigate one kind of Linkability attacks which are based on recognising the different authentication responses.

When the UE receives the authentication request message, UE generates authentication response with unified format for successful authentication, MAC failure and sync failure. Successful authentication can be distinguished through verification of RES* in AMF. MAC failure and sync failure can be differentiated through verification of AUTS in UDM. This solution modifies the authentication response message.

The attackers are unable to get the failure type with the authentication response message. This solution impacts the visited network.

### 6.2.4	Solution #2.4:  MAC-S based solution

#### 6.2.4.1	Introduction

This solution addresses the key issues #2.1 and #4.1.

#### 6.2.4.2	Solution details

The procedure works as follow when the USIM detects a MAC failure or a synchronisation failure during the authentication procedure.

2. Authentication request

11. Authentication Response

AV)

12. Nausf_UEAuthentication_Authenticate Request

(synchronisation failure)

5. Nausf_UEAuthentication_Authenticate Request

(synchronisation failure)

1. Authentication request (5G SE AV)

4. Authentication Response

9. Authentication request

UE

Home network

AUSF/ UDM/ARPF

3. The USIM checks AUTN and detects a MAC failure or a synchronization failure

The USIM returns AUTS parameter

AMF/SEAF

10. The USIM verifies the MAC, RAND_Sync and AMF field.

The USIM returns AUTS with SQNMS

Authentication for resynchronization

The USIM returns AUTS parameter

7. The home network generates new authentication vector with RAND_Sync and uses the AMF field to indicate a synchronization procedure

6. The home network checks MAC-S value to determine the type of the failure.

8. Authentication request (5G SE AV)

13. The home network verifies the integrity of MAC-S using AK computed with RAND_Sync and retrieves SQNMS.

Figure 6.2.4.2-1: Mitigation against linkability issue

- 1.	The home network sends an authentication request.

- 2.	The SEAF sends the authentication request to the UE.

- 3.	The USIM verifies the freshness of the 5G authentication vector by checking whether the AUTN can be accepted.

If the USIM detects a MAC failure, then the USIM returns a synchronisation failure message (AUTS) consisting of a random number.

If the USIM detects a synchronisation failure, then the USIM generates a random value RAND_SQN, returns an AUTS containing RAND_SQN instead of the SQNMS and the corresponding MAC-S based on RAND_SQN.

- 4.	The UE sends the authentication response to the SEAF.

- 5.	The SEAF sends to the home network a Nausf_UEAuthentication_Authenticate Request with a synchronization failure indication.

- 6.	The home network checks the MAC-S value send by the USIM.

If the MAC-S verification is successful then the home network knows that the failure corresponds to a synchronization failure. Otherwise, the home network concludes that the failure corresponds to a MAC failure.

The home network can take the decision to create a new authentication vector either to run a new authentication procedure or to run an authentication procedure for resynchronisation in order to retrieve the SQNMS value from the UE.

In case of authentication procedure for resynchronisation

- 7.	The home network decides to perform authentication procedure.

The home network retrieves RAND_SQN from the AUTS previously sent by the USIM, generates a new random RAND_SVR and computes RAND_Sync, where RAND_Sync = RAND_SQN || RAND_SVR

The home network computes an authentication vector built with RAND_Sync, sets AMF field to indicate a resynchronization procedure, and sends the authentication vector to the SEAF.

- 8.	The home network sends an authentication request.

- 9	The SEAF sends the authentication request to the UE.

- 10.	The USIM verifies the freshness of the 5G authentication vector by checking whether the AUTN can be accepted.

If the MAC is valid and if the AMF field indicates a resynchronization procedure, then the USIM verifies that the received RAND_Sync has been generated based on the USIM Random_SQN previously sent. If the verification of the RAND_Sync is successful, then the USIM computes AK = f5K (RAND_Sync), and returns AUTS (SQNMS  AK || MAC-S).

- 11.	The UE sends the Authentication response to the SEAF.

- 12.	The SEAF sends to the home network a Nausf_UEAuthentication_Authenticate Request with a synchronization failure indication.

- 13.	The home network checks the MAC-S value sent by the USIM.

If the MAC-S verification is successful then the home network knows that the failure corresponds to a synchronization failure. Additionnaly, the home network knows that an authentication procedure for resynchronisation procedure is ongoing. Consequently, the home network uses RAND_Sync to compute AK and retrieves SQNMS.

Additionally, in order to improve the security, the following steps can take place:

-	The USIM records all authentication errors (MAC and Sequence Number errors)

-	The USIM informs the user of potential attack when counters reaches a pre-defined threshold

-	Once the connection is established, the USIM reports the attack location and attack scenario.

The solution impacts only the USIM and the home network. The solution requires change of the USIM.

#### 6.2.4.3	Evaluation

##### 6.2.4.3.1	Active attack

The solution protects against linkability attacks based on recognizing MAC / synchronization failures on the radio interface, and protects the procedure for resynchronization thanks to the addition of new parameter RAND_Sync.

##### 6.2.4.3.2	Modification of failure message

The solution proposed that both MAC failure message and Sync failure has same format, only one of them includes the MAC. The attacker could modify any of them to make the MAC verification failed.

-   If the failure message is MAC failure message, it contains a random number instead of the MAC-S in AUTS. Supposing the attacker tamper the content, then after the network verify the MAC-S, it will fail, then the network take this failure message as the the MAC failure.

-   If the failure message is Sync failure message, it contains a MAC-S in AUTS. Supposing the attacker tampers the MAC-S, then after the network verify the MAC-S, it will fail, then the network takes this failure message as the MAC failure, which is incorrect.

This could lead to attack in which all the failure message will be taken as the MAC failure, and the original MAC failure is tampered while the network is not aware, then upon receipt of an authentication failure message, the AMF/SEAF may initiate new authentication towards the UE.

But, this attack is not new, it already exists in AKA.

##### 6.2.4.3.3	Unified failure message

The solution proposes a unified format for MAC failure and synchronisation failure since the key issue on "resilience against identifier linkability" (key issue in clause 5.2) results from the fact that an attacker could distinguish the MAC failure and the synchronization failure sent by the UE. The solution does not propose to have a unified authentication response to cover scenarios where 1- the verification of the AUTN successful, or 2- there is MAC failure, or 3- there is synchronization failure since the integration of the RES in the unified format would have implied substantial changes to the AMF.

##### 6.2.4.3.4	Authentication for resynchronization

The changes to the authentication procedure with addition of RAND_SQN improves the security since the presence of RAND_SQN, generated by the USIM, ensures that the RAND_Sync value (RAND_Sync = RAND_SQN || RAND_SVR) used for the authentication for resynchronization is not a RAND value corresponding to previous successful authentications. Additionally, thanks to the use of the AMF field sent by the home network to indicate a resynchronization in the authentication request of step 8, the USIM knows that the resynchronization is requested by the home network.

The solution impacts the SEAF/AMF.

### 6.2.5	Solution #2.5: Encryption of authentication failure message with SUCI method

#### 6.2.5.1	Introduction

This solution addresses key issue #3.1: Mitigation against the linkability attack.

#### 6.2.5.2	Solution details

Figure 6.2.5-1: Signaling flow of mitigation against the linkability attack

1.	In the authentication procedure, the AMF sends the authentication request message, which contains AUTN and RAND, to the UE.

2.	After receiving the authentication request message, the UE verifies the MAC and the SQN in the AUTN. If the verification fails, the UE encrypts the authentication failure cause value (MAC failure or Synch failure) by using the method of calculating the SUCI，as specified in the following:

-	If the authentication failure cause is MAC failure, the UE constructs a scheme-input including 5GMM Cause (MAC failure) AUTS consisting of a random number.

-	If the authentication failure cause is Synch failure, the UE constructs a scheme-input including 5GMM Cause (Synch failure) and AUTS.

The UE executes the protection scheme (Profile A or Profile B) with the constructed scheme-input as input and take the output as the encrypted cause (indicated by "Enc(Cause)"). The encryption of the authentication failure cause value is calculated in either USIM or ME.

3.	The UE sets the 5GMM Cause in the message to the newly defined "MAC failure or synch failure", to indicate that the authentication failure is caused by MAC failure or Synch failure. The UE sends the newly defined 5GMM Cause and the encrypted authentication failure cause to the AMF within the authentication failure message.

4.	Upon receive of the authentication failure message, the AMF parses the 5GMM Cause IE in the message. If the 5GMM Cause IE is the newly defined "MAC failure or synch failure", the AMF decides to send the encrypted authentication failure cause to the UDM for decryption.

5.	The AMF sends the Enc(Cause) and RAND to the AUSF through Nausf_UEAuthentication_Authenticate Request.

6.	The AUSF sends the Enc(Cause) and RAND to the UDM through Nudm_UEAuthentication_Get Request.

7.	UDM decrypts the Enc(Cause). The UDM decrypts the Enc(Cause) by using the method of decrypting the SUCI, and obtains the specific authentication failure cause. The UDM determines the subsequent procedures (e.g. re-authentication or notifying the AMF of the authentication failure cause) based on the decrypted authentication failure cause.

8a.	If the authentication failure cause is Synch failure, the UDM shall acts as described in TS 33.501 [2], clause 6.1.3.3.2, where a new authentication procedure with UE will be initiated. The Step 9 to Step 10 is omitted.

8b.	If the authentication failure cause is MAC failure, the UDM sends the decrypted authentication failure cause to the AUSF through Nudm_UEAuthentication_Get Response.

9.	The AUSF sends the decrypted authentication failure cause to the AMF through Nausf_UEAuthentication_Authenticate Response.

10.	The AMF may initiate new authentication towards the UE based on the authentication failure cause.

#### 6.2.5.3	Evaluation

In this solution, the UE encrypts the authentication failure cause value (MAC failure or Synch failure) by using the method of calculating the SUCI and sends it to the core network. Only the NF in the core network can decrypt and obtains the specific failure cause. Thus, the attacker cannot distinguish the specific authentication failure cause, thereby preventing the breach of the user’s untraceability and mitigating the linkability attack.

The solution fulfils the potential security requirements from Key Issue #2.1: "The 5G system shall support mechanisms to mitigate the Linkability attacks". The solution only addresses one kind of linkability attacks which are based on recognising SYNC and MAC failures.

The visited network is impacted, and backward compatible.

The UE constructs a scheme-input including 5GMM Cause (MAC failure), the length of which may exceed the current maximum length of MAC failure message, hence the size of the MAC failure message may be impacted.

This solution relies on the availability of SUCI mechanism. If the 5GS or the UE does not support the SUCI mechanism, the solution cannot work.

### 6.2.6	Solution #2.6: Certificate based encryption of unicast NAS messages

#### 6.2.6.1	Introduction

This solution addresses the following key issues:

Key Issue #2.1: Mitigation against the linkability attack

Key Issue #4.1: Protection of SQN during AKA re-synchronisations.

This solution provides certificate-based encryption to unicast NAS messages during authentication.

The core network and UEs can encrypt unicast NAS signaling messages during authentication. The core network and UEs shall accept the messages after the message is successfully decrypted.

#### 6.2.6.2	Solution details

##### 6.2.6.2.1	Provisioning and certificate distribution

This solution requires the network to support PKI (Public Key Infrastructure), which needs the Mobile Network Operator (MNO) to have one or more Certificate Authorities (CAs) as the root of the trust chain. UE shall have the capability to support storing more than one CA Root certificate, which can be stored in USIM or other implementation-dependent way that can provide secure storage.

NOTE: The vendor can provision a list of Root certificates into the UEs, the length of the list depends on the capability of the UE. When the certificate(s) needs to be changed or updated, then the update can be pushed to UEs, like a software update.

- Each UDM is provisioned to have stored certificates for all authorized UEs, and each UE to have the UDM’s certificate.

Editor‘s Note: It is FFS for the associated security requirement(s) for UDMs on storage and processing the keys/certificates.

##### 6.2.6.2.2	Provisioning Process

The core network and UEs use PKI principles to perform mutual authentication.

The following method is used to provision the certificates into the UEs.

UEs are pre-loaded with X.509 certificates with signed public keys for the UDM in which they are allowed to connect. The UDM would likewise be pre-loaded with certificates of eligible UEs.

-	Each UDM has a public and private key pair (UDM_PUB_Key, UDM_PRI_Key).

-	UEs have a corresponding public and private key pair (UE_PUB_Key, UE_PRI_Key).

-	Each core network shares its public key (UDM_PUB_Key) with all UEs with a certificate. The certificate has a subject name that ties with the UDM ID.

-	UEs share their public keys (UE_PUB_Key) with a certificate with the core network.

##### 6.2.6.2.3	Call Flows

The call flow procedures are illustrated below.

###### 6.2.6.2.3.1		Scenario 1 for valid SUPI:

1.	AMF sends the registration request message to the UDM. The UDM retrieves the corresponding subscription information for the UE including the UE_PUB_Key.

2.	For the UDM, if the SUPI is valid, the UDM encrypts the authentication request message using UE_PUB_Key and sends it to the AMF. The AMF sends the encrypted authentication request message to the UE.

3.	After receiving the encrypted authentication request message, the UE uses the UE_PRI_Key to decrypt the message and then verifies the MAC and the SQN in the AUTN.

4a.	If the verification succeeds, the UE encrypts the authentication response message by using UDM_PUB_Key, and then execute step 5a.

4b.	If the verification fails, the UE encrypts the authentication failure message regardless of the 5GMM cause (e.g. MAC failure or Synch failure) by using UDM_PUB_Key, and then execute step 5b. (Note: The authentication failure message has the same size regardless of the 5GMM cause. See Table 8.2.4.1.1: AUTHENTICATION FAILURE message content in TS24.501.)

5a.	The UE sends the encrypted authentication response message to the AMF.

5b.	The UE sends the encrypted authentication failure message with 5GMM cause to the AMF.

6.	Upon receiving of the encrypted authentication response message or authentication failure message, the AMF sends the message to UDM for decryption.

7.	The UDM uses the UDM_PRI_Key to decrypt the authentication response message or the authentication failure message. The UDM shall determine the subsequent procedures as described in TS 33.501 [2].

###### 6.2.6.2.3.2		Scenario 2 for invalid SUPI:

1.	AMF sends the registration request message to the UDM. The UDM retrieves the corresponding subscription information for the UE including the UE_PUB_Key.

2.	For the UDM, if the SUPI is invalid, the UDM encrypts the registration reject message using UE_PUB_Key and sends it to the AMF. The AMF sends the encrypted registration reject message to the UE.

3.	After receiving the encrypted registration reject message, the UE uses the UE_PRI_Key to decrypt the message. The UE determines the subsequent procedures as described in TS 24.501  [6]

Editor’s Note: The authentication procedure includes the AUSF. Sequence flows and explanation showing the AUSF behaviour is needed.

#### 6.2.6.3	Evaluation

In this solution, the UE encrypts NAS messages during authentication by leveraging PKI certificates and public / private key pair. Only the UDM in the core network can decrypt and obtain the contents of these messages. The decryption of the encrypted SUPI for every Registration request adds much load to the UDM and will create more congestion at UDM.

For the linkability attack, the attacker cannot distinguish the specific authentication failure cause in the authentication failure message, thereby preventing the breach of the user’s untraceability and mitigating the linkability attack.

The solution fulfills the security requirements from Key Issue #2.1: "The 5G system shall support mechanisms to mitigate the linkability attacks".

For the attack of SQN during AKA re-synchronisations, the contents of the authentication failure message (with an authentication failure message with synchronisation failure [AUTS]) is encrypted, where the AUTS is a function of the protected SQN (using f5 and key stream [RAND]). As a result, the attacker is not able to exploit the AUTS for breaking the protection of SQN inside.

The solution fulfills the security requirements from Key Issue #4.1: "The 5G system shall support mechanisms to mitigate the attacks of SQN during AKA re-synchronisations".

It is not backward compatible.

There is minimal overhead for the messages due to the nature of public key encryption; however, there is processing cost.

This solution relies on the availability of PKI architecture.

Editor’s Note: It is FFS how provisioning of certificates to all UE and UDM works.

Editor‘s Note: It is FFS for the impact to the visited network.

Editor’s Note: It is FFS to evaluate the complexity of PKI for Key Issue#2.1 and Key Issue #4.1.

Editor’s Note: it is FFS how to securely update the root CA(s) in all the UEs in case that the root CA is common to different operators.

Editor’s Note: The impact on the solution of multiple UDM instances in the PLMN is FFS

## 6.3	Solutions for availability aspects of SUCI usage

### 6.3.Y	Solution #3.Y: <Solution Name>

#### 6.3.Y.1	Introduction

Editor’s Note: Each solution should list the key issues being addressed.

#### 6.3.Y.2	Solution details

#### 6.3.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

## 6.4	Solutions on re-synchronisation in AKA

### 6.4.1	Solution #4.1: Using MACS as freshness in the calculation of AK

#### 6.4.1.1	Introduction

This solution addresses the key issue #4.1.

#### 6.4.1.2	Solution details

This solution adds MACS as an input parameter to the calculation of AK in the case of synchronisation failure for AKA.

NOTE 1: As MACS is calculated using SQNMS, this ensures that a fresh input is used for the calculation of AK in a re-synchronistaion.

NOTE 2: The use of MACS would be up to the operator as it is used in their HLR/HSS/ARPF and UISM.

The calculation of the AUTS parameter for re-synchronistaion is described in clauses 6.3.3 and 6.3.5 of TS 33.102 [3]. In terms of the modifying that clause, this solution would require replacing the concealed value of the counter SQNMS, i.e. Conc(SQNMS), as follows;

Conc(SQNMS) = SQNMS   f5*K(RAND, [MAC-S])

and also modify the figure in clause 6.3.3 to the figure below.

Figure 6.4.1.2-1: Modified construction of the parameter AUTS

For re-synchronisation procedure, when the HE/AuC receives an authentication data request with a "synchronisation failure indication" and AUTS, the detailed procedure is defined as follows:

- MAC-S and SQNMS AK are retrieved from AUTS.

- AK is calculated using f5 function with MAC-S as one input.

- SQNMS is retrieved by computing SQNMS AK AK.

- MAC-S is verified with MAC-S’, where MAC-S’ = f1*K(SQNMS || RAND || AMF).

- It means MAC-S is one of the inputs to retrieve SQNMS. SQNMS is one of the inputs to verify MAC-S.

- The home network knows whether to use MAC-S to calculate AK based on the subscription identifier, e.g. IMSI or SUPI.

#### 6.4.1.3	Evaluation

The solution address the key issue by providing a fresh input to the calculation of AK to ensure that the same AK is not used to protected two different SQNs.

This solution requires changing the USIM and the HSS/HLR/UDM. The change is essentially updating these to support a new authentication and key agreement algorithm, although in this case the change could be a small change to an existing algorithm.

The solution does not affect any entity between the USIM and home network.

The solution is a deployment option for the home operator.

Deploying this protects the use of AKA for all usage scenarios, e.g. it is not specific to 5G.

### 6.4.2	Solution #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA

#### 6.4.2.1	Introduction

This solution addresses the key issue #5.4.1.

#### 6.4.2.2	Solution details

This solution proposes to use symmetric encryption function to protect SQN with input key of AK in the case of synchronisation failure for AKA.

The calculation of the AUTS parameter for re-synchronistaion is described in clauses 6.3.3 and 6.3.5 of TS 33.102 [3]. In terms of the modifying that clause, this solution would require adding a new function f*6 and replacing the concealed value of the counter SQNMS, i.e. Conc(SQNMS), as follows;

Conc(SQNMS) = f6*AK(SQNMS )

f6* is the symmetric encryption function used to encrypt SQNMS using AK as input key. f6*AK(SQNMS ) is the encrypted value of the counter SQNMS in the MS. The figure in clause 6.3.3 of [3] is modified to the figure below.

Figure 6.4.2.2-1: Modified construction of the parameter AUTS

- To decrease the complexity, the f5* function can be replaced by XOR.

#### 6.4.2.3	Evaluation

Solution requires a major change of authentication algorithms, and the examples defined in 3GPP (Milenage and TUAK).

Solution requires change of USIM

### 6.4.3		Solution #4.3: SQN protection by concealment with SUPI in USIM

#### 6.4.3.1	Introduction

This solution addresses the key issue #4.1 Protection of SQN during AKA re-synchronisations, the linkability attack in key issue #2.2, and key issue #3.2 to mitigate the SUPI guessing attacks.

#### 6.4.3.2	Solution details

Current usage of ECIES for concealment of SUPI can be expanded to accommodate SQNMS and SUPI. Maximum allowed size of cipher text from concealment of protection scheme output is 3000 digits. SUPI utilizes only few bytes of those maximum allowed digits and still can adapt SQNMS.

Figure 6.4.3.2-1 Encryption based on ECIES at UE

Figure 6.4.3.2-1 shows the encryption based on ECIES at UE side, where SUPI is concatenated with SQNMS and taken as one plain text block for symmetric encryption. In case of SUPI type as IMSI, then MSIN (9 to 10 digits) and SQNMS (48 bits: 6 bytes) is concatenated in UE.

Figure 6.4.3.2-2: Decryption based on ECIES at home network

Figure 6.4.3.2-2 shows the decryption based on ECIES at home network, where SUPI and SQNMS is dissociated after the symmetric decryption.

Figure 6.4.3.2-3: Structure of SUCI

Figure 6.4.3.2-3 shows the structure of SUCI with SUPI Type, which consists values in the range 0 to 7 as specified in clause 2.2B of TS 23.003 [8]. SUPI Type identifies the type of the SUPI concealed in the SUCI.

For this solution, the encoding of SUCI for ‘SUPI plus SQNMS' will be represented by a new SUPI Type value, e.g. value 4.

-	0: IMSI

-	1: Network Specific Identifier

-	2: Global Line Identifier (GLI)

-	3: Global Cable Identifier (GCI)

-	4: SUPI plus SQNMS

-    5 to 7: spare values for future use.

#### 6.4.3.3	Adaptation of Authentication procedures

##### 6.4.3.3.0	General

The following sequence charts illustrates the text already specified in 3GPP TS 33.501[2], clause 6.1.2 for the case of initiation of authentication and selection of authentication method including the additional changes needed for this solution. Changes are marked in bold.

##### 6.4.3.3.1	Initiation of authentication and selection of authentication method

2.  <N1 message>

(SUCI containing SQNMS )

UE

3. Nausf_UEAuthentication_ Authenticate Request

(SUCI containing SQNMS or SUPI, SN-Name)

4. Nudm_UEAuthentication_ Get Request (SUCI containing SQNMS or SUPI, SN-Name)

UDM/ARPF

AMF/

SEAF

AUSF

1. Conceal SUPI and SQNMS

5. [SUCI to SUPI, SQNMS de-concealment].

Authentication Method selection.

Figure 6.4.3.3.1-1: Illustration of UE and HE is sharing SQNMS along with SUCI

- During the primary authentication procedure, USIM concatenates SUPI and SQNMS. The concatenated plain text block is encrypted using ECIES method. New value is introduced for “SUPI Type”, for e.g. Value 4 represents SUCI encoded with SUPI plus SQNMS.

- UE uses SUCI containing SQNMS in Registration request message, which is sent to AMF/SEAF.

- AMF/SEAF invokes the Nausf_UEAuthentication service by sending a Nausf_UEAuthentication_Authenticate Request message to the AUSF whenever the AMF/SEAF wishes to initiate an authentication.

The Nausf_UEAuthentication_Authenticate Request message contains either:

-	SUCI containing SQNMS, as defined in the current specification, or

-	SUPI, as defined in 3GPP TS 23.501 [6].

The AMF/SEAF includes the SUPI in the Nausf_UEAuthentication_Authenticate Request message in case the AMF/SEAF has a valid 5G-GUTI and re-authenticates the UE. Otherwise the SUCI containing SQNMS is included in Nausf_UEAuthentication_Authenticate Request. SUPI/SUCI structure is part of stage 3 protocol design.

The Nausf_UEAuthentication_Authenticate Request shall furthermore contains:

-	the serving network name, as defined in sub-clause 6.1.1.4 of 3GPP TS 33.501 [2] the present document.

- Upon receiving the Nausf_UEAuthentication_Authenticate Request message, the AUSF checks that the requesting AMF/SEAF in the serving network is entitled to use the serving network name in the Nausf_UEAuthentication_Authenticate Request by comparing the serving network name with the expected serving network name. The AUSF stores the received serving network name temporarily. If the serving network is not authorized to use the serving network name, the AUSF responds with "serving network not authorized" in the Nausf_UEAuthentication_Authenticate Response.

The Nudm_UEAuthentication_Get Request sent from AUSF to UDM includes the following information:

-	SUCI containing SQNMS or SUPI;

-	the serving network name;

- Upon reception of the Nudm_UEAuthentication_Get Request, the UDM invokes SIDF if a SUPI type is SUPI plus SQNMS, then SIDF de-conceals SUCI to gain SUPI and SQNMS before UDM can process the request.

- Based on SUPI, the UDM/ARPF chooses the authentication method. SQNMS is temporarily stored in UDM for future use. At UDM, Authentication vector is generated with existing SQNHE..

The solution is restricted to 5G.

NOTE 1:	The Nudm_UEAuthentication_Get Response in reply to the Nudm_UEAuthentication_Get Request and the Nausf_UEAuthentication_Authenticate Response message in reply to the Nausf_UEAuthentication_Authenticate Request message are described as part of the authentication procedures in 3GPP TS 33.501[2], clause 6.1.3.

NOTE 2: SQNMS is not considered for Authentication vector generation on purpose.

##### 6.4.3.3.2	Successful Authentication case

In success case the Authentication procedure (considering example of 5G AKA) is the same as 3GPP TS.33.501[2], clause 6.1.3.2.0. The solution does not impact the existing call flow. . In case of successful AKA, success means SQNms and SQNHE are already aligned hence temporarily stored SQNms is deleted.

UE

3. Nudm_UEAuthentication_ ResultConfirmation Response

UDM

AMF/SEAF

AUSF

0. UE Authentication using an EAP method or 5G-AKA (e.g. as in sections 6.1.3)

1. Nudm_UEAuthentication_ ResultConfirmation Request

2. UDM stores authentication status of UE and delete unused SQNms (if available)

4. UDM authorizes subsequent procedures (e.g. from AMF as defined in 6.1.4.2) based on UE authentication status stored in step 2).

Figure 6.4.3.3-1: Authentication procedure for 5G AKA (success case)

##### 6.4.3.3.3	Authentication failure case

Figure 6.4.3.3.3-1: Authentication procedure for 5G AKA (Failure case)

Figure 6.4.3.3.3-1 shows the failure case of Authentication procedure (considering example of 5G AKA). Changes are shown in bold.

1.	For each Nudm_Authenticate_Get Request, the UDM/ARPF creates a 5G HE AV. The UDM/ARPF does this by generating an AV with the Authentication Management Field (AMF) separation bit set to "1" as defined in TS 33.102 [3]. The UDM/ARPF then derives KAUSF (as per Annex A.2) and calculate XRES* (as per Annex A.4). Finally, the UDM/ARPF creates a 5G HE AV from RAND, AUTN, XRES*, and KAUSF.

2.	The UDM then returns the 5G HE AV to the AUSF together with an indication that the 5G HE AV is to be used for 5G-AKA in a Nudm_UEAuthentication_Get Response. In case SUCI was included in the Nudm_UEAuthentication_Get Request, UDM will include the SUPI in the Nudm_UEAuthentication_Get Response.

3.	The AUSF stores the XRES* temporarily together with the received SUCI or SUPI.

4.	The AUSF then generates the 5G AV from the 5G HE AV received from the UDM/ARPF by computing the HXRES* from XRES* (according to 3GPP TS 33.501 [2] Annex A.5) and KSEAF from KAUSF(according to 3GPP TS 33.501 [2]Annex A.6), and replacing the XRES* with the HXRES* and KAUSF with KSEAF in the 5G HE AV.

5.	The AUSF then removes the KSEAF return the 5G SE AV (RAND, AUTN, HXRES*) to the SEAF in a Nausf_UEAuthentication_Authenticate Response.

6.	The SEAF sends RAND, AUTN to the UE in a NAS message Authentication -Request. This message also includes the ngKSI that will be used by the UE and AMF to identify the KAMF and the partial native security context that is created if the authentication is successful. This message also includes the ABBA parameter. The SEAF sets the ABBA paremeter as defined in Annex A.7.1. The ME forwards the RAND and AUTN received in NAS message Authentication Request to the USIM.

7. At receipt of the RAND and AUTN, the USIM verifies the freshness of the 5G AV by checking whether AUTN can be accepted as described in 3GPP TS 33.102[3]. If the verification of the AUTN fails, then the USIM indicates to the ME the reason for failure.

8. The ME responds with NAS message Authentication Failure only with a CAUSE value indicating the reason for failure (as SQN failure/mismatch). AUTS is not calculated by the UE and not shared to network.

9. Upon receiving an authentication failure message from the UE, the SEAF sends an Nausf_UEAuthentication_Authenticate Request message to the AUSF.

10. AUSF sends an Nudm_UEAuthentication_Get Request message to the UDM/ARPF.

11. When the UDM/ARPF receives an Nudm_UEAuthentication_Get Request message it acts as described in 3GPP TS 33.102 [3], clause 6.3.5 where ARPF is mapped to HE/AuC. The UDM/ARPF sends an Nudm_UEAuthentication_Get Response message with a new authentication vector by considering the SQNMS from database (i.e. SQNMS received in Nudm_UEAuthentication_ Get Request). UDM synchronizes its SQNHE value, i.e SQNHE = SNQMS.The AUSF runs a new authentication procedure with the UE according to clauses 6.1.3.1 or 6.1.3.2 of 3GPP TS 33.501 [2] depending on the authentication method applicable for the user.

#### 6.4.3.4	Solution summary

The solution adds a new SUPI type in 3GPP TS 23.003 [8], clause 2.2B.

Encryption and decryption on ECIES consider plain text block as "SUPI and SQNms" at USIM and UDM / ARPF in 3GP TS 33.501 [2].

Registration request is sent with SUPI plus SQNMS.

Sync failure during AKA challenge sends only failure cause and no AUTS is sent to the network.

Editor’s Note: It is ffs if this solution works for synchronization failure when AMF receives 5G-GUTI and decides to initiate authentication.

#### 6.4.3.5	Evaluation

The solution protects the sequence number in 5GS and ensures the freshness of the SUCI generated as current as the last successful primary authentication.

The solution requires changes on the USIM and the UDM. Since HPLMN owns the subscription, both USIM changes and UDM changes can be synchronised by the HPLMN operator at the same time.

The solution does not work if the SUCI calculation is performed by the ME

At USIM, during ECIES procedure of primary authentication, New SUPI type is added, SQNMS is concatenated with SUPI (plain text block).

At UDM, when the SUPI indicates ‘SUPI plus SQNMS' the de-concealment needs to disassociate SUPI and SQNMS. UDM stores SQNMS temporarily until the success or failure of the authentication is known.

There is no change to any entities if the authentication succeeds.

If there is authentication failure at the UE, UE sends only Authentication failure message to the HE with SQN failure cause code (new value), without AUTS.

At the UDM, if an authentication failure message with cause code (SQN failure) is received, the temporarily stored value of SQNMS received at the very first step is processed. UDM synchronizes its value of SQN, i.e SQNHE = SQNMS. Sequence number management profiles detailed in Annex C in 3GPP TS 33.102 [3] are kept intact.

### 6.4.4		Solution #4.4: SQN protection during re-synchronisation procedure in AKA

#### 6.4.4.1	Introduction

This solution addresses the key issue #4.1.

The following sequence chart illustrates what is already specified in TS 33.501 [2] clause 6.1.3.3 for the case of authentication request that results in a synchronization failure. Additions are in step 5 and step 9. The description is applicable when 5G AKA is used. However, the proposed mechanism of setting AUTS to all zeros is not limited to the use of 5G AKA but is also applicable if EAP-AKA' is used. Further, the solution would be also applicable to older generations than 5G.

#### 6.4.4.2	Solution details

3. Nausf_UEAuthentication_ Authenticate Response

(5G SE AV)

4. Authentication Request                     (RAND,AUTN)

UE

2. Nudm_Authentication_Get Response (5G HE AV, [SUPI])

7. Nausf_UEAuthentication_ Authenticate Request

(AUTS)

6. Authentication Failure (AUTS)

UDM/ARPF

5. Detect synchronization failure; check for RAND repetition; compute AUTS

AMF/SEAF

AUSF

1. Generate AV

8. Nudm_UEAuthentication_ Get Request

(AUTS, RAND)

9.Re-synchronisation Handling

Figure 6.4.4.2-1: Illustration of UE and HE synchronization failure detection handling

1.	Authentication vector is generated during authentication procedure in UDM/ARPF.

2.	Nudm_UEAuthentication_Get Request carrying the AUTN, RAND value in 5G HE AV from UDM/ARPF to AUSF.

3.	Nausf_UEAuthentication_Authenticate Response carries the AUTN, RAND value in 5G SE AV from AUSF to AMF/SEAF.

4.	AMF/SEAF sends AUTN, RAND value to UE in NAS Authentication Request message.

5.	After the receipt of Authentication Request message, when there is no synchronization failure, the UE computes an answer as described in TS 33.501 [2]. If, however the USIM detects a synchronization failure, it checks whether the current RAND value is stored on the USIM as a received RAND value. If this is not the case, it stores the RAND value in the USIM as a received RAND value and computes AUTS as specified in TS 33.102 [3]. Otherwise, it sets AUTS to all zeros.

NOTE: According to this, only RAND values of challenges that result in a synchronization failure are stored on the USIM. The number of RAND values that the USIM stores is entirely up to the operator and the choice of the value affects only the USIM.  The USIM can discard the oldest stored RAND value in case there is otherwise no more room to store a new RAND value.

6.	UE includes AUTS provided by USIM in Authentication failure message to AMF/SEAF.

7. Upon receiving an authentication failure message with AUTS from the UE, the AMF/SEAF sends an Nausf_UEAuthentication_Authenticate Request message with a "synchronisation failure indication" to the AUSF.

8.	AUSF sends a Nudm_UEAuthentication_Get Request message to the UDM/ARPF with RAND (sent to UE in preceding Authentication Request) and AUTS (received from AMF/SEAF).

9.	UDM/ARPF receives the AUTS. If AUTS is all zeros, UDM/ARPF proceeds by sending a Nudm_UEAuthentication_Get Response message with a new authentication vector with a new RAND value. Otherwise, UDM/ARPF proceeds as specified in TS 33.501 [2] in case of a synchronisation failure.

#### 6.4.4.3	Solution summary

The solution would need to introduce in TS 33.102 [3], section 6.3.5, a new step 0 which checks for AUTS. If AUTS received is all zeros, then HE/AuC continues with step (6), otherwise it continues with step (1).

Note, this solution cannot fully avoid the discovery of SQNms values, but it provides a means for limiting the probability of SQNms leakage by recommending a high RAND storage in USIM.

#### 6.4.4.4	Evaluation

The proposed solution has impact on the USIM and UDM.

The solution prevents the attacker from retrieving any SEQMS information from UE in a way that the existing AK (Anonymity Key) used in AUTS is enough to protect SEQMS without the need for changes of the protocol messages or the cryptographic operations.

By setting AUTS to all zeros in case of RAND-reusage, a sniffer or active attacker on the radio interface does not get any information from the synchronization failure, except the fact that there was a RAND repetition. In case the RAND repetition is due to a false base station replaying an authentication request, which the network has previously sent to a UE, to this same UE, the attacker is anyway aware that this is a RAND repetition. In regular traffic, on the other hand, it is highly unlikely that two authentication requests that cause a synchronization failure will have the same RAND. If this still happens, and the network receives an all zeros AUTS, the network gets no proof that this AUTS really comes from the UE, and the network will not be able to synchronize the SEQHE with the SEQMS. In this case it is still clear for the network that something went wrong with the authentication procedure, so the authentication procedure needs to be repeated. The network repeats the authentication request until it creates a RAND that has not been used to compute an AUTS before.

A potential attack on recording RAND values could estimate the number of RAND values stored on the USIM by trial-and-error. Once an attacker has filled the RAND storage and can repeat previously sent RANDs, the solution no longer provides significantly more effective protection of the SQN than a USIM without such RAND storage. The difficulty of the attack can be increased by configuring the USIM with high RAND storage.

Editor's Note: The limitation of RAND storage in USIM is FFS.

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.3.1	Conclusion on key issue #3.1

No normative work is needed for the key issue #3.1.

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-03 | SA3#94-Ad hoc | S3-190809 |  |  |  | TR skeleton | 0.0.0 |
| 2019-03 | SA3#94-Ad hoc | S3-190908 |  |  |  | Version after incorporating changes in S3-190810 and S3-190909. | 0.1.0 |
| 2019-06 | SA3#95-Bis | S3-192432 |  |  |  | Version after incorporating changes in S3-192431 and S3-192433. | 0.2.0 |
| 2019-08 | SA3#96 | S3-193188 |  |  |  | Version after incorporating changes in S3-192920, S3-193187, S3-193189, S3-193190, S3-193191. | 0.3.0 |
| 2019-10 | SA3#96-Ad hoc | S3-193850 |  |  |  | Version after incorporating changes in S3-193810, S3-193811, S3-193812, S3-193813, S3-193814, S3-193815. | 0.4.0 |
| 2019-11 | SA3#97 | S3-194676 |  |  |  | Version after incorporating changes in S3-194071, S3-194673, S3-194675, S3-194677, S3-194678, S3-194679, S3-194681. | 0.5.0 |
| 2020-05 | SA3#99-e | S3-201460 |  |  |  | Version after incorporating changes in S3-200941, S3-201213, S3-201474. | 0.6.0 |
| 2020-08 | SA3#100-e | S3-202072 |  |  |  | Version after incorporating changes in S3-201929, S3-201934, S3-202020, S3-202070, S3-202076, S3-202094, S3-202098, S3-202099, S3-202100. | 0.7.0 |
| 2020-10 | SA3#100bis-e | S3-202735 |  |  |  | Version after incorporating changes in S3-202442, S3-202443, S3-202495, S3-202643, S3-202710, S3-202711, S3-202712, S3-202747, S3-202786, S3-202788. | 0.8.0 |
| 2020-11 | SA3#101-e | S3-203388 |  |  |  | Version after incorporating changes in S3-202812, S3-202917, S3-203080, S3-203082, S3-203218, S3-203327, S3-203328, S3-203329, S3-203372, S3-203376, S3-203393, S3-203394. | 0.9.0 |