3GPP TSG-SA3 Meeting #101-e 	S3-203269-r1

- e-meeting, , 9 - 20 November 2020	Revision ofS3-202820

- Source:	InterDigital

- Title:	TR 33.847 Update for solution #10

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

It is proposed to approve this pCR in TR 33.847.

# 2	References

# 3	Rationale

The contribution proposes to address the following ENs:

Editor's note: Details for how a Remote UE that is already authenticated and authorized reconnects for relay communication are FFS.

Editor’s Note: Whether and how the AMF serving the Relay UE always be able to serve the remote UE is FFS.

Editor's note: How the AMF and Relay UE determines whether the authentication messages are for to the Remote UE is FFS.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.847.

***	BEGIN OF CHANGES	***

## 6.10	Solution #10: Authorization and security with UE-to-Network relay using Remote UE network primary authentication

### 6.10.1	Solution overview

The contribution proposes a solution to address the following Key Issues:

- KI #3: Security of UE-to-Network Relay

- KI #4: Authorization in the UE-to-Network relay scenario

- KI #9: Key management in 5G Proximity Services for UE-to-Network relay communication

This solution builds on top of common high-level principles from existing solution #46 and solution #47 specified in TR 23.752 [2]. These solutions address Remote UE and UE-to-Network authorization aspects in the case of L3 relay. This solution leverages a Remote UE primary authentication run to establish keys used to secure the PC5 link between the Remote UE and the UE-to-Network relay.

This solution assumes that the Remote UE selects a relay based on the connectivity service (e.g., S-NSSAI, DNN) that the relay can provide. The Remote UE learns about the connectivity service as part of the discovery procedure. It is assumed that the relay's Allowed NSSAI includes the S-NSSAIs needed to provide the connectivity service. The UE-to-Network relay either has a PDU session or is able to establish a new one without having to request a S-NSSAI (same assumption as solution #6 in TR 23.752 [2]). Therefore, in the context of the connectivity service provided by the relay, the AMF serving the relay is always able to serve the Remote UE.

### 6.10.2	Solution details

The procedure for Authorization and security with UE-to-Network relay using Remote UE network primary authentication is depicted in Figure 6.10.2-1.

Figure 6.10.2-1: Procedure for Authorization and security with UE-to-Network relay using Remote UE network primary authentication

0. The Relay UE is registered and authorized to operate as a UE-to-Network relay.

1. The Remote UE sends a Direct Communication request message to the Relay UE. The Remote UE includes its SUCI in the message to request UE-to-Network relay service. The Remote UE also provides its security capabilities and security policy as in TS 33.536[8]. If the Remote UE is reconnecting to the same Relay UE and is already authenticated and authorized for relay communication via that Relay UE, it includes the Krelay ID (instead of SUCI) established during the previous connection using this procedure. If the Relay UE has the Krelay and Krelay ID it skips steps 2 to 12, otherwise the Relay UE rejects the connection request.

NOTE 1: Krelay and Krelay ID are reused during reconnection the same way as KRNP and KRNP ID in TS 33.536[8].

Editor's note: Whether and how the Remote UE may use its 5G native security context to connect via the Relay UE is FFS.

2. The Relay UE sends a NAS Relay Authorization request message to its serving AMF. The Relay UE includes the Remote UE's SUCI in the message.

3. The Relay UE's AMF checks that the Relay UE is authorized to act as a Relay based on subscription information obtained during Relay UE's registration

4-8. The Relay UE's AMF initiates Remote UE authentication with Remote UE's AUSF according to existing primary authentication procedures. The authentication messages are exchanged transparently via the Relay UE. Authentication messages between AMF and AUSF and AMF and Relay UE include an indication that it is for a relayed authentication i.e. to authenticate Remote UE via Relay UE.

Editor's note: Whether UDM is made aware that the Remote UE is being authenticated via a Relay UE is FFS.

9. Upon successful authentication of the network, the Remote UE derives a PC5 link root key Krelay and its Krelay ID from KAMF

NOTE 2: Krelay and its Krelay ID can be considered as equivalent to KRNP and KRNP ID in TS 33.536[8].

Editor's note: Details for how Krelay and Krelay ID are derived are FFS

10. Upon successful authentication of the Remote UE, Relay UE's AMF checks with Remote UE's UDM that Remote UE is authorized to use UE-to-Network relaying. Upon successful authorization check, Relay UE's AMF registers with Remote UE's UDM as its Relay's AMF, providing the Relay UE identity (SUPI or GPSI).

11. Relay UE's AMF derives a PC5 link root key Krelay and its Krelay ID from KAMF as performed by Remote UE in step 9.

12. Relay UE's AMF sends a NAS Relay Authorization response message to the Relay UE. The Relay UE's AMF includes the PC5 link root key Krelay and its Krelay ID in the message. The Relay UE stores the key and its id and associates them with the PC5 link with Remote UE.

13. The Relay UE initiates PC5 link security establishment with Remote UE based on PC5 link root key Krelay. The Relay UE derives PC5 session key Krelay-sess from Krelay, and confidentiality and integrity keys from Krelay-sess the same way KNRP-sess is derived from KNRP, and confidentiality and integrity keys from KNRP-sess in TS 33.536[8]. The Relay UE integrity protects the Direct Security Mode Command and includes parameters as in TS 33.536[8]. The Relay UE includes the Krelay ID to indicate that the PC5 security establishment shall be based on Remote UE's primary authentication run.

Editor's note: Details for key hierarchy used for PC5 link security above is FFS

14. The Remote UE checks that the received Krelay ID matches the one derived in step 9. If the provided key id matches, then the Remote UE proceeds with PC5 session, confidentiality, and integrity keys derivation using Krelay as the PC5 link root key as performed by the Relay UE. The Remote UE performs security checks of the Direct Security Mode Command message as in TS 33.536[8].

15. The Remote UE sends integrity and confidentiality protected Direct Security Mode Complete message to Relay UE as in TS 33.536[8].

16. Procedure continues as per L3 relay setup procedure as defined in TR 23.752 [2] (e.g., in step 3 in solution#6).

Editor's note: PC5 link security handling during changes of Relay UE's AMF and/or Remote UE re-authentication is FFS

Editor's note: Authorization revocation of Remote UE to use Relay is FFS

### 6.10.3	Solution evaluation

***	END OF CHANGES	***