- 3GPP TSG SA WG3 Meeting #102bis-e                            	S3-210906

- e-meeting, 1 – 5 March 2021	                         revision S3-21abcd

- Source:	Huawei, HiSilicon

- Title:	New Solution for Security for Provisioning of SNPN Credentials without Initial Credential

- Document for:	Approval

Agenda Item:	2.12

# Decision/action requested

This contribution proposes to add a new solution in TR 33.857

# References

# Rational

This contribution adds a new solution for key issue #2 in TR 33.857.

# Detailed proposal

*************** Start of 1st Change ****************

- 2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

[3]	3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

[4]		3GPP TS 23.501: "System Architecture for the 5G System"

[5]	IETF RFC 5281: "Extensible Authentication Protocol Tunneled Transport Layer Security             	Authenticated Protocol Version 0 (EAP-TTLSv0)"

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[yy]	3GPP TS 33.535: “Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)”

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

## 6.X	Solution #X: Security for Provisioning of SNPN Credentials without Initial Credential

### 6.X.1	Introduction

The solution addresses key issue #2, especially for provisioning SNPN credentials in case that the PS does not preconfigure initial credential to authenticate the UE.

### 6.X.2	Solution details

The assumptions include:

- PS does not preconfigure initial credential to authenticate the UE.

- PS belongs to SO, so network entities in SO and PS are in the same trust domain. SO has been preconfigured with PS identity

- O-SNPN and SO-SNPN have been involved in primary authentication based on default credential, e.g. as depicted in solution 12.

The detailed procedure is:

After successful primary authentication based on default credential between UE, O-SNPN and SO-SNPN, UE and O-SNPN have maintained NAS key, UE and SO-SNPN have maintained KAUSF.

The UE requests to establish a restricted PDU session, and the PCF in SO-SNPN provides PS identity in PDU Session Establishment Response message to UE. The UE can only access the PS in the restricted PDU session. This procedure is protected based on NAS key.

The UE requests the PS based on PS identity using AKMA as depicted in TS 33.535 [yy], i.e. the UE sends A-KID to the PS, and the PS requests AAnF in SO-SNPN for KAF based on KAUSF. The UE and the PS uses KAF to establish a secure tunnel to protect the following provisioning. The PS may notify the UDM of successful provisioning via NEF protected as depicted in clause 12 in TS 33.501 [2], after that, the UDM updates UE’s subscription. Then, subscription on both UE and UDM side has been synchronized.

Editor’s Note: Details and figures clarifying the steps are needed.

Editor’s Note: Security implications/constrains/assumptions of deploying AMKA in this context is FFS.

### 6.X.3	System Impact

UE, SMF, NEF, UDM and PS are impacted.

### 6.X.4	Solution evaluation

TBA

*************** End of 2nd Change ****************