3GPP TSG-SA3 Meeting #102Bis  	draft_S3-211042-r1

- e-meeting, 1 – 5 March													Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Solution on Detection of anomalous NF behaviour by NWDAF

- Document for:	Approval, Information, Discussion

Agenda Item:	2.16

# 1	Decision/action requested

Introduction of a solution related to key issue on anomalous NF behaviour detection by NWDAF

# 2	References

[1]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services ".

[2]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[3]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

# 3	Rationale

A key issue has been introduced on anomalous behavior detection by NWDAF. This contribution provides a solution.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.866.

********** START OF CHANGES

## 6.Y	Solution#Y: Detection of anomalous NF behaviour by NWDAF

### 6.Y.1	Introduction

This solution addresses key issue #2.2.

The 5GC supports various NF deployments in order to ensure that the NF can provide services from multiple locations. These cloud platforms may be owned and operated by the PLMN operator himself or they may be run on commercial public cloud platforms. When NFs are distributed across multiple cloud infrastructures, it is possible that the NFs behave in an anomalous manner. Such anomalous behaviour can arise either due to configuration errors or internal data corruption, or due to an attack.

To allow an operator to manage its NF cloud deployment efficiently, additional security relevant log data from the NFs can be provided to the NWDAF, which can then be used to analyse and detect the root cause for an anomaly.

### 6.Y.2	Solution details

The OAM or some operator defined AF may subscribe/request to the NWDAF to notify/respond when it detects an abnormal behaviour for a particular NF instance (for example excessive resource usage or consistent failure to provide its service). There can be various causes of abnormal behaviour such as an erroneous operation at a NF, an error at the side of the cloud provider where the NF is deployed, or an impending attack at a particular NF by a malicious entity in the system.

In order to analyse the root cause of the anomaly with certain degree of confidence, more information or related data can help. Thus, NWDAF requests inputs from different entities in the system (such as OAM, NRF and NFs) for detailed analysis. Security related data for analytics can also be collected by NWDAF from the NFs. The final output analytics is then sent to the OAM or the AF for adequate measures.

The procedure depicted in Figure 6.Y.2-1 allows a consumer to request analytics from NWDAF for anomalous NF behaviour and its root cause.

Figure 6.Y.2-1: NF anomalous behaviour analytics provided by NWDAF

1.	The OAM or operator's AF sends a request/subscription to the NWDAF for NF anomalous behaviour analytics using either the Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe service operation.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to OAM services to retrieve resource usage and NF resources configuration of all targeted NF instances, following the procedure specified in clause 6.2.3.2 in TS 23.288 [1].

3a.	The NWDAF subscribes to NRF to receive notification on changes, e.g., on the load and status of NF instances registered in NRF, using Nnrf_NFManagement_NFStatusSubscribe service operation for all targeted NF instances. NF instances are identified by their NF id.

3b.	NRF notifies NWDAF of changes on the load and status of the requested NF instances by using Nnrf_NFManagement_NFStatusNotify service operation.

4a.	The NWDAF subscribes or requests the additional security specific log info (as specified in the table 6.Y.2-1) for a particular NF by invoking the Nnf_EventExposure_Subscribe service operation.

4b.	The NF then notifies the NWDAF (e.g. with the complete log report) by invoking Nnf_EventExposure_Notify service operation.

5.	The NWDAF derives the relevant analytics using the inputs provided by the OAM, NRF, and the NF (as specified in the table 6.Y.2-2)

6.	The NWDAF provides requested NF anomalous information along with the corresponding root cause (for instance a malicious NF trying to attack other NF for denial of service, or an erroneous NF unable to provide service to other NFs) using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

Table 6.Y.2-1 Security specific parameters provided by NFs (NF/NRF) to assist in detection

| Information | Description |
| --- | --- |
| Timestamp | A time stamp associated with the service request which was sent by the NF Service Consumer to a NF Service Producer. |
| NF Service Consumer / SCP Identifier | The consumer instance or the SCP which sends the service request along with the access token to the NF Service Producer. |
| NF Service Producer Identifier | The producer instance which receives the requests and which verifies the access token received along with the requests |
| Authorization status of NF Service Consumer | Indicated if a given NF Service Consumer is authorized to receive an access token or not, as provided by NRF. |
| Access Token Authenticity | Information such as, if access token provided is for the service request it is sent, and if it is generated for the NF Service Consumer which is requesting the service. |
| Access Token Validity | Verification result, i.e. whether the access token is valid or invalid. |
| Number of requests to access a service | Number of simultaneous requests received at the NF Service Producer for a particular time window. |
| Requested Service Name | Name of the service for which the requests had been received. |
| Service Response Confirmation | Confirmation whether the NF Service Producer was able to fulfil the service requests or not. |

Table 6.Y.2-2 Inputs provided to NWDAF in assisting the detection of anomalous NF

| Information | Source | Description |
| --- | --- | --- |
| Security Log Data | NF | Additional security relevant log info as described in table 6.Y.2-1. |
| NF Load | NRF | The load of specific NF instance(s) recorded in their NF profile as defined per TS 29.510 [2]. |
| NF resource usage | OAM | The usage of assigned virtual resources for specific NF instance(s) (e.g., mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [3] clause 5.7. |

Editor's Note: Output analytics from NWDAF is FFS.

Editor's Note: Rationale about how to derive the output from the input is FFS.

### 6.Y.3	Evaluation

TBD

********** END OF CHANGES