3GPP TSG-SA3 Meeting #102-e 	S3-210094-r1

- e-meeting, 18 - 29 January 2021	Revision of

- Source:	InterDigital

- Title:	Solution for UAV location privacy protection

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is proposed to approve this new solution in TR 33.854.

# 2	References

# 3	Rationale

The contribution proposes a solution to address KI#5 " Privacy protection of UAS identities ". The fake USS/UTM issue part of KI#1 is also addressed in this solution.

Solution #25 in TR 23.754 [3], adopted as the basis for normative work for UAV location tracking, includes the support for a new "unknown UAV tracking" feature. This feature allows a given USS/UTM to obtain a list of UAVs that are present in a specified target area. Currently, solution #25 does not have any provision to prevent exposure of UAV location information to unauthorized USS/UTM (e.g., competitor USS/UTM).

This solution proposes enhancements to solution #25 to ensure that only authorized entities (e.g., USS/UTM serving the UAV) can obtain location information for a UAV or set of UAVs from the 3GPP system.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.854. Note that all text is new.

***	BEGIN OF CHANGES	***

## 6.X	Solution #X: UAV location privacy protection

### 6.X.1	Solution overview

The contribution proposes a solution to address KI#5 " Privacy protection of UAS identities ". The fake USS/UTM issue part of KI#1 is also addressed in this solution.

Solution #25 in TR 23.754 [3], adopted as the basis for normative work for UAV location tracking, includes the support for a new "unknown UAV tracking" feature. This feature allows a given USS/UTM to obtain a list of UAVs that are present in a specified target area. Currently, solution#25 does not have any provision to prevent exposure of UAV location information to unauthorized USS/UTM (e.g., competitor USS/UTM).

This solution proposes enhancements to solution #25 to ensure that only authorized entities (e.g., USS/UTM serving the UAV) can obtain location information for a UAV or set of UAVs from the 3GPP system.

The solution assumes (as per TR 23.754 [3] clause 8.5) that UAV location information is provided to USS/UTM via a UAVF which may encompass the NEF/SCEF functionality.

### 6.X.2	Solution details

#### 6.X.2.1	USS/UTM identifier association with individual UAV during UAV A&A

During a UAV A&A procedure, UAVF and/or AMF stores a USS/UTM identifier (e.g., FQDN or IP address of the USS/UTM) and associates it with the 3GPP UAV ID (e.g., GPSI) and the CAA level UAV ID of the UAV that is successfully authenticated and authorized. The USS/UTM identifier may be obtained from the UE during the UAV A&A procedure (e.g., in Registration or PDU Session establishment request). The UAVF may verify the authenticity of the USS/UTM identifier (e.g., check included USS/UTM digital signature). The UAVF may also resolve the USS/UTM address based on the UE provided CAA Level UAV ID by means outside the scope of this solution (e.g., via a trusted resolution function).

During UAV location tracking procedure, the UAVF which verifies that the location tracking request is authorized (i.e., checks that identifier of the USS/UTM making the request matches the USS/UTM identifier previously associated with the 3GPP UAV ID during UAV A&A procedure).

NOTE: It is assumed that the location request from USS/UTM can be authenticated by UAVF (e.g., check included USS/UTM digital signature).

6.X.2.2	Verification of USS/UTM authorization for unknown UAV location tracking

When receiving an "unknown UAV" location tracking request from a USS/UTM, The UAVF checks the validity of the request (described in 6.X.2.1) before forwarding the request to the appropriate location tracking function (e.g., AMF, GMLC).  The request includes an indication that the request is for any (e.g., unknown) UAV(s) in the target area.

The AMF obtains location information of all the UAVs in the given area. The AMF may perform filtering of UEs in that area such as to select only those that fullfill relevant UAV selection criteria (e.g., with a valid UAV subscription and/or that have been authorized by a USS/UTM, as per TR 23.754 [3] clause 8.5). The AMF sends the location information to the UAVF for each of the UAVs that are in the given area including the 3GPP UAV ID for each UAV.

For each UAV, the UAVF selects UAVs whose 3GPP UAV ID is associated a USS/UTM identifier that matches the USS/UTM id from the location request and sends the UAV location information to the USS/UTM accordingly.

Alternatively, the AMF may perform filtering on the UEs in the given area such as to select only UEs that are UAVs served by the requesting USS/UTM (assuming AMF has stored USS/UTM identifier as described in 6.X.2.1 or UAVF provides it to AMF in the location request).

### 6.X.3	Solution evaluation

***	END OF CHANGES	***