3GPP TSG-SA3 Meeting #102-e 	S3-210163-r4

- e-meeting, 18 - 29 January 2021											Revision of S3-20xxxx

- Source:	ZTE

- Title:	Solution of Mitigation against the SUCI replay attack

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

(Reference - in list form - should be made to previous related SA5/3GPP/etc. documents.)

(For changes against a draft TS/TR, a pseudo CR - a.k.a. pCR - will be provided using this Tdoc template. In this case, the number, name and version of the draft TS/TR used as base must be provided and the version must be the latest available version of the draft TS/TR.)

[1]			3GPP TR 33.846 v0.9.0

[2]			3GPP TS 33.501

[3]			3GPP TS 33.303

# 3	Rationale

SUCI replay attacks can attack user privacy, obtain user location information, and also DoS attack the home network of the user. Therefore, the home network of the UE and the UE requires mechanisms to mitigate SUCI replay attacks.

It is proposed to add the security requirement of SUCI replay.

# 4	Detailed proposal

It is proposed that SA3 include the below pCR for inclusion into the TR [1].

**** START OF CHANGES ****

### 6.2.X		Solution #2.Y: Mitigation against the SUCI replay attack

#### 6.2.X.1	Introduction

This solution addresses the key issue #2.2 SUCI replay.

#### 6.2.X.2	Solution details

Current usage of ECIES for concealment of SUPI can be expanded to accommodate MESSAGE_TIME and SUPI.  The MESSAGE_TIME represents the UTC-based time when the message is sent. Maximum allowed size of cipher text from concealment of protection scheme output is 3000 digits. SUPI utilizes only few bytes of those maximum allowed digits and still can adapt MESSAGE_TIME.

#### 6. 2.X.3	Adaptation of proposal in TS 33.501

The following sequence chart illustrates the text already specified in TS 33.501[2] clause 6.1.2 for the case of initiation of authentication and selection of authentication method. Changes are marked in blue.

2.  <N1 message>

(SUCI containing MESSAGE_TIME )

3. Nausf_UEAuthentication_ Authenticate Request

(SUCI containing MESSAGE_TIME or SUPI, SN-Name)

4. Nudm_UEAuthentication_ Get Request (SUCI containing MESSAGE_TIME or SUPI, SN-Name)

AMF/

SEAF

AUSF

1. Conceal SUPI and MESSAGE_TIME

5. [SUCI to SUPI, MESSAGE_TIME de-concealment].

a. If SUCI containing MESSAGE_TIME,

The UDM compares the received MESSAGE_TIME and the current UTC-based time.

If the received MESSAGE_TIME is less than the current UTC-based time minus MAX_DELAY, UDM response a failure code or discard the message.

If the received  MESSAGE_TIME is greater than or equal to the current UTC-based time minus MAX_DELAY, and less than the current UTC-based time, UDM selects the authentication method based on SUPI and generates AV.

b. If SUPI,

UDM selects the authentication method based on SUPI and generates AV.

UE

UDM/ARPF

Figure 6. 2.X.3-1: Illustration of UE and HE is sharing MESSAGE_TIME along with SUCI

- During the primary authentication procedure, USIM concatenates SUPI and MESSAGE_TIME. The MESSAGE_TIME represents the current UTC-based time (number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900), similar to the mechanism used in ProSe Discovery protection TS 33.303[3]. The UE may obtain UTC time from any sources available, e.g. the RAN (via SIB, as in LTE via SIB16), NITZ, NTP, GPS (depending on which is available). MESSAGE_TIME is provided by the ME to the USIM. The concatenated plain text block is encrypted using ECIES method. New value is introduced for “SUPI Type”, for e.g. Value 4 represents SUCI encoded with SUPI plus MESSAGE_TIME.

- Editor’s Note: It is FFS when null scheme used in the SUCI generation.

- Editor’s Note: The UTC time in UE and the UDM may be different because of different source or other errors, and the synchronization mechanism is FFS.

- UE shall use SUCI containing  MESSAGE_TIME in Registration request message, which is sent to AMF/SEAF.

- AMF/SEAF shall invoke the Nausf_UEAuthentication service by sending a Nausf_UEAuthentication_Authenticate Request message to the AUSF whenever the AMF/SEAF wishes to initiate an authentication.

The Nausf_UEAuthentication_Authenticate Request message shall contain either:

-	SUCI containing MESSAGE_TIME, as defined in the current specification, or

-	SUPI, as defined in TS 33.501 [2].

The AMF/SEAF shall include the SUPI in the Nausf_UEAuthentication_Authenticate Reques t message in case the AMF/SEAF has a valid 5G-GUTI and re-authenticates the UE. Otherwise the SUCI containing MESSAGE_TIME is included in Nausf_UEAuthentication_Authenticate Request. SUPI/SUCI structure is part of stage 3 protocol design.

The Nausf_UEAuthentication_Authenticate Request shall furthermore contain:

-	the serving network name, as defined in sub-clause 6.1.1.4 of the present document.

- Upon receiving the Nausf_UEAuthentication_Authenticate Request message, the AUSF shall check that the requesting AMF/SEAF in the serving network is entitled to use the serving network name in the Nausf_UEAuthentication_Authenticate Request by comparing the serving network name with the expected serving network name. The AUSF shall store the received serving network name temporarily. If the serving network is not authorized to use the serving network name, the AUSF shall respond with "serving network not authorized" in the Nausf_UEAuthentication_Authenticate Response.

The Nudm_UEAuthentication_Get Request sent from AUSF to UDM includes the following information:

-	SUCI containing MESSAGE_TIME or SUPI;

-	the serving network name;

- Upon reception of the Nudm_UEAuthentication_Get Request, the UDM shall invoke SIDF if a SUPI type is SUPI plus MESSAGE_TIME, then SIDF shall de-conceal SUCI to gain SUPI and  MESSAGE_TIME before UDM can process the request.

a. If SUCI containing MESSAGE_TIME

The UDM compares the received MESSAGE_TIME and the current UTC-based time which obtained from the same UTC source as UE.

If the received MESSAGE_TIME is less than the current UTC-based time minus MAX_DELAY, UDM response a failure code or discard the message. MAX_DELAY represents the maximum transmission time threshold.

If the received  MESSAGE_TIME is greater than or equal to the current UTC-based time minus MAX_DELAY, and less than the current UTC-based time, UDM selects the authentication method based on SUPI and generates AV.

b. If SUPI,

UDM selects the authentication method based on SUPI and generates AV.

Note : The Nudm_UEAuthentication_Get Response in reply to the Nudm_UEAuthentication_Get Request and the 	Nausf_UEAuthentication_Authenticate Response message in reply to the 	Nausf_UEAuthentication_Authenticate 	Request message are described as part of the authentication procedures in 	clause 6.1.3 in TS 33.501 [2].

Note: The solution is restricted to 5G.

The Authentication procedure (considering example of 5G AKA) is the same as TS.33.501 [2] section 6.1.3.2.0. The 	solution does not impact the existing call flow.

Editor’s Note: The value of MAX_DELAY is FFS to avoid replay attack within the timer buffer.

Note: UE might re-send the same SUCI with earlier time stamp if T3519 timer is running. To avoid discarding a lot of legtimate authentication requests, the value of MAX_DELAY should be larger than T3519.

#### 6. 2.X.4	Solution summary

The solution adds a new SUPI type in 23.003 section 2.2B.

Encryption and decryption on ECIES consider plain text block as “SUPI and MESSAGE_TIME” at UE and UDM / ARPF in 33.501[2].

Registration request is sent with SUPI concatenate MESSAGE_TIME.

UDM need to compare the received MESSAGE_TIME and the current time.

Editor’s Note: Backward compatibility analysis is FFS.

#### 6. 2.X.5	Evaluation

The solution mitigates the SUCI replay attack.

The solution requires changes on the USIM and the UDM.

At USIM, during ECIES procedure of primary authentication, New SUPI type is added, MESSAGE_TIME is concatenated with SUPI (plain text block).

At UDM, when the SUPI indicates ‘SUPI plus  MESSAGE_TIME’ the de-concealment needs to disassociate SUPI and  MESSAGE_TIME. UDM compares the current time and the received MESSAGE_TIME to decide whether there is a SUCI replay attack.

MAX_DELAY is used to avoid discarding a large number of legitimate authentication requests. It may be set based on local policy, network status, MNO configuration.

Editor’s Note: Further evaluation is FFS.

**** END OF CHANGES ****