- 3GPP TSG-SA3 Meeting #102e	draft_S3-210296-r2

- e-meeting, 18 – 29 January 2021	revision of S3-210296

- Source:	CATT

- Title:	pCR to TR33.847- New solution on UE-to-Network Relay based on primary authentication

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

Approve the pCR below

# 2		Rationale

A proposal for the security of UE-to-Network relay.

# 3	Detailed proposal

#### *** BEGIN CHANGES ***

## 6.X	Solution #X: UE-to-Network Relay security based on primary authentication

### 6.X.1	Introduction

This solution addresses the Key Issue #3, Key Issue #4 and Key Issue #9. It is applicable to both L2 and L3 UE-to-Network Relay architectures.

The solution reuses the PC5 unicast communication security procedure defined in TS 33.536 [8] for the PC5 communication security.

Editor’s note: The difference from LTE procedures needs to be highlighted.

Editor’s note: It is FFS whether this is a feasible solution under unprotected one-to-one PC5 communication when reuses unicast mechanism in TS 33.536 [8].

### 6.X.2	Solution details

The UE-to-network relay security flow based on primary authentication is described as follows:

1.	The Remote UE generates a freshness parameter Nonce_1 for the one-to-one communication as specified in TS 33.536[8] and sends a Direct Communication Request to the Relay UE. In addition to the one-to-one communication parameters, the message includes SUCI and Relay Service Code.

Editor’s note: Whether and how the Remote UE may use its 5G native security context to connect via the Relay UE is FFS.

2.	The Relay UE sends a NAS Relay Key Request to its serving AMF. The message includes Remote UE's SUCI, Relay Service Code and Nonce_1.

3.	The AMF of the Relay UE checks whether the Relay UE is authorized to be a Relay UE according to the Relay Service Code.

4.	The AMF of the Relay UE sends a Relay Key Request to the AUSF of the Remote UE. The message includes Remote UE's SUCI, Relay Service Code and Nonce_1.

5-6. The AUSF of the Remote UE retrieves an Authentication Vector (AV) from the UDM of the Remote UE.

Editor’s note: The necessity of AV in step 5 & 6 is FFS.

7.	The AUSF of the Remote UE checks whether the Remote UE is authorized to be a Remote UE according to Remote UE’s SUPI and Relay Service Code.

Editor’s note: Whether AUSF can check authorization with UDM that Remote UE is authorized as Remote UE and based on Relay Service code is FFS.

8.	If the Remote UE is authorized, primary authentication procedure is performed between the Remote UE and AMF and AUSF as defined in TS 33.501[14] through the Relay UE.

9.	The AUSF of the Remote UE derives 5GPRUK using Kausf and some other parameters, generates a corresponding 5GPRUK ID. The AUSF of the Remote UE also needs to generate a 5GPRUK_Info from which the Remote UE can derive 5GPRUK and obtain 5GPRUK ID. The AUSF of the Remote UE generates a new random number as the 5GKd Freshness Parameter, and then generates a new 5GKd using 5GPRUK, 5GKd Freshness Parameter, Nonce_1, Relay Service Code etc.

“NOTE1:	 5GPRUK and 5GPRUK ID are equivalent to PRUK and PRUK ID in TS 33.303 [6], respectively.

“NOTE2:	 The detailed structure of  5GPRUK_Info will be defined in normative phase.

10-11.	The AUSF of the Remote UE sends 5GKd, 5GKd Freshness and 5GPRUK_Info to the AMF of the Relay UE, and then further passes them to the Relay UE.

12.	The Relay UE sends a Direct Security Mode Command to the Remote UE as specified in TS 33.536[8]. In addition to one-to-one communication parameters, the message includes 5GKd Freshness Parameter and 5GPRUK_Info.

13.	The Remote UE derives the 5GPRUK and obtains the 5GPRUK ID using the information in 5GPRUK_Info. The Remote UE further derives the 5GKd and performs other procedure as specified in TS 33.536[8].

14.	The Remote UE sends Direct Security Mode Complete message to Relay UE as specified in TS 33.536[8].

Figure 6.X.2-1: UE-to-Network Relay security flow based on primary authentication

### 6.X.3	Evaluation

TBD

#### *** END OF CHANGES ***