3GPP TSG-SA3 Meeting #102-e 	draft_S3-210329-r1

- e-meeting, 18 – 29 January 2021													Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	KI on Linkability by SUCI

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Additional linkability attack - new KI

# 2	References

[1]	TR 33.846

# 3	Rationale

It is proposed to add a new KI for the attack on linkability by SUCI.

In Sa3#101e it was said that the description and threats already exist in other key issues: the SUCI replay KI#2.2 and the SUPI guessing attack KI#3.2.

This KI is not about SUCI replay as part of KI#2.2, because each time a new SUCI is generated in the new KI described attack.

Even though the attack is similar to the one described in the SUPI guessing KI#3.2, the goal of the attack in KI#3.2 is to identify the subscribers of the network. The goal of below new KI is to observe a specific UE with a known SUPI. The attack does not fit in KI group 3, because it is about linkability, therefore, the description is added as a new KI in the KI group 2.

# 4	Detailed proposal

************ START OF CHANGES

TO BE REPLACED BY “NEXT CHANGE” IF AGREEABLE

- 5.2.x	Key Issue #2.x: Linkability by SUCI

- 5.2.x.1	Issue details

In contrasts to key issue #2.2 where replaying a previously captured SUCI to the network, i.e. replacing a SUCI that a UE sends to the network in some NAS message (e.g. registration request, identity response) with the previously captured SUCI, and observing whether the authentication is successful was addressed, this key issue shows that the linkability attack is also possible without SUCI replay.

This SUCI-based linkability attack does not use a previously captured SUCI but uses a SUCI generated by the attacker from a SUPI. This is possible, if an attacker has gained knowledge of the SUPI of a subscriber, e.g. by IMSI catching in 4G. Or, if an attacker wants to trace a specific person, e.g. the leader of a country or the CEO of a company, from whom the SUPI is known.

Since the Home Network Public Key deployed on the USIMs of all subscribers in the Home Network HPLMN, is not secret, even though an operator may attempt to make it not visible to the outside, an attacker can use this "public" key to create a valid SUCIs from a SUPI. With this ability, an attacker can perform a SUCI-based linkability attack without the need for replaying a previously captured SUCI. This means, even if SUCI replay is prevented e.g. by introducing a time stamp, the attacker will be able to carry out the linkability attack by generating valid SUCIs based on the known SUPI/IMSI and the current time.

- 5.2.x.2	Security Threats

An attacker operating a false base station can detect a known subscriber's presence in a specific area and a specific time by linking authentication responses for different requests, if different SUCIs were generated from the same SUPI/IMSI.

In case this linkability attack occurs, traceability of a subscriber becomes possible. The attacker observes that the UE under attack is receiving a response to a registration request. This allows to locate a UE in different cells and therefore allows correlation. Thus, it represents a breach of the user’s untraceability, thus, this attack compromises privacy.

- 5.2.x.3	Potential security requirements

Same as in 5.2.1.3.

************ NEXT CHANGES#

### 5.2.2	Key Issue #2.2: SUCI replay

#### 5.2.2.1	Key issue details

This key issue addresses SUCI replay attacks.

A specific SUCI linkability attack and a DoS attack related to SUCI replay are described in the following.

NOTE: KI#2.1 holds the related generic requirement for mitigation of linkability attacks, but this key issue holds specific requirements related to SUCI usage.

Linkability attacks

Linkability by SUCI replay: The SUCI can be used for a linkability attack, i.e. an attack by which it is possible to find out whether a UE observed at some location/time X is identical to a UE observed at some location/time Y. For this, an attacker records a SUCI that is used over the radio interface by a UE_A.

As the UE uses a 5G-GUTI rather than a SUCI in most cases, the attacker may also execute an active attack, such as corrupting the 5G-GUTI when it is sent by the UE, which leads with a significant probability to an identity request, so the UE will send a SUCI subsequently in an identity response. An attacker may catch many SUCIs by this method from location/time X.

The attacker can set up a base station at a different location and actively changes the registration request of users registering to this base station. The linkability attack works then like this: If some UE_B makes a registration request to this false base station operated as a relay by the same attacker, the attacker can modify UE_B's registration request message by exchanging the SUCI used in this request by the previously captured SUCI of UE_A or in case of 5G GUTI being used forcing the UE first to respond to an identity request and then modifying the registration request..The modified request is forwarded to the network. Note, this is possible, because UE B's first message is unsecured pre-authentication traffic.

Subsequently, the attacker observes whether a successful AKA run is performed, and the registration request is accepted by the network. If so, then UE_ B needs to be the same as  UE_A. Thus the location and time of a UE moving from one tracking area to another can be linked and may compromise the privacy of the owner of the UE.

Note that this attack cannot by mitigated by hiding only the content of the AKA response, because the attacker can detect from the subsequent messages (e.g. RRC SMC procedure, repetition of the AKA, or RRC release) whether the AKA run was successful or not. If the content of the AKA response is not hidden that could be used directly to determine the UE under attacke is present.

Linkability by SUCI not replayed: In contrasts to above scenario where replaying a previously captured SUCI to the network, i.e. replacing a SUCI that a UE sends to the network in some NAS message (e.g. registration request, identity response) with the previously captured SUCI, and observing whether the authentication is successful was addressed, this key issue shows that the linkability attack is also possible without SUCI replay.

This SUCI-based linkability attack does not use a previously captured SUCI but uses a SUCI generated by the attacker from a SUPI. This is possible, if an attacker has gained knowledge of the SUPI of a subscriber, e.g. by IMSI catching in 4G. Or, if an attacker wants to trace a specific person, e.g. the leader of a country or the CEO of a company, from whom the SUPI is known.

Since the Home Network Public Key deployed on the USIMs of all subscribers in the Home Network HPLMN, is not secret, even though an operator may attempt to make it not visible to the outside, an attacker can use this "public" key to create a valid SUCIs from a SUPI. With this ability, an attacker can perform a SUCI-based linkability attack without the need for replaying a previously captured SUCI. This means, even if SUCI replay is prevented e.g. by introducing a time stamp, the attacker will be able to carry out the linkability attack by generating valid SUCIs based on the known SUPI/IMSI and the current time.

DoS attack

The current ECIES scheme is vulnerable to replay attacks because it does not have the mechanisms in the network side to justify whether the received SUCI was the previous one sent by the UE to the network or not.

If an attacker launches the replay attack multiple times, the UDM and the UE have to spend a lot of resources to process the replayed SUCI and the authentication request message respectively because these messages are legitimate. This raises a DoS attack on the UDM and the UE respectively.

#### 5.2.2.2	Potential threats

Linkability attack:

In case of SUCI replay attack: An attacker observes whether a successful AKA run is performed with a replayed SUCI, i.e. if the registration request is accepted by the network. If so, the attacker can link a UE observed in one location with a UE observed in another location. If done at several location, even though UE may still be anonymous, the tracking profile of the same UE being in different location may compromise privacy.

Based on the response to the valid AUTN and RAND from the UE, the attacker could trace the UE with some confidence even though its SUPI is unknown to the attacker.

In case of linkability by SUCI not replayed: An attacker operating a false base station can detect a known subscriber's presence in a specific area and a specific time by linking authentication responses for different requests, if different SUCIs were generated from the same SUPI/IMSI. In case this linkability attack occurs, traceability of a subscriber becomes possible. The attacker observes that the UE under attack is receiving a response to a registration request. This allows to locate a UE in different cells and therefore allows correlation. Thus, it represents a breach of the user’s untraceability, thus, this attack compromises privacy.

DoS attack:

Replaying the same SUCI many times could lead to a DoS attack on the UDM and the UE respectively. A DoS attack on the UE may result in a decrease in the processing capability of the UE and a rapid consumption of the battery. A DoS attack on UDM will cause the processing power of the UDM to decrease and the response to the request of the legitimate UE to be slow.

5.2.2.3		Potential security requirements

The 5G system shall support mechanisms to mitigate the Linkability attacks.

************ END OF CHANGES