3GPP TSG-SA3 Meeting #102-e 	draft_S3-210330-r1

- e-meeting, 18 – 29 January 2021													Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Assuring SUCI generation by legitimiate SUPI owner

- Document for:	Approval

Agenda Item:	5.5

# 1	Decision/action requested

Solution to assure SUCI generation by legitimate SUPI owner

# 2	References

[1]	TR 33.846

# 3	Rationale

Solution proposal for new KI resolving another type of linkability attack.

# 4	Detailed proposal

************ START OF CHANGES

- 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#1.X |  | KI#2.1 |  | KI#2.2 | KI#2.Y | KI#3.1 | KI#3.2 | KI#4.1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions for anchor keys security |  |  |  |  |  |  |  |  |  |  |  |  |
| No solution so far |  |  |  |  |  |  |  |  |  |  |  |  |
| Solutions for resilience against identifier linkability |  |  |  |  |  |  |  |  |  |  |  |  |
| #2.1: Handling of Sync failure by AUTS encryption |  |  | x |  |  |  |  |  | x |  |  |  |
| #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF |  |  | x |  |  |  |  |  | x |  |  |  |
| #2.3: Unified authentication response message by UE |  |  | x |  |  |  |  |  |  |  |  |  |
| #2.4: MAC-S based solution |  |  | x |  |  |  |  |  | x |  |  |  |
| #2.5: Encryption of authentication failure message with SUCI method |  |  |  |  | x |  |  |  |  |  |  |  |
| #2.6: Certificate based encryption of unicast NAS message |  |  | x |  |  |  |  |  | x |  |  |  |
| #2.X: Assuring SUCI generation by legitimiate SUPI owner |  |  |  |  |  | x |  |  |  |  |  |  |
| Solutions for availability aspects of SUCI usage |  |  |  |  |  |  |  |  |  |  |  |  |
| No solution so far |  |  |  |  |  |  |  |  |  |  |  |  |
| Solutions on re-synchronisation in AKA |  |  |  |  |  |  |  |  |  |  |  |  |
| #4.1: Using MACS as freshness in the calculation of AK |  |  |  |  |  |  |  |  | x |  |  |  |
| #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA |  |  |  |  |  |  |  |  | x |  |  |  |
| #4.3: SQN protection by concealment with SUPI in USIM |  |  | x |  | x |  |  |  | x |  |  |  |
| #4.4: SQN protection during re-synchronisation procedure in AKA |  |  |  |  |  |  |  |  | x |  |  |  |

************ NEXT CHANGE

### 6.2.X	Solution #2.X: Assuring SUCI generation by legitimiate SUPI owner

#### 6.2.X.1	Introduction

This solution addresses Key Issue #2.Y: Linkability by SUCI.

This solution proposed to use the long-term key K in SUCI generation, such that it is assured that the created SUCI has been created by the user to whom the long-term key K belongs to.

#### 6.2.X.2	Solution details

The solution follows the SUCI generation scheme, but the MAC tag is generated out of a new MAC key, which is the result of a one-time function taking the Ephemeral MAC key and K as input. The rest of the known SUCI generation scheme is not touched.

After the Ephemeral MAC key has been generated (according to the scheme described in Annex C of TS 33.501), a new one-way function f using the long-term key K as additional input to the ephemeral MAC key is added. The result is used as the new MAC key: MAC key  f (K, Ephemeral MAC key).

This new MAC key cannot be generated by an attacker, unless he would be in possession of the individual long-term key K of the subscriber. With the new MAC key, the computation of the SUCI is continued as specified in 3GPP TS 33.501 [V].

NOTE: What one-way function to use is outside the scope of this solution. In 3GPP specifications, there are various examples for one-way functions, e.g. the key derivation function used for key derivations according to Annex A in 3GPP TS 33.501 [V], or the functions of the Milenage and Tuak algorithm sets (3GPP TS 35.205 [Y], 3GPP TS 35.231 [Z]).

#### 6.2.X.3	Evaluation

TBD

************ END OF CHANGES