- 3GPP TSG-SA3 Meeting #102-e 	draft_S3-210352-r1

- e-meeting, 18 – 29 January 2021

- Source:	Huawei, Hisilicon

- Title:	New KI on Roaming-related security mechanisms for SNPNs

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a new KI for FS_eNPN_SEC.

# 2	References

[1]  3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

# 3	Rationale

This pCR proposes to add a new key issue on service authorization in eNPN.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.857.

*** BEGIN OF CHANGE ***

## 5.x	Key Issue #x Roaming-related security mechanisms for SNPNs

### 5.x.1	Key issue details

SA2 has defined roaming architecture to support SNPN along with credentials owned by an entity separation from SNPN in [2]. It needs to be studied whether and how security mechanisms related to roaming between PLMN are applicable for the roaming scenario between SNPN and SNPN/PLMN.

In current roaming architecture for PLMN, access token is a requirement for a NF in PLMN1 to access the services provided by a NF in PLMN2.  This key issue proposes to study how current access token mechanism can be applied for SNPN when a NF consumer in a SNPN access the NF producer  belonging to another SNPN/PLMN.

Note: exsiting service authorization mechnsim for PLMN roaming architecture shall be re-used as much as possible.

### 5.x.2	Security threats

Without authorization in place for the roaming scenario, an unauthorized NF consumer in a SNPN can access an NF producer in another SNPN/PLMN.

### 5.x.3	Potential security requirements

Service authorization shall be supported for the roaming architecture between SNPN and SNPN/PLMN.

*** END OF CHANGE ***