3GPP TSG-SA3 Meeting #102-e 	draft_S3-210402-r9_ERI

- e-meeting, 18 - 29 January 2021												Revision of S3-21xxxx

- Source:	Ericsson, China Mobile, Huawei, InterDigital, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell, Philips, Samsung, ZTE

- Title:	Adding details to provisioning key issue

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is proposed to complement KI#2 with requirements and other details.

# 2	References

[1]		3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

# 3	Rationale

In order to address security related aspects of provisioning in SA2’s TR 23.700-07[1], SA3 needs a complete corresponding key issue.

During SA3#101-e many companies showed support for the proposal below and therefore a majority of companies are of the same understanding what is needed for SA3 in order to address SA2’s pending Editor’s Notes.

# 4	Detailed proposal

***START OF CHANGES***

## 5.2	Key Issue #2 Provisioning of Credentials

### 5.2.1	Key issue details

This Key Issue aims at addressing security implications introduced in solutions related to Key Issue #4 in TR 23.700-07 [3].

The objective of Key Issue #4 in TR 23.700-07 [3] is twofold, UE onboarding and then remote provisioning of non USIM credentials for SNPN and PNI-NPN.. This Key Issue aims at studying the corresponding security implications related to the provisioning. For PNI-NPNs, only credentials for secondary and slice-specific authentication need to be considered.

The UE can perform the onboarding procedure with an onboarding network, and then the UE may be remotely provisioned with the credentials by a Provisioning Server (PS). Trust relationship between the PS and the credential owners is necessary.

Designing completely new protocols is not in scope of this key issue.

### 5.2.2	Security threats

An unauthorized UE may be able to access PS for maliciously requiring remote provisioning service.

An unauthorized PS may be able to provide wrong remote provisioning service to the UE.

Unprotected provisioning of credentials may cause the SNPN credentials to be obtained or manipulated.

### 5.2.3	Potential security requirements

The UE and the PS should be authorized for remote provisioning.

Editor’s note: The entity granting the authorization is FFS.

Credentials shall be confidentiality protected, integrity protected, and replay protected during remote provisioning.

Editor’s note: It is FFS whether the protection in the above requirement requires to specify a solution in normative phase or whether it is left to implementation. However, it is possible to study solutions for this key issue in this TR.

Editor’s note: The end points for the protection in the above requirement are FFS.

***END OF CHANGES***