3GPP TSG-SA3 Meeting #102-e 	draft_S3-210408-r1

- e-meeting, 18 - 29 January 2021											Revision of S3-21xxxx

- Source:	Ericsson

- Title:	Resolving prerequisite EN in solution #10

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

It is proposed to resolve one of the ENs in solution #10

# 2	References

[1]		3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

# 3	Rationale

In Solution #10 "Secure initial access to an SNPN onboarding network" the following ENs are present:

Editor’s Note: Security implications of UE information pre-configuration, e.g. UE identity, authentication method, in O-SNPN regarding to the trust relationship between O-SNPN, DCS owner and PS owner is FFS.

Editor’s Note: It is ffs that what should be preconfigured in the UE and how does the UE select the O-SNPN who can route the UE to the specific PS since the DCS may be shared by multiple O-SNPNs.

Regarding the first Editor's Note, this solution assumes default credentials have been provisioned to UEs as mentioned in the introduction. The identifier in the Default Credentials acts as unique UE identifier. The trust relationship between the O-SNPN and the DCS is that the O-SNPN trusts the DCS to perform authentication on its behalf. This is also clarified in the proposed updates below. The trust relationship regarding the PS are out of scope of the solution as the main objective of key issue #4 is to establish secure initial access.

Regarding the second Editor's Note, it is already stated in the introduction of the solution that the UE is provisioned with default credentials. This is further clarified in the proposed updates below. Selection of the O-SNPN is not in the scope of SA3, but in the scope of SA2. In the updates below, a Note has been added for clarification.

# 4	Detailed proposal

***START OF CHANGES***

## 6.10	Solution #10: Secure initial access to an SNPN onboarding network

### 6.10.1	Introduction

This solution addresses key issue#4 Securing initial access for UE onboarding between UE and SNPN. The proposed solution relies on the deployment scenario described in Key issue #1 Credentials owned by an external entity where the deployment utilizes an external AAA-S. Therefore, the solution assumes the UE has been preprovisioned with default credentials to be used for primary authentication, the solution uses EAP-TLS as an example.

Note that any solution candidate to Key issue #1 fits the concept of this solution. Using a key generating EAP method allows for derivation of keys to use protecting the air interface and the DCS provides a temporary SUPI to the onboarding network, this solution proposes to use the identifier from the default credentials. This way the UE becomes uniquely identifiable and verifiably secure. Since EAP-TLS credentials does not have storage requirements on USIM, the UE cannot be assumed to have a USIM therefore it might not be possible to have a Home network public key available at the UE, therefore an anonymous SUCI is adopted.

The onboarding network trusts the DCS to perform authentication on its behalf. The solution does not take a stand on how the PS relates to DCS and onboarding network.

### 6.10.2	Solution details

Figure 6.10.2-1: Initial access with key derivation

1.	The UE sends a registration request to the onboarding SNPN acting as onboarding network. The UE includes an onboarding indication and an anonymous SUCI as described in clause B 2.1.2.2 of TS 33.501 [2].

NOTE: How the UE selects an onbarding network is out of scope of the present document.

2.	AMF forwards the registration request to AUSF.

3.	The AUSF decides based on the onboarding indication that an external authentication is to be performed and uses the realm part of the SUCI to route the request to the right DCS.

4.	The AUSF interacts with the DCS in order to have the DCS perform primary authentication. The AUSF uses a AAA-P/IWF to interact with the DCS.

5.	UE and DCS performs primary authentication based on EAP-TLS. Since the SUCI was anonymous in line with clause B 2.1.2.2 of TS 33.501 [2] the tunnel is setup first before certificates are exchanged.

6.	The DCS sends an EAP response to the AUSF. Including keying material and a SUPI. In this case the UE ID from the certificate would act as SUPI.

7.	The AUSF sends a success message to the AMF including keying material and the SUPI.

8.	The AMF sends an indicator on how the UE shall derive its keys to the UE in a NAS message.

9.	The UE derives its keys and the registration is complete.

This solution proposes that the DCS sends information about how to reach the PS to the ON if the administrator running the DCS knows to which company the devices are sold. The ON or UE can then use this information to select the PS. The details are out of scope of this solution.

Editor’s Note: How to protect the provisioning procedure via Control Plane regarding to the trust relationship between Onboarding SNPN and PS owner’s domain is FFS.

Editor’s Note: The need for sending the ABBA parameter and the key indicator is FFS.

Editor’s Note: Privacy implication of disclosing the SUPI to the O-SNPN is FFS.

### 6.10.3	System impact

UE

Potentially key hierarchy depending on the outcome of KI#1.

AMF

Relay of potentially needed indicator for how the UE should derive keys. Relay of onboarding indicator.

AUSF

AAA-P functionality in order to communicate with external party.

AAA-S

Depends on if KI#1 decides the AAA-S should be 5G aware or not.

### 6.10.4	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

***END OF CHANGES***