3GPP TSG-SA3 Meeting #102-e 	draft_S3-210423-r2

- e-meeting, 18 - 29 January 2021												Revision of S3-21xxxx

- Source:	Nokia, Nokia Shanghai Bell

- Title:	KI on Authentication of NRF and NFp in indirect communication

- Document for:	Approval

Agenda Item:	5.20

# 1	Decision/action requested

KI added according to objectives of the SID added.

# 2	References

[1]	3GPP TS

# 3	Rationale

KI added according to objectives, i.e. copy from SID justification of the objective.

First proposal for threat clause added.

# 4	Detailed proposal

*********  START OF CHANGES

- 4.X	Key issue #X: Authentication of NRF and NF producer in indirect communication

- 4.X.1	Key issue details

When SCP is present, the TLS between an NF Service Consumer and NRF/NF Service Producer can be split into at least two segments (NFc-SCP, SCP-NRF or SCP-NFp). In this case, the NF Service Consumer and NRF/NF Service Producer do not directly authenticate each other via TLS.

Client Credentials Assertion (CCA) has been specified to allow NRF or another NF to directly authenticate an NF Service Consumer in the presence of an SCP, but direct authentication of the NRF/NF Service Producer by the NF Service Consumer has not been addressed in indirect communication. The key issue will investigate for a solutions allowing the NF service consumer to directly authenticate the NRF/NF Service Producer responding to the NF Service Consumer is neededin indirect communication. Whether signing CCAs by NF Service Producers needs to be specified versus the complexity in the system such solution will create needs to be evaluated.

- 4.X.2	Security threats

An NF Service Consumer may send service requests to an unintended NF.

An NF Service Consumer may receive service responses from an unintended NF.

- 4.X.3	Potential security requirements

The 5GS should provide a mechanism that allows the NF Service Consumer to authenticate the 5GC NF responding to its request.

The 5GS should provide a mechanism that allows an NF Service Consumer to directly authenticate an NRF or an NF Service Producer during an indirect communication with them via an SCP.

*********  END OF CHANGES